# 🚀 DCCN — LAST-MINUTE REVISION SHEET (1-Hour Read)

Course: BCA / B.Sc(IT) | Subject Code: US6001 | Sem VI

Read this entire file in the **last hour before exam**. Everything important is condensed here.

## 📋 1. PROTOCOLS & PORT NUMBERS (Memorize Cold)

| Protocol | Port | Transport | Use |
| --- | --- | --- | --- |
| HTTP | 80 | TCP | Web pages |
| HTTPS | 443 | TCP | Secure web |
| FTP | 20 (data), 21 (control) | TCP | File transfer |
| TFTP | 69 | UDP | Trivial file transfer |
| SMTP | 25 (587 submission) | TCP | Email send |
| POP3 | 110 | TCP | Email retrieve |
| IMAP | 143 | TCP | Email server access |
| DNS | 53 | UDP (TCP for zone) | Name resolution |
| DHCP | 67 (server), 68 (client) | UDP | IP auto-config |
| Telnet | 23 | TCP | Remote login |
| SSH | 22 | TCP | Secure remote login |
| SNMP | 161, 162 | UDP | Network management |
| NTP | 123 | UDP | Time synchronization |
| LDAP | 389 | TCP/UDP | Directory service |

Memory trick:

20/21 (FTP), 22 (SSH), 23 (Telnet), 25 (SMTP) → "20s for transfer/login/mail"

53 (DNS), 80 (HTTP), 110 (POP3), 443 (HTTPS) → core internet

## 📐 2. ALL FORMULAS USED IN THIS SUBJECT

### Capacity & Bandwidth:

Nyquist (noiseless): C = 2B × log₂(L) bps

Shannon (noisy): C = B × log₂(1 + SNR) bps

SNR(dB) = 10 × log₁₀(SNR)

Bandwidth-Delay Product = Bandwidth × RTT

### Time & Speed:

Propagation Time = Distance / Speed

Transmission Time = Message size / Bandwidth

Round-Trip Time (RTT) = 2 × Propagation Time

### Hamming Code:

2ᵖ ≥ m + p + 1 (where p = parity bits, m = data bits)

Hamming distance for d-error detection: dmin ≥ d+1

Hamming distance for d-error correction: dmin ≥ 2d+1

### Network Topology:

Mesh links = n(n-1)/2

Mesh I/O ports = n-1 per device

### Subnetting:

Number of subnets = 2ⁿ (n = borrowed bits)

Hosts per subnet = 2ʰ – 2 (h = host bits)

Total addresses in subnet = 2ʰ

### ARQ Efficiency:

Stop-and-Wait: η = 1 / (1 + 2a) where a = T_prop / T_trans

Go-Back-N: η = N / (1 + 2a)

Selective Repeat: η = N / (1 + 2a)

### ALOHA:

Pure ALOHA max efficiency = 1/(2e) ≈ 18.4%

Slotted ALOHA max efficiency = 1/e ≈ 36.8%

### IP Address:

IPv4 = 32 bits (4 octets)

IPv6 = 128 bits (8 groups of 4 hex digits)

MAC = 48 bits (6 bytes hex)

### Cryptography (RSA):

n = p × q, φ(n) = (p-1)(q-1)

e × d ≡ 1 mod φ(n)

Encryption: C = M^e mod n

Decryption: M = C^d mod n

### Checksum (Internet):

1's complement sum of all 16-bit words → take 1's complement

## 📚 3. UNIT-WISE TOP 5 IMPORTANT POINTS

### 🟦 UNIT 1: Introduction

OSI: 7 layers, TCP/IP: 4 layers (Network Access, Internet, Transport, Application)

Topologies: Mesh (most reliable, expensive), Star (hub failure = total fail), Bus (backbone fail = total fail), Ring (ring breaks = total fail)

Transmission media: Twisted pair < Coaxial < Optical fiber (in cost, bandwidth, distance)

LAN < MAN < WAN (geographic scope)

Networking devices' layers: Repeater/Hub (L1), Bridge/Switch (L2), Router (L3), Gateway (all)

### 🟩 UNIT 2: Physical & Data Link

Nyquist (noiseless), Shannon (noisy) for capacity

Data Link Layer services: Framing, Addressing, Flow Control, Error Control, Access Control

Error Detection: Parity, Checksum, CRC. Error Correction: Hamming Code

Switching: Circuit (telephone), Packet (Internet, Datagram/Virtual Circuit)

ARQ Protocols: Stop-and-Wait, Go-Back-N, Selective Repeat

### 🟨 UNIT 3: LLC / MAC

Multiple Access: Random (ALOHA, CSMA), Controlled (Polling, Token), Channelization (FDMA, TDMA, CDMA)

CSMA/CD for wired (Ethernet), CSMA/CA for wireless (Wi-Fi)

Pure ALOHA = 18.4% efficient; Slotted ALOHA = 36.8% efficient

IP Classes: A (0-127), B (128-191), C (192-223), D (multicast), E (reserved)

CIDR uses /n notation, supports VLSM, replaces classful addressing

### 🟧 UNIT 4: Network/Transport

TCP: reliable, connection-oriented, 20-byte header, 3-way handshake. UDP: unreliable, 8-byte header

TCP Flags: URG, ACK, PSH, RST, SYN, FIN

Congestion Control: Slow Start (exponential), Congestion Avoidance (linear), Fast Retransmit, Fast Recovery

Cryptography: Symmetric (DES, AES) vs Asymmetric (RSA, ECC)

QoS parameters: Reliability, Delay, Jitter, Bandwidth

### 🟪 UNIT 5: Application

DNS translates name → IP, hierarchical, port 53/UDP

HTTP: stateless, request-response, port 80

SMTP: push, port 25, commands HELO/MAIL FROM/RCPT TO/DATA/QUIT

FTP: two connections (control 21, data 20), out-of-band control

HTTP methods: GET, POST, PUT, DELETE, HEAD; status codes 1xx/2xx/3xx/4xx/5xx

## 📖 4. 20 KEY DEFINITIONS (One Line Each)

Network: Interconnected collection of autonomous devices that share resources and exchange data.

Topology: Physical/logical arrangement of nodes and links in a network.

Protocol: Set of rules governing data exchange (syntax, semantics, timing).

OSI Model: 7-layer reference architecture for network communication standardized by ISO.

TCP/IP Model: 4-layer practical protocol stack used in the Internet.

Bandwidth: Range of frequencies a channel can transmit (Hz) or its data rate (bps).

Throughput: Actual data delivery rate (≤ bandwidth).

Latency: Total delay from sender to receiver (transmission + propagation + processing + queuing).

Encapsulation: Each layer adding its header to upper-layer data before transmission.

Framing: Dividing bit stream into discrete frames at data link layer.

CRC: Cyclic Redundancy Check — error detection using polynomial division.

Hamming Code: Error-correcting code that detects/corrects single-bit errors using parity bits.

MAC Address: 48-bit physical address assigned to NIC (e.g., 00:1A:2B:3C:4D:5E).

IP Address: 32-bit logical address (IPv4) used for routing across networks.

Subnet Mask: 32-bit number dividing IP into network and host portions.

ARP: Address Resolution Protocol — maps IP to MAC.

ICMP: Internet Control Message Protocol — error reporting and diagnostics.

DNS: Domain Name System — translates names to IPs (UDP port 53).

Socket: Endpoint of network communication (IP + port).

Firewall: Hardware/software that filters traffic between trusted and untrusted networks.

## 🆚 5. CRUCIAL DIFFERENCE TABLES

### 5.1 OSI vs TCP/IP

| Feature | OSI | TCP/IP |
| --- | --- | --- |
| Layers | 7 | 4 (or 5) |
| Origin | ISO (1984) | DARPA (1970s) |
| Approach | Theoretical | Practical |
| Combines | – | App+Pres+Sess into App; DL+Phy into Net Access |
| Adoption | Reference only | Implemented (Internet) |
| Layer dependence | Strict | Loose |

### 5.2 TCP vs UDP

| Feature | TCP | UDP |
| --- | --- | --- |
| Connection | Connection-oriented | Connectionless |
| Reliability | Reliable | Unreliable |
| Order | Ordered delivery | No order |
| Speed | Slower | Faster |
| Header | 20-60 bytes | 8 bytes |
| Flow control | Yes (sliding window) | No |
| Congestion control | Yes | No |
| Use case | HTTP, FTP, SMTP | DNS, DHCP, video, gaming |
| Handshake | 3-way handshake | None |

### 5.3 IPv4 vs IPv6

| Feature | IPv4 | IPv6 |
| --- | --- | --- |
| Address size | 32 bits | 128 bits |
| Notation | Dotted decimal (192.168.1.1) | Hexadecimal colon (2001:db8::1) |
| Header size | 20-60 bytes | 40 bytes (fixed) |
| Address space | ~4.3 × 10⁹ | ~3.4 × 10³⁸ |
| Broadcast | Yes | No (multicast/anycast) |
| Auto-config | DHCP | Built-in (SLAAC) |
| Security | Optional (IPSec) | Built-in IPSec |
| NAT needed | Yes | No |
| Fragmentation | At sender + routers | Only at sender |

### 5.4 CSMA/CD vs CSMA/CA

| Feature | CSMA/CD | CSMA/CA |
| --- | --- | --- |
| Used in | Wired (Ethernet) | Wireless (Wi-Fi) |
| Approach | Detect collision after | Avoid collision before |
| Mechanism | Listen → Send → Detect → Backoff | Listen → IFS → Backoff → RTS/CTS → Send → ACK |
| Efficiency | Higher | Lower (RTS/CTS overhead) |
| Why? | Wired can detect collisions | Wireless can't (hidden node) |

### 5.5 Stop-and-Wait vs Go-Back-N vs Selective Repeat

| Feature | Stop-and-Wait | Go-Back-N | Selective Repeat |
| --- | --- | --- | --- |
| Sender window | 1 | N | N |
| Receiver window | 1 | 1 | N |
| Retransmission | One frame | All from lost onward | Only the lost frame |
| Receiver buffer | None | None | N frames |
| ACK type | Individual | Cumulative | Individual |
| Complexity | Low | Medium | High |
| Bandwidth use | Poor | Better | Best |

### 5.6 Pure ALOHA vs Slotted ALOHA

| Feature | Pure ALOHA | Slotted ALOHA |
| --- | --- | --- |
| Timing | Send anytime | Send at slot boundary |
| Vulnerable period | 2 × frame time | 1 × frame time |
| Max efficiency | 18.4% | 36.8% |
| Synchronization | Not needed | Required |

### 5.7 Circuit vs Packet Switching

| Feature | Circuit | Packet |
| --- | --- | --- |
| Path | Dedicated | None |
| Setup phase | Required | Not required |
| Bandwidth | Reserved | Shared |
| Delay | Setup + minimal | Variable (queuing) |
| Reliability | High | Lower |
| Example | Telephone | Internet |

### 5.8 FTP vs TFTP

| Feature | FTP | TFTP |
| --- | --- | --- |
| Transport | TCP | UDP |
| Port | 20, 21 | 69 |
| Connections | Two | One |
| Authentication | Yes | No |
| Reliability | Reliable | Limited |
| Use | General | Bootstrap, firmware |

### 5.9 Hub vs Switch vs Router

| Feature | Hub | Switch | Router |
| --- | --- | --- | --- |
| Layer | Physical (L1) | Data Link (L2) | Network (L3) |
| Forwarding | Broadcast all | MAC-based | IP-based |
| Collision domain | One | Per port | Per port |
| Speed | Slowest | Fast | Slower than switch |
| Intelligence | None | MAC table | Routing table |

### 5.10 Symmetric vs Asymmetric Cryptography

| Feature | Symmetric | Asymmetric |
| --- | --- | --- |
| Keys | One shared | Two (public + private) |
| Speed | Fast | Slow |
| Distribution | Hard | Easy |
| Algorithms | DES, AES, 3DES | RSA, ECC, DSA |
| Key length | 56-256 bits | 1024-4096 bits |
| Use | Bulk data | Key exchange, signatures |

### 5.11 Twisted Pair vs Coaxial vs Optical Fiber

| Feature | Twisted Pair | Coaxial | Optical Fiber |
| --- | --- | --- | --- |
| Cost | Low | Medium | High |
| Bandwidth | 100 Mbps | 1 Gbps | THz |
| Distance | ~100 m | ~500 m | 100+ km |
| EMI | Susceptible | Moderate | Immune |
| Signal | Electrical | Electrical | Light |
| Use | LAN | TV | Backbone |

### 5.12 LAN vs MAN vs WAN

| Feature | LAN | MAN | WAN |
| --- | --- | --- | --- |
| Range | <1 km | 10-50 km | >100 km |
| Speed | High | Medium | Lower |
| Cost | Low | Medium | High |
| Ownership | Single org | Single org/consortium | Multiple |
| Example | Office | City cable TV | Internet |

## 🎯 6. EXAM DAY STRATEGY

### Time Management (3-hour exam, 100 marks):

5-mark questions: ~10-12 mins each

10-mark questions: ~20-25 mins each

2-mark questions: ~3-5 mins each

### Order of attempt:

Start with 2-mark short-answers (build confidence)

Then 5-mark questions you know best

Then numericals (clean working = full marks)

Finally 10-mark theory with diagrams (most time-consuming)

### Diagram strategy:

OSI/TCP-IP stacks → easy quick marks

3-way handshake → always draw with seq/ack

Hamming code/CRC → show calculation steps

Topologies → simple ASCII-style sketches

### Power phrases that boost marks:

"Layer N of OSI model is responsible for..."

"The key difference between X and Y is..."

"This solves the problem of..."

"The formula for this is..."

"An example of this is..."

## 🔥 7. ULTRA-CONDENSED MEMORY MAP

╔══════════════════════════════════════════════════╗
║   OSI: All People Seem To Need Data Processing   ║
║         App-Pres-Sess-Trans-Net-DataLink-Phys    ║
║                                                   ║
║   TCP/IP: App | Trans | Internet | Net Access    ║
║                                                   ║
║   Topology: Mesh > Star > Bus > Ring (reliability)║
║                                                   ║
║   Devices Layer-wise:                             ║
║   L1: Hub, Repeater                               ║
║   L2: Bridge, Switch                              ║
║   L3: Router                                      ║
║   All: Gateway                                    ║
║                                                   ║
║   Capacity:                                       ║
║   Nyquist (noiseless): C = 2B × log₂L            ║
║   Shannon (noisy):     C = B × log₂(1+SNR)       ║
║                                                   ║
║   ALOHA Efficiency:                               ║
║   Pure: 18.4%   |   Slotted: 36.8%                ║
║                                                   ║
║   ARQ:                                            ║
║   Stop-Wait (1) → Go-Back-N (cumulative ACK) →    ║
║   Selective Repeat (individual ACK + buffer)      ║
║                                                   ║
║   IP Classes:                                     ║
║   A: 0-127 (/8)    B: 128-191 (/16)               ║
║   C: 192-223 (/24) D: 224-239 (multicast)         ║
║                                                   ║
║   TCP Flags: URG ACK PSH RST SYN FIN              ║
║                                                   ║
║   3-Way Handshake: SYN → SYN+ACK → ACK            ║
║                                                   ║
║   Famous Ports:                                   ║
║   20/21=FTP, 22=SSH, 23=Telnet, 25=SMTP,          ║
║   53=DNS, 67/68=DHCP, 69=TFTP, 80=HTTP,           ║
║   110=POP3, 143=IMAP, 443=HTTPS                   ║
║                                                   ║
║   RSA: C = M^e mod n  |  M = C^d mod n            ║
║                                                   ║
║   Hamming: parity bits at positions 1, 2, 4, 8... ║
║   Syndrome = error position                       ║
╚══════════════════════════════════════════════════╝

## 🎓 FINAL TIPS

Always draw diagrams — Examiners reward visualization. Even simple ASCII art works.

Tabulate comparisons — much cleaner than paragraphs.

Show formulas first, then plug in values, then compute.

State assumptions in numericals (e.g., "Assuming 1 KB = 1000 bytes").

Don't skip units — bps, bytes, ms — wrong units = wrong answer.

Number your steps — helps the examiner follow.

Highlight final answer with a box or underline.

Don't leave anything blank — partial credit is better than zero.

# 🎯 GOOD LUCK — YOU'VE GOT THIS! 🚀

Read this revision sheet 2-3 times, glance at the comparison tables, and confidence will follow. All the best!