IMAGE PROCESSING — PROBLEM SET SOLUTIONS

Course: BCA / B.Sc(IT) | Subject Code: UCS6105 | Sem VI

Final Semester VI Examination

Shri Ramswaroop Memorial University

Date: 15 May 2026

# IMAGE PROCESSING – PROBLEM SET SOLUTIONS

Course: BCA / B.Sc(IT) | Subject Code: UCS6105 | Semester: VI

This document solves every question from Mid-Term Set I, Mid-Term Set II, DIT 1, DIT 2, and Problem Sets I–V.

## ====== MID-TERM EXAMINATION — SET I (25/03/2026, 2 hrs, 40 marks) ======

### Q1. What is a 4-neighbor of a pixel?

The 4-neighbor N₄(p) of pixel p(x, y) is one of the four pixels directly adjacent horizontally or vertically:

Coordinates: (x−1, y), (x+1, y), (x, y−1), (x, y+1)

[N]
[W] [p] [E]
    [S]

4-neighbors exclude diagonal pixels. Used in 4-connectivity, region growing, and boundary algorithms.

Exam Tip: Always draw the + shape and label coordinates of the four neighbors.

### Q2. Characteristics of an edge in a digital image

An edge is a set of connected pixels lying on the boundary between two regions of significantly different intensity.

Characteristics:

Sharp intensity change (high gradient magnitude)

Local feature — exists in a small spatial extent

Has a direction perpendicular to the gradient

Detected by first-derivative (Sobel, Prewitt) or second-derivative (Laplacian)

Four ideal profiles: step, ramp, line, roof

Edges carry critical structural information used in segmentation, recognition, and tracking.

Exam Tip: Mention "high gradient + direction perpendicular to gradient" + four edge profiles.

### Q3. What is a negative image?

A negative image reverses gray levels — dark becomes light and vice versa.

Formula: s = L − 1 − r → for 8-bit: s = 255 − r

Example: Input r = 100 → Output s = 155

Application: Enhances white/gray detail in dark backgrounds (mammograms, X-rays).

Exam Tip: Always write the formula + give one X-ray application example.

### Q4. Purpose of log transformations

The log transformation compresses the dynamic range of an image, mapping wide input ranges to narrower output ranges.

Formula: s = c · log(1 + r)

Purposes:

Expands dark pixels so detail becomes visible

Compresses bright pixels to fit display range

Used for displaying Fourier spectra (very wide range)

Mimics human eye's logarithmic perception

Exam Tip: Mention "compresses dynamic range" + Fourier spectrum example.

### Q5. Difference between histogram equalization and histogram matching

| Feature | Histogram Equalization | Histogram Matching |
| --- | --- | --- |
| Output | Uniform (flat) histogram | User-specified histogram |
| Function | T(r) = (L−1)·cdf(r) | G⁻¹(T(r)) |
| Flexibility | Low (automatic) | High (designer-defined) |
| Use case | General contrast enhancement | Match to reference image |
| Steps | One CDF mapping | Two CDFs + inverse mapping |

HE automatically maximizes contrast; HM lets the user choose the target histogram shape.

Exam Tip: Use a clear comparison table with at least 4 rows.

### Q6. Effect of a low-pass filter on noise

A low-pass filter allows low-frequency components to pass while attenuating high frequencies.

Effect on noise:

Most random noise (Gaussian, uniform) has high-frequency components

LPF averages neighborhoods → reduces noise

Side effect: blurs edges and fine details

3×3 mean mask example:

1 1 1
1/9 × [1 1 1]
       1 1 1

Trade-off: More smoothing → cleaner image but more blur.

Exam Tip: Mention noise + edge blur trade-off + provide one 3×3 mask.

### Q7. Primary goal of image restoration

The primary goal of image restoration is to recover the original image f(x, y) from its degraded version g(x, y) using a mathematical model.

Model: g(x, y) = h(x, y) * f(x, y) + η(x, y)

The objective is to compute an estimate f̂(x, y) that minimizes the difference from f. Restoration is objective (model-based), unlike enhancement which is subjective.

Exam Tip: Quote the model formula + state difference from enhancement.

### Q8. Define the "Max filter"

The Max filter is a non-linear order-statistic filter that replaces each pixel with the maximum value in its neighborhood.

Formula: f̂(x, y) = max { g(s, t) : (s, t) ∈ S_xy }

Best for: Removing pepper noise (dark spots = 0), because the max picks brighter neighbors.

Side effect: Brightens the overall image.

Exam Tip: Pair "Max → pepper" and "Min → salt" together.

### Q9. Motion blur degradation in images

Motion blur is degradation caused by relative motion between camera and scene during exposure — modeled as convolution with a linear PSF.

Spatial PSF (1D linear motion length L):

h(x, y) = 1/L for 0 ≤ x ≤ L, y = 0; else 0

Frequency domain (motion a, b over time T):

H(u, v) = (T / π(ua + vb)) · sin(π(ua + vb)) · exp(−jπ(ua + vb))

Parameters: length L, angle θ, exposure T

Causes: camera shake, object motion, long exposure

Restoration: inverse / Wiener filtering with known PSF

Exam Tip: Mention parameters L, θ, T + restoration method.

### Q10. Two applications where image restoration is critical

Astronomy — recovering Hubble / James-Webb telescope images degraded by atmospheric turbulence and optical aberrations

Medical imaging — de-blurring CT/MRI scans and enhancing low-dose X-rays where details affect diagnosis

Other valid examples: forensics (CCTV enhancement), satellite remote sensing, microscopy (defocus blur).

Exam Tip: Choose medical + astronomy — universally accepted examples.

### Q11 (5-mark). Fundamental difference between image enhancement and image restoration with examples

Image Enhancement improves the subjective appearance of an image so it looks better. It is heuristic and based on visual perception.

Image Restoration is the objective recovery of an image from a degraded version using a mathematical model of the degradation.

Comparison Table:

| Feature | Image Enhancement | Image Restoration |
| --- | --- | --- |
| Goal | Improve subjective appearance | Recover original from degradation |
| Approach | Heuristic / visual | Mathematical / objective |
| Knowledge | Visual preference | Degradation function H, noise η |
| Techniques | Histogram, sharpen, gamma | Inverse, Wiener, constrained LS |
| Output | "Looks better" image | "Closer to original" image |

Enhancement example: A dark night photo is brightened using gamma correction (γ = 0.5) for better viewing.

Restoration example: A satellite image blurred by motion is recovered using a Wiener filter with known PSF.

Mathematical contrast:

Enhancement: g = T[f] (no model of f's origin)

Restoration: g = h * f + η; estimate f̂ ≈ f

Exam Tip: Define both → comparison table → one example each.

### Q12 (5-mark). How noise and blurring together affect restoration; which is harder?

When an image is degraded by both blurring (H) and noise (η), restoration becomes substantially harder than either alone.

Combined model: g = h * f + η

Challenges by case:

(a) Blurring alone: Can be reversed with inverse filter F̂ = G/H — straightforward.

(b) Noise alone: Can be reduced by spatial filters (mean, median) — simple.

(c) Both together: Inverse filter amplifies noise at frequencies where |H(u, v)| is small. The reciprocal blows up and noise N(u, v) gets divided by small H, producing huge noise spikes.

Which is harder? → Blurring is more difficult, especially with noise, because:

Naive inversion of H amplifies noise

True H may be unknown (blind deconvolution)

Need regularization (Wiener, constrained LS)

Noise alone is local and statistical; blurring is global and structural

Strategy with both: Use Wiener filter F̂ = [|H|² / (|H|² + S_η/S_f)] · (G/H) — balances inversion against noise suppression.

Conclusion: Blur is harder when combined with noise because the inversion process itself amplifies the noise.

Exam Tip: Mention "noise amplification at zeros of H" + Wiener as solution.

### Q13 (5-mark). Complete image processing system end-to-end with block diagram

A complete digital image processing system traces an image from physical light source to final display.

Block Diagram:

[Energy Source]      →   [Scene / Object]    →   [Optical Lens]
       ↓                                                ↓
[Reflection / Transmission]                    [Image Sensor (CCD/CMOS)]
                                                        ↓
                                              [Analog Signal]
                                                        ↓
                                              [Sampling + Quantization
                                                (A/D Converter)]
                                                        ↓
                                              [Frame Buffer / Storage]
                                                        ↓
                                              [Image Processor (CPU/GPU)]
                                                        ↓
                                              [Image Memory / Mass Storage]
                                                        ↓
                                              [Display Device]
                                                        ↓
                                              [Hardcopy Device / Network]

Component Descriptions:

Energy Source — Provides illumination (sun, lamp, X-ray tube)

Scene/Object — Reflects or transmits energy

Optical Lens — Focuses light onto sensor

Image Sensor (CCD/CMOS) — Converts photons → electrical signal

Analog Signal — Continuous voltage from sensor

A/D Converter — Sampling + Quantization → digital image

Frame Buffer — Temporary storage

Image Processor — CPU/GPU performs filtering, enhancement, etc.

Mass Storage — HDD, SSD, cloud

Display Device — Monitor, projector

Hardcopy Device — Printer

Network — Transmit to remote systems

Exam Tip: Always draw the block diagram + describe each block in 1 line.

### Q14 (5-mark). Compare log and power-law transformations

| Feature | Log Transformation | Power-law (Gamma) |
| --- | --- | --- |
| Formula | s = c · log(1 + r) | s = c · r^γ |
| Parameters | c only | c and γ |
| Behavior | Fixed (concave curve) | Tunable via γ |
| γ < 1 effect | — | Brightens image |
| γ > 1 effect | — | Darkens image |
| Typical use | Fourier spectrum display | Gamma correction (CRT/LCD) |
| Flexibility | Low | High |
| Reversibility | Inverse log | Inverse power law |

Curve shapes:

Log     :  /‾‾   (sharp rise then flatten)
γ<1     :  ⌒    (similar to log, tunable)
γ=1     :  /    (identity)
γ>1     :  ⌣    (slow rise then fast)

Key insight: Power-law is more flexible (γ tunable); log is a fixed function giving strong dark expansion.

Applications:

Log → display Fourier magnitude spectrum

Power-law → calibrate monitor display (γ ≈ 2.2)

Exam Tip: Compare both formulas + γ < 1 vs γ > 1.

### Q15 (5-mark). Evaluate performance of Arithmetic Mean, Geometric Mean, Median, Contra-harmonic Mean for salt-and-pepper noise

Salt-and-pepper noise = isolated pixels with extreme values (0 or 255). Filter performance depends on how each filter handles outliers.

(a) Arithmetic Mean Filter — ❌ Poor

Average over neighborhood

Outliers (0/255) distort the average

Result: noise is blurred but not removed; salt becomes light spots, pepper becomes dark spots

(b) Geometric Mean Filter — ❌ Fails Catastrophically

(Π pixels)^(1/mn)

Pepper pixels (value 0) make the entire product zero → outputs zero

Result: black holes in image

(c) Median Filter — ✅ BEST

Sort and take middle value

Outliers appear at ends of sorted list and are discarded

Result: salt-and-pepper completely removed; edges preserved

Optimal for impulse noise

(d) Contra-harmonic Mean Filter — ⚠ Partial

Q > 0 removes pepper, Q < 0 removes salt

Single Q removes only one type, not both

Helpful for one variety but not simultaneously

Performance Ranking: Median (best) → Contra-harmonic Mean → Arithmetic Mean → Geometric Mean (worst)

Why Median wins:

Non-linear — outliers don't affect middle value

Edge-preserving

Computationally simple

Practical recommendation: Use Median filter (3×3 or 5×5) for salt-and-pepper noise.

Exam Tip: Rank filters and explain WHY each succeeds or fails.

## ====== MID-TERM EXAMINATION — SET II (25/10/2025, 2 hrs, 40 marks) ======

### Q1. Define 8-neighborhood

The 8-neighborhood N₈(p) of pixel p(x, y) consists of all 8 surrounding pixels — 4 horizontal/vertical + 4 diagonal.

[NW] [N] [NE]
[W]  [p] [E]
[SW] [S] [SE]

N₈(p) = N₄(p) ∪ N_D(p). Used in 8-connectivity and morphological operations.

Exam Tip: Show the 3×3 matrix labeling all 8 neighbors.

### Q2. Concept of a path between two pixels

A path from pixel p(x₀, y₀) to pixel q(xₙ, yₙ) is a sequence of distinct pixels (x₀, y₀), ..., (xₙ, yₙ) where each consecutive pair is adjacent (4-, 8-, or m-adjacent).

Types: 4-path, 8-path, m-path depending on adjacency used.

If p = q, the path is closed.

Used in region growing, connected component labeling, distance computation.

Exam Tip: Mention all 3 path types + closed path concept.

### Q3. Spatial domain in image processing

The spatial domain is the image plane itself where image processing is performed by direct manipulation of pixel intensities — no transform involved.

General form: g(x, y) = T[f(x, y)]

T = transformation operator (point operation or mask operation).

Examples: histogram equalization, convolution, gamma correction.

Exam Tip: State formula g(x,y) = T[f(x,y)] + give one example.

### Q4. Power-law (gamma) transformations

Power-law transformation maps input r to output s using s = c · r^γ.

Behavior:

γ < 1 → brightens image (boost shadows)

γ > 1 → darkens image (compress shadows)

γ = 1 → identity

Application: Gamma correction for displays (CRT γ ≈ 2.2; capture pre-applies γ ≈ 1/2.2 ≈ 0.45).

Exam Tip: Always give γ<1 vs γ>1 effects + display correction.

### Q5. Define a low-pass filter in the spatial domain

A spatial-domain low-pass filter is a smoothing mask that passes low frequencies and attenuates high frequencies (noise + edges).

Example 3×3 mean mask:

1 1 1
1/9 × [1 1 1]
       1 1 1

Each output pixel = average of neighborhood → blur + denoise.

Exam Tip: Mention "averages neighborhood → blur + denoise."

### Q6. Effect of high-pass filter on edges

A high-pass filter enhances edges by emphasizing high-frequency content (rapid intensity changes).

Effect:

Edges become brighter and more pronounced

Smooth regions become suppressed

Output contains mostly edge structure

Laplacian mask example:

0 -1  0
-1  4 -1
 0 -1  0

Used in edge detection (Sobel, Prewitt, Laplacian).

Exam Tip: Mention "enhance edges, suppress smooth" + one mask.

### Q7. Difference between image enhancement and image restoration

| Feature | Enhancement | Restoration |
| --- | --- | --- |
| Goal | Improve appearance | Recover original |
| Approach | Subjective | Objective (model-based) |
| Knowledge | Visual heuristic | Degradation model |
| Examples | Histogram eq., gamma | Inverse / Wiener filter |

Enhancement makes the image look better; restoration mathematically reverses a known degradation.

Exam Tip: Mention "subjective vs objective" + one example each.

### Q8. Explain the "Min filter"

The Min filter is an order-statistic filter that replaces each pixel with the minimum of its neighborhood.

Formula: f̂(x, y) = min { g(s, t) : (s, t) ∈ S_xy }

Best for: Removing salt noise (bright spots = 255), as salt is the maximum and gets replaced by smaller neighbors.

Side effect: Darkens the overall image.

Exam Tip: Pair "Min → salt, Max → pepper."

### Q9. "Blurring" as a form of degradation

Blurring reduces image sharpness — caused by:

Defocus (lens focused wrong)

Motion (camera or object movement)

Atmospheric turbulence

Optical aberrations

Model: g(x, y) = h(x, y) * f(x, y) — convolution with PSF h.

In frequency domain, blurring attenuates high frequencies (loss of fine detail).

Exam Tip: Mention "convolution with PSF" + 2 causes.

### Q10. Define restoration in the presence of noise only

Restoration in the presence of noise only means the degradation function H is identity — only additive noise affects the image.

Model: g(x, y) = f(x, y) + η(x, y)

Approach: Use spatial-domain filters (mean, median, geometric mean) chosen based on noise PDF:

Gaussian → Mean filter

Salt-and-pepper → Median filter

Salt only → Min filter

Pepper only → Max filter

Exam Tip: Quote model + filter–noise mapping.

### Q11 (5-mark). General model of image degradation/restoration with block diagram

Spatial domain: g(x, y) = h(x, y) * f(x, y) + η(x, y)

Frequency domain: G(u, v) = H(u, v) · F(u, v) + N(u, v)

Block Diagram:

η(x, y)  noise
                          ↓
   f(x, y) →  [ H ] → ⊕ → g(x, y) → [Restoration Filter] → f̂(x, y)

Components:

f(x, y) — original (unknown) image

H — degradation function (PSF)

η — additive noise

g — observed degraded image

f̂ — estimate of original

Assumptions:

H is linear and position-invariant (LSI)

Noise is independent of signal

H and noise statistics may be known (non-blind) or unknown (blind)

Restoration approaches:

| Filter | Knowledge Needed | Behavior |
| --- | --- | --- |
| Inverse | H | Simple division G/H; amplifies noise |
| Pseudo-inverse | H | Threshold limit to avoid blow-up |
| Wiener | H + S_η + S_f | Optimal MSE |
| Constrained LS | H + smoothness | Robust |

Goal: Minimize ‖f − f̂‖.

Exam Tip: Draw the block diagram + state both spatial and frequency formulas.

### Q12 (5-mark). Why restoring both blur and noise is harder

Three compounding issues:

(a) Noise amplification at zeros of H — Inverse filter F̂ = G/H = F + N/H; when |H(u,v)| is small (high frequencies where blur dominates), N/H explodes.

(b) Trade-off between sharpness and noise — Aggressive deblurring amplifies noise; conservative deblurring leaves blur. Need regularization.

(c) Frequency content overlap — Both noise (high freq) and signal high-frequency content reside in the same band; can't simply low-pass filter.

Wiener solution: F̂ = [H* / (|H|² + NSR)] · G

When NSR → 0 (no noise), Wiener → inverse filter

When NSR large (high noise), Wiener becomes attenuating

Why harder:

Need TWO pieces of info: H AND noise statistics

Regularization parameter must be tuned

Blind cases (unknown H) require iterative methods

Exam Tip: Mention "noise amplification + tradeoff + Wiener solution."

### Q13 (5-mark). Image formation model with diagram

Block Diagram:

[Illumination Source]
       ↓ (light rays)
[3D Scene / Object Surface]
       ↓ (reflected / transmitted light)
[Optical System (Lens)]
       ↓ (focused light)
[Image Sensor (2D Plane)]
       ↓ (sampled + quantized)
[Digital Image f(x, y)]

Components:

Illumination Source — provides electromagnetic energy (i(x, y), 0 < i < ∞)

3D Scene — reflects energy based on surface property r(x, y), 0 ≤ r ≤ 1

Image Formation Equation: f(x, y) = i(x, y) · r(x, y)

Range Constraint: L_min ≤ f(x, y) ≤ L_max

Optical System — focuses 3D scene onto 2D sensor (projection)

Image Sensor — CCD/CMOS converts photons to digital values

Examples of reflectance:

| Surface | r |
| --- | --- |
| Black velvet | 0.01 |
| Stainless steel | 0.65 |
| White wall | 0.80 |
| Snow | 0.93 |

Exam Tip: Use f = i × r formula + range constraint + reflectance table.

### Q14 (5-mark). Point vs Local vs Global processing

| Type | Scope | Examples |
| --- | --- | --- |
| Point processing | 1 pixel → 1 pixel | Negative, gamma, threshold |
| Local (mask) processing | Neighborhood → 1 pixel | Mean filter, Sobel, Laplacian |
| Global processing | Whole image → 1 pixel / whole | Histogram equalization, FFT, image statistics |

Point Processing:

Output depends only on input pixel at same location

Formula: s = T(r)

Examples: Negative (s=255−r), Gamma (s=cr^γ), Threshold (s=0 or L−1)

Fastest; per-pixel parallelizable

Local (Mask) Processing:

Output depends on a neighborhood of input pixels

Formula: g(x, y) = ΣΣ w(s, t) · f(x+s, y+t)

Examples: Mean filter (3×3 box), Sobel, Median

Mask defines effect

Global Processing:

Output uses information from all pixels

Examples: Histogram equalization (CDF computed globally), Fourier transform, image statistics

Computationally heavier

Exam Tip: Provide clear table + 1 example per category.

### Q15 (5-mark). Atmospheric turbulence blur restoration

Atmospheric Turbulence Blur occurs in long-exposure ground-based astronomical imaging when light passes through Earth's turbulent atmosphere.

Degradation Model:

Spatial: g(x, y) = h(x, y) * f(x, y) + η(x, y)

Frequency: H(u, v) = exp[−k · (u² + v²)^(5/6)] where k = turbulence severity

Properties of Turbulence PSF:

Radially symmetric (depends only on √(u²+v²))

High frequencies attenuated more

Worse with longer exposure

Restoration Pipeline:

Degraded g(x, y) → Estimate H(u, v) using k → Estimate noise S_η →
Apply Wiener filter → Inverse FFT → Restored f̂(x, y)

Why Wiener is best:

Atmospheric blur is strong at high frequencies — naive inverse blows up

Wiener automatically weights between inversion and attenuation via NSR

Atmospheric models give H; noise variance can be estimated

Alternatives:

Lucky imaging — keep sharp frames

Adaptive optics — physical correction

Speckle imaging — combine short exposures

Blind deconvolution — when H unknown

Conclusion: H(u, v) = exp[−k(u²+v²)^(5/6)] is the model; Wiener filter is the most effective restoration.

Exam Tip: Quote the H(u,v) formula + recommend Wiener.

## ====== DEPARTMENTAL INTERNAL TEST — DIT 1 (Set II) — 17/02/2026 ======

### Q1. What is image processing?

Image processing is the manipulation of a digital image f(x, y) using computer algorithms for enhancement, restoration, analysis, compression, or interpretation.

Three levels:

Low-level (preprocessing): smooth, sharpen

Mid-level (segmentation, classification)

High-level (interpretation, cognition)

Used in medical, remote sensing, robotics, security.

### Q2. Define image quantization

Image quantization is the discretization of intensity values of an image — converting continuous brightness into a finite set of discrete gray levels.

For k-bit quantization: L = 2^k levels (e.g., 8-bit → 256)

Too few levels → false contouring

Exam Tip: Always state L = 2^k.

### Q3. Explain digital image with example

A digital image is a 2D matrix f(x, y) where:

(x, y) = discrete spatial coordinates

f(x, y) = discrete gray-level value

Example 3×3 image:

f = [ 100  120  150 ]
    [ 110  130  160 ]
    [  90  140  170 ]

At position (2, 3), f = 160. Digital images are produced via sampling + quantization from continuous images.

### Q4. What is image sensor?

An image sensor is a hardware device that converts incident photons (light) into an electrical signal corresponding to scene brightness.

Types:

CCD (Charge-Coupled Device) — high quality, more expensive

CMOS (Complementary Metal-Oxide Semiconductor) — cheaper, low-power, mobile

Specialized: X-ray, IR, UV sensors

Each photosite = 1 pixel; output digitized via A/D converter.

### Q5. What does f(x, y) represent in a digital image?

f(x, y) is the 2D image function where:

x, y = spatial coordinates (row, column) — pixel position

f = intensity / gray-level value at that position

For 8-bit gray image: 0 ≤ f(x, y) ≤ 255

For color image: f(x, y) = (R, G, B)

### Q6. What is reflectance?

Reflectance r(x, y) is the fraction of illumination reflected by an object's surface at point (x, y).

Range: 0 ≤ r(x, y) ≤ 1

r = 0 → total absorption (black)

r = 1 → total reflection (snow, polished silver)

Used in image formation: f(x, y) = i(x, y) · r(x, y)

### Q7. Effect of low sampling rate

A low sampling rate causes:

Aliasing — high frequencies fold into low → false patterns / moiré

Pixelation — blocky / staircase appearance

Loss of fine detail — small structures disappear

Reduced spatial resolution — blurry image

To avoid: sample at ≥ 2 × highest frequency (Nyquist criterion).

Exam Tip: Always mention Nyquist + aliasing.

### Q8 (5-mark). Components of a digital image processing system

A DIP system has 8 components:

Image Sensors — capture light (CCD, CMOS, X-ray)

Specialized Hardware — frame grabber, ALU, GPU

Computer (Host) — general-purpose PC

Image Processing Software — MATLAB, OpenCV, ImageJ

Mass Storage — RAM (short-term), HDD (online), tape/cloud (archival)

Image Display — Monitor, LCD/LED, projector

Hardcopy Device — printer, plotter

Network — transmit images

Block Diagram:

[Image Sensor]→[Specialized HW]→[Computer]→[DIP Software]
                                              ↓
[Mass Storage]←→[Display]←→[Hardcopy]←→[Network]

Knowledge base spans all components, providing domain expertise.

Exam Tip: Draw block diagram + describe each block in 1-2 lines.

### Q9 (5-mark). Compare sampling and quantization with examples

Comparison Table:

| Feature | Sampling | Quantization |
| --- | --- | --- |
| Acts on | Spatial coords (x, y) | Intensity amplitude f |
| Determines | Spatial resolution | Intensity (gray-level) resolution |
| Parameter | M × N | L = 2^k levels |
| Low-value problem | Aliasing, pixelation | False contouring |
| High-value benefit | More detail | Smoother gradients |

Examples:

Sampling example: A photo sampled at 1024×1024 vs 64×64 — the high-res shows fine textures; the low one looks blocky.

Quantization example: A face image with 256 gray levels looks smooth; the same with 4 levels shows visible bands (false contouring).

Storage impact: 512×512 8-bit image = 256 KB; 4-bit version ≈ 128 KB.

Conclusion: Sampling → spatial dimension; Quantization → amplitude dimension; both required for a digital image.

Exam Tip: Use clear table + 1 numerical example each.

## ====== DEPARTMENTAL INTERNAL TEST — DIT 2 (Set I) — 21/04/2026 ======

### Q1. Define image compression

Image compression is the process of reducing the data size required to store or transmit an image — exploiting redundancies in the data.

Two types:

Lossless — exact recovery (PNG, GIF)

Lossy — approximate, higher CR (JPEG, MPEG)

### Q2. What does HSI stand for?

HSI stands for Hue, Saturation, Intensity — a perceptual color model:

Hue — dominant wavelength (color type)

Saturation — purity of color

Intensity — brightness

Decouples color from brightness → useful for image processing.

### Q3. Explain lossy compression

Lossy compression reduces image data size by discarding information not perceptually significant — original cannot be exactly recovered.

Trade-off: Higher CR (10:1–100:1) at cost of some quality.

Examples: JPEG, MPEG, WebP, MP3 (audio).

Techniques: DCT + quantization, wavelet, vector quantization.

### Q4. Explain closing operation

Closing is a morphological operation: dilation followed by erosion.

Formula: A • B = (A ⊕ B) ⊖ B

Effects:

Fills small holes in objects

Connects nearby objects

Smooths concave contours

Preserves overall shape

Useful in OCR (fill character gaps), noise removal (pepper noise).

### Q5. Define boundary extraction in morphological processing

Boundary extraction finds the edge pixels of objects in a binary image:

Formula: β(A) = A − (A ⊖ B)

Erode A by SE B → object shrinks

Subtract from original → only boundary pixels remain

Boundary thickness = (size of SE − 1) / 2

### Q6. Primary and secondary colors of light

Primary colors of light (additive):

Red, Green, Blue (RGB) — combine to form white

Secondary colors (mixtures of two primaries):

Cyan = G + B

Magenta = R + B

Yellow = R + G

Adding all three → white; adding none → black.

Used in monitors, cameras, image sensors.

### Q7. Need for image compression in modern digital systems

Storage — Modern devices store thousands of images

Bandwidth — Internet transmission is bandwidth-limited

Streaming — Real-time video delivery

Cost — Storage and transmission cost money

User experience — Faster downloads, less buffering

Without compression, 4K raw images (~25 MB) would be impractical for web; with JPEG → ~2 MB.

### Q8 (5-mark). Mathematical relationship between RGB and HSI

RGB → HSI Conversion:

I = (R + G + B) / 3
S = 1 − [ 3 · min(R, G, B) / (R + G + B) ]
θ = arccos { [½((R−G) + (R−B))] / √((R−G)² + (R−B)(G−B)) }
H = θ        if B ≤ G
H = 360 − θ  if B > G

HSI → RGB (three sectors based on H):

Sector RG (0° ≤ H < 120°):

B = I · (1 − S)

R = I · [1 + (S · cos H) / cos(60 − H)]

G = 3I − (R + B)

Sector GB (120° ≤ H < 240°): H' = H − 120, similar formulas

Sector BR (240° ≤ H < 360°): H' = H − 240, similar formulas

Challenges near saturation axis:

Hue undefined when S = 0 (pure gray)

Numerical instability near S = 0

Out-of-gamut after computation (clip to [0, 1])

Sector boundary errors at 0°, 120°, 240°

Solutions: Use double precision, add ε tolerance, clamp output.

Exam Tip: Write all conversion formulas + 3 numerical issues.

### Q9 (5-mark). Explain closing operation in detail

Closing A • B = (A ⊕ B) ⊖ B is dilation followed by erosion.

Step-by-step example (3×3 SE):

Original A:        After Dilation (A ⊕ B):
1 0 1              1 1 1
0 0 0              1 1 1
1 0 1              1 1 1

After Erosion = A • B
1 1 1
1 1 1   (Small holes filled)
1 1 1

Properties:

Extensive: A ⊆ A • B

Idempotent: (A • B) • B = A • B

Increasing

Translation-invariant

Effects:

Fills small holes (smaller than SE)

Connects nearby objects (gaps smaller than SE)

Smooths concave contours

Removes pepper noise (dark spots inside objects)

Applications:

OCR — connect broken character strokes

Medical imaging — fill small holes in tissue segmentation

Industrial inspection — fill voids in product images

Satellite imagery — connect fragments of land features

Noise removal — remove pepper-like noise

Opening vs Closing:

| Feature | Opening | Closing |
| --- | --- | --- |
| Sequence | Erode → Dilate | Dilate → Erode |
| Removes | Small bright objects | Small dark holes |
| Effect | Smooth outside | Smooth inside |

Exam Tip: Define formula + show step-by-step example + list 4+ applications.

## ====== PROBLEM SET I — UNIT I ======

### Q1. f(4, 6) = 180 — what does this mean?

Identification:

Coordinates (x, y) = (4, 6) → pixel at row 4, column 6

f(4, 6) = 180 → intensity at that pixel

Interpretation:

For an 8-bit grayscale image (range 0–255):

0 = black

255 = white

180 = light gray (~70% of max)

Percentage = 180/255 ≈ 70.6%

The pixel has a brightness of 180 out of 255 — a fairly bright gray pixel.

Exam Tip: Always state range (0–255), interpret as percentage.

### Q2. Effect of very low sampling rate

Problem: Aliasing (along with pixelation and loss of detail).

Justification (Nyquist theorem): Sampling frequency must be ≥ 2× highest frequency in the image.

If violated:

Aliasing — high frequencies fold back into low → false patterns, moiré

Pixelation — blocky look

Loss of fine detail — small structures unrecognizable

Reduced spatial resolution — blurry image

Visual effect with decreasing sampling rate:

512×512 → clear

128×128 → slight blur

64×64 → visible pixelation

32×32 → severe aliasing

Solution: Increase sampling rate or apply anti-aliasing filter before sampling.

Exam Tip: Always cite Nyquist theorem + aliasing.

### Q3. Sampling vs Quantization for resolution

| Feature | Sampling | Quantization |
| --- | --- | --- |
| (a) Spatial resolution | ✅ Affects | ✗ Does not |
| (b) Intensity resolution | ✗ Does not | ✅ Affects |
| Parameter | M × N | L = 2^k |
| Low-value problem | Aliasing | False contouring |

(a) Spatial resolution = smallest discernible spatial detail; determined by sampling.

(b) Intensity resolution = smallest discernible intensity change; determined by quantization.

Examples:

256×256 with 256 gray levels → moderate spatial, full intensity

1024×1024 with 4 levels → high spatial, poor intensity

64×64 with 256 levels → low spatial, good intensity

Exam Tip: Match sampling↔spatial, quantization↔intensity.

### Q4. Satellite image blurred despite high-resolution sensors

Model: g(x, y) = h(x, y) * f(x, y) + η(x, y)

Possible Responsible Stages:

Atmospheric turbulence — H(u,v) = exp[−k(u²+v²)^(5/6)] — most common

Motion blur — satellite movement during exposure

Defocus / optical aberrations — imperfect lens

Sensor response — finite pixel area integration

Compression artifacts — JPEG block artifacts

Resampling — geometric correction

Most likely culprit: Atmospheric turbulence + motion blur during long exposure.

Solutions:

Pre-launch: better optics, adaptive optics

Post-processing: Wiener filter, blind deconvolution, deep-learning deblurring

Exam Tip: List all stages systematically and rank likelihood.

### Q5. Convert continuous image to digital step-by-step

Step 1 — Continuous Image f_c(x, y) — captured by sensor with continuous coords/amplitude.

Step 2 — Sampling (discretize coordinates):

Grid with spacing (Δx, Δy)

f(m, n) = f_c(m·Δx, n·Δy) for m = 0..M-1, n = 0..N-1

Determines spatial resolution

Step 3 — Quantization (discretize amplitude):

Map continuous intensity to L = 2^k discrete levels

f_q = round((f / max) × (L − 1))

Determines intensity resolution

Step 4 — Storage as 2D matrix:

Total bits = M · N · k

Step 5 — Output Digital Image f(x, y) — fully discrete; ready for processing.

Pipeline:

Continuous Image → Sampling (discretize x, y) →
   Quantization (discretize f) → Digital Matrix f(m, n)

Example: 8.5×11 inch image at 300 DPI = 2550×3300 pixels; 8-bit gray → ~8.4 MB.

Exam Tip: Always explain "sampling = spatial, quantization = amplitude."

### Q6. 4-connectivity vs 8-connectivity (pixel with diagonal + horizontal neighbors)

The pixel has neighbors in both horizontal/vertical AND diagonal directions:

Horizontal/vertical → 4 neighbors (N₄)

Diagonal → 4 neighbors (N_D)

Combined → 8 neighbors total

Verdict: 8-connectivity (N₈)

[NW] [N] [NE]
[W]  [p] [E]
[SW] [S] [SE]

4-connectivity uses only horizontal + vertical (4 neighbors, + pattern)

8-connectivity adds diagonals (8 neighbors, square pattern)

m-connectivity refines 8-connectivity to eliminate path ambiguity

Exam Tip: Count neighbors → if 8 → 8-connectivity.

### Q7. Why low quantization causes false contouring; how to reduce

Why it occurs:

Smooth intensity changes require many gray levels to look continuous

With few quantization levels (e.g., 4), adjacent areas get same value

Where different regions get different quantized values, a visible boundary (ridge) appears

Human eye is highly sensitive to these artificial edges

Visual Example:

| Original | 0 | 50 | 100 | 150 | 200 | 255 |
| --- | --- | --- | --- | --- | --- | --- |
| 8-bit (256 lv) | 0 | 50 | 100 | 150 | 200 | 255 |
| 4-bit (16 lv) | 0 | 48 | 96 | 144 | 192 | 240 |
| 2-bit (4 lv) | 0 | 0 | 85 | 170 | 170 | 255 |

How to reduce:

Increase quantization levels (move from 4-bit to 8-bit)

Dithering — add random noise before quantization

Error diffusion (Floyd-Steinberg) — propagate error to neighbors

Halftoning — pattern of dots

Companding — non-uniform quantization

Exam Tip: Show "few levels → ridges" + dithering as solution.

### Q8. Image formation model — illumination + reflectance forming pixel intensity

Formula: f(x, y) = i(x, y) × r(x, y)

Components:

i(x, y) — illumination from source (0 < i < ∞)

r(x, y) — reflectance of surface (0 ≤ r ≤ 1)

How they combine:

Bright light + reflective surface → bright pixel

Bright light + dark surface → medium pixel

Dim light + reflective surface → medium pixel

Dim light + dark surface → dark pixel

Examples:

| Scene | i (lm/m²) | r | f = i × r |
| --- | --- | --- | --- |
| Snow in sunlight | 90,000 | 0.93 | 83,700 |
| Black coal in sunlight | 90,000 | 0.01 | 900 |
| White wall in office | 1,000 | 0.80 | 800 |
| Black wall in office | 1,000 | 0.05 | 50 |

Range: L_min = i_min · r_min ≤ f ≤ L_max = i_max · r_max — after mapping to 0–255, this becomes the gray scale.

Conclusion: The model cleanly separates the physics of light (illumination) from the physics of surface (reflectance).

Exam Tip: State formula + give range + 1 numerical example.

### Q9. Image with dark histogram — analyze and suggest enhancement

Analysis:

Most pixels are 0–80 → image is dark / under-exposed

Low contrast — narrow intensity range

Details in bright regions barely visible

Image looks dim or shadowy

Suggested Technique: Histogram Equalization

Reason:

Automatically redistributes pixels across full intensity range

Stretches the narrow dark histogram into uniform distribution

Increases contrast without manual tuning

Formula: s_k = (L − 1) · cdf(r_k)

Alternatives:

Power-law (Gamma) with γ < 1 — specifically brightens dark regions

Log transformation — s = c · log(1 + r) for very wide ranges

Linear contrast stretching — map [r_min, r_max] linearly to [0, 255]

Best choice: Histogram equalization for general under-exposed images.

Exam Tip: State problem + suggest HE + provide formula.

### Q10. Meaning of x, y, f(x, y)

x = horizontal coordinate (column index in image matrix); discrete integer 0 ≤ x ≤ M−1

y = vertical coordinate (row index); discrete integer 0 ≤ y ≤ N−1

f(x, y) = intensity (gray-level) at pixel (x, y); for 8-bit, 0 ≤ f ≤ 255

For color image, f(x, y) = (R, G, B).

Example: f(2, 3) = 150 means at row 2, column 3, pixel has gray level 150.

Exam Tip: Give all 3 meanings + example f(2, 3) = 150.

## ====== PROBLEM SET II — UNIT II ======

### Q1. "Spatial domain" in image processing

The spatial domain is the image plane itself where image processing is done by direct manipulation of pixel intensities.

General form: g(x, y) = T[f(x, y)]

T = transformation operator — point operation (single pixel) or mask operation (neighborhood).

Categories:

Point processing: Negative, gamma, threshold

Mask processing: Mean filter, median, Sobel, Laplacian

Global ops: Histogram equalization, image stats

Contrast with frequency domain: Frequency-domain processing transforms image via DFT, multiplies in Fourier space, then inverts.

Advantages of spatial domain: Fast for small kernels, simpler to understand, direct pixel control.

### Q2. Histogram equalization vs histogram matching goals

| Feature | Histogram Equalization | Histogram Matching |
| --- | --- | --- |
| Goal | Maximize global contrast | Produce specified histogram |
| Output histogram | Uniform (flat) | User-defined shape |
| Function | T(r) = (L−1)·cdf(r) | G⁻¹(T(r)) |
| User control | None (automatic) | Full (specify target) |
| Flexibility | Low | High |
| Use case | General enhancement | Match reference image |

HE always produces a uniform output; HM allows the user to choose any target distribution (useful when uniform isn't ideal, e.g., medical images often need bimodal contrast).

### Q3. Negative of grayscale image — math expression

Formula: s = L − 1 − r

Where:

L = number of gray levels (e.g., 256 for 8-bit)

r = input pixel intensity

s = output pixel intensity

For 8-bit: s = 255 − r

Examples:

| r | s = 255 − r |
| --- | --- |
| 0 | 255 |
| 100 | 155 |
| 128 | 127 |
| 200 | 55 |
| 255 | 0 |

Application: Enhancing white details on dark backgrounds (X-rays).

### Q4. Histogram equalization step-by-step + justify contrast enhancement

Algorithm:

Compute histogram: h(r_k) = n_k

Normalize: p_r(r_k) = n_k / MN

Compute CDF: cdf(r_k) = Σ_{j=0..k} p_r(r_j)

Map pixels: s_k = round((L − 1) · cdf(r_k))

Replace each pixel.

Numerical Example (3-bit, 64 pixels):

| r_k | n_k | p | cdf | (L-1)·cdf | s_k |
| --- | --- | --- | --- | --- | --- |
| 0 | 8 | 0.125 | 0.125 | 0.875 | 1 |
| 1 | 10 | 0.156 | 0.281 | 1.97 | 2 |
| 2 | 10 | 0.156 | 0.437 | 3.06 | 3 |
| 3 | 2 | 0.031 | 0.469 | 3.28 | 3 |
| 4 | 12 | 0.187 | 0.656 | 4.59 | 5 |
| 5 | 16 | 0.250 | 0.906 | 6.34 | 6 |
| 6 | 4 | 0.063 | 0.969 | 6.78 | 7 |
| 7 | 2 | 0.031 | 1.000 | 7.00 | 7 |

Justification — How HE Enhances Contrast:

Stretches narrow histograms across full range

Maps frequent gray levels to wider output range

Compresses rare gray levels

Final output approx uniform → maximum global contrast

Limitations:

May enhance noise

May produce unnatural look

Local features can be lost

### Q5. Smoothing vs sharpening filters

| Feature | Smoothing | Sharpening |
| --- | --- | --- |
| Frequency response | Low-pass | High-pass |
| Effect | Blurs, reduces noise | Enhances edges, fine details |
| Math basis | Average / weighted sum | Derivatives (1st, 2nd order) |
| Typical kernels | Mean, Gaussian, median | Sobel, Prewitt, Laplacian |
| Edge effect | Blurred | Enhanced |
| Noise effect | Reduces | Amplifies |
| Use | Pre-processing, denoise | Edge detection, feature extraction |

Objectives:

Smoothing: Reduce noise, remove fine details, pre-process for segmentation, produce uniform regions.

Sharpening: Enhance edges, highlight fine structures, feature extraction, OCR pre-processing.

Example Kernels:

3×3 Mean (smooth):       Laplacian (sharpen):
       1 1 1               0 -1  0
1/9 × [1 1 1]             -1  4 -1
       1 1 1               0 -1  0

### Q6. Arithmetic operations — applications in noise reduction, motion detection, masking

Addition (s = f + g):

Noise reduction by averaging: ḡ = (1/K) Σ g_i → variance reduces by 1/K

Long-exposure simulation

Double exposure (artistic)

Subtraction (s = f − g):

Motion detection — subtract consecutive frames; moving objects appear

Background subtraction — remove static background

Digital Subtraction Angiography (DSA) — subtract pre/post contrast medical images

Multiplication (s = f × g):

Masking — binary mask × image to extract ROI

Shading correction — multiply by inverse illumination pattern

Watermarking

Division (s = f / g):

Shading correction — divide image by illumination pattern

Flat-field correction in microscopy

Calibration — normalize against reference

### Q7. Logical operations — truth tables + 3×3 examples + applications

Truth Table:

| p | q | AND | OR | XOR | NAND | NOR |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 | 1 | 0 |
| 1 | 0 | 0 | 1 | 1 | 1 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 |

Example 3×3 matrices:

A = 1 0 1     B = 1 1 0
    0 1 0         0 1 1
    1 1 0         1 0 0

AND: ROI extraction, intersection

1 0 0
0 1 0
1 0 0

OR: Region union

1 1 1
0 1 1
1 1 0

NOT(A): Complement

0 1 0
1 0 1
0 0 1

XOR: Change detection

0 1 1
0 0 1
0 1 0

Applications:

AND → masking, intersection

OR → region union

NOT → complement

XOR → change detection

NAND/NOR → logic completeness

### Q8. Hospital MRI: contrast stretch + HE + noise reduction — details lost — analyze

Analysis: HE automatically maximizes global contrast but can lose subtle local details — critical issue for medical imaging.

Advantages of HE in Medical Images:

Automatic — no parameters

Global contrast enhancement

Computationally cheap

Reproducible

Easy to implement

Disadvantages of HE in Medical Images:

Over-enhancement of noise — random noise amplified

Loss of subtle details — small intensity differences (tumors, micro-calcifications) merged

Unrealistic appearance

Global approach — doesn't adapt locally

Saturation — bright/dark may saturate

Sensitive to outliers

Suggested Improvements:

CLAHE (Contrast-Limited Adaptive Histogram Equalization) — applies HE on small tiles with clipping; gold standard for medical

AHE (Adaptive HE) — local HE on sliding windows

Histogram Matching to a reference good image

Gamma Correction — specific γ for ROI

Wavelet-based enhancement — preserves details at all scales

Denoising before HE — non-local means or BM3D

Recommended: CLAHE + denoise pre-processing.

### Q9. Compare 4 noise removal filters (Arithmetic, Geometric, Median, Gaussian)

| Feature | Arithmetic Mean | Geometric Mean | Median | Gaussian |
| --- | --- | --- | --- | --- |
| Formula | (1/mn)·Σg | (Πg)^(1/mn) | median{g} | weighted avg |
| Linearity | Linear | Non-linear | Non-linear | Linear |
| Best for | Gaussian, uniform | Gaussian (less blur) | Salt-and-pepper | Gaussian |
| Edge effect | Strong blur | Less blur | Preserves | Mild blur |
| Zero-pixel issue | None | Fails | None | None |
| Computational cost | O(mn) | O(mn) | O(mn·log(mn)) | O(mn) |

Quick recommendation:

| Noise Type | Best Filter |
| --- | --- |
| Salt-and-pepper | Median |
| Gaussian | Gaussian or Geometric mean |
| Uniform | Arithmetic mean |
| Mixed | Alpha-trimmed mean |

### Q10. Image gradient + Sobel magnitude and direction

Image gradient ∇f is a 2D vector representing the direction of greatest intensity change at each pixel.

Gradient vector:

∇f = [ ∂f/∂x ]
     [ ∂f/∂y ]

Magnitude (edge strength):

|∇f| = √((∂f/∂x)² + (∂f/∂y)²) ≈ |G_x| + |G_y|

Direction (gradient angle):

θ = arctan(G_y / G_x)

Edge direction is perpendicular to ∇f.

Sobel Masks:

G_x = [-1  0  1]      G_y = [-1 -2 -1]
      [-2  0  2]            [ 0  0  0]
      [-1  0  1]            [ 1  2  1]

Example (3×3 patch):

f = 50  60  70
    55  65  75
    60  70  80

G_x = (-50 + 70) + (-110 + 150) + (-60 + 80) = 80
G_y = (-50 - 120 - 70) + 0 + (60 + 140 + 80) = 40
|∇f| = √(80² + 40²) = √8000 ≈ 89.4
θ = arctan(40/80) = arctan(0.5) ≈ 26.6°

Applications: Edge detection (Canny uses Sobel), feature extraction, segmentation.

## ====== PROBLEM SET III — UNIT III ======

### Q1. Image enhancement vs restoration (with examples)

(Same as Mid-Term Set I Q11 — see above)

Comparison Table:

| Feature | Enhancement | Restoration |
| --- | --- | --- |
| Goal | Subjective | Objective |
| Approach | Heuristic | Mathematical |
| Knowledge | Visual | Degradation model |
| Examples | Histogram, gamma | Inverse, Wiener |

Enhancement example: Brightening with γ = 0.5.

Restoration example: Wiener filter on motion-blurred satellite image.

### Q2. Min filter — which noise reduced

Min filter: f̂(x, y) = min { g(s, t) : (s, t) ∈ S_xy }

Replaces each pixel with smallest neighborhood value.

Noise reduced: Salt noise (bright spots = 255).

Why? Salt pixels are maximum. Min filter picks darker neighbors → salt removed.

Side effects:

Darkens image overall

Erodes bright objects (like morphological erosion on grayscale)

### Q3. Motion blur — mathematical model

(Same as Mid-Term Set I Q9 — see above)

Spatial PSF: h(x, y) = 1/L if 0 ≤ x ≤ L, y = 0; else 0

Frequency: H(u, v) = (T / π(ua + vb)) · sin(π(ua + vb)) · exp(−jπ(ua + vb))

Parameters: L (length), θ (angle), T (exposure time).

Causes: Camera shake, object motion, long exposure.

### Q4. Max filter — salt or pepper?

Max filter: f̂(x, y) = max { g(s, t) }

Effective for: Pepper noise (dark spots = 0)

Why? Pepper pixels are 0 (minimum). Max picks brightest neighbor → pepper replaced.

NOT effective for salt — salt pixels are 255 (max), Max filter preserves them.

Side effects: Brightens image, dilates bright regions.

### Q5. Prior knowledge in restoration

Without H (blind restoration): must estimate H from observed image — challenging.

With H (non-blind restoration):

Enables inverse filtering: F̂ = G/H

Enables Wiener filtering (with noise stats)

Enables constrained least squares

Enables choosing filter type (motion-specific, defocus-specific)

Avoids wasted iterations of blind methods

Sources of prior knowledge:

Camera calibration data

Atmospheric models (astronomy)

Optical system characteristics

Motion sensors (gyroscope readings)

Calibration targets

Conclusion: Prior knowledge of H transforms restoration from blind guessing to mathematical inversion.

### Q6. Restoration with both blur and noise — complexity comparison

| Case | Complexity | Approach | Risk |
| --- | --- | --- | --- |
| Noise only | Low | Spatial filter (median) | Mild edge blur |
| Blur only | Low–Medium | Inverse filter | None if H known |
| Blur + Noise | High | Wiener / CLS | Severe noise amplification |

Difficulties with both:

Noise amplification — inverse filter F̂ = G/H + N/H; small |H| → N/H explodes

Frequency overlap — noise (high freq) and edge content coexist

Need multiple parameters — H, S_η, S_f, regularization

Trade-off required — aggressive deblur amplifies noise

Blind cases are worst — H unknown

Conclusion: Blur + noise is hardest case; needs regularized restoration (Wiener / CLS).

### Q7. Mathematical model for motion blur

Spatial PSF for uniform linear motion (length L):

h(x, y) = 1/L for 0 ≤ x ≤ L, y = 0; 0 otherwise

Frequency domain:

H(u, v) = (T / π(ua + vb)) · sin(π(ua + vb)) · exp(−jπ(ua + vb))

Parameters:

Length L — distance traversed during exposure

Angle θ — direction of motion

Exposure time T — longer T = more accumulated motion

Motion type — linear uniform, non-uniform, rotational

Restoration: Estimate L and θ → construct H → apply inverse / Wiener filter.

### Q8. Comprehensive image degradation/restoration model with block diagram

Block Diagram:

η(x, y) noise
                              ↓
   f(x,y) → [ H ] → ⊕ → g(x,y) → [Restoration Filter] → f̂(x,y)

Components:

f(x, y) — original (unknown)

H — linear, position-invariant degradation

η(x, y) — additive noise

g(x, y) — observed degraded image

Restoration filter — recovery operator

f̂(x, y) — estimate

Mathematical Derivation:

Step 1 — Spatial: g = h * f + η

Step 2 — Frequency (FT): G = H · F + N

Step 3 — Inverse restoration: F̂ = G/H = F + N/H (noise amplified)

Step 4 — Wiener restoration: F̂ = [|H|² / (|H|² + S_η/S_f)] · G/H

Behavior:

High SNR (S_η/S_f → 0): Wiener → inverse

Low SNR: Wiener → 0 (suppress)

Assumptions: LSI, additive noise, signal-independent.

### Q9. Inverse vs Wiener — when does Wiener become pseudo-inverse?

(Same as Unit III topic — see SYLLABUS_NOTES)

Inverse: F̂ = G/H (amplifies noise where |H| ≈ 0)

Wiener: F̂ = [H*/(|H|² + S_η/S_f)] · G

Wiener → Pseudo-Inverse condition: When NSR → 0 (no noise):

F̂ = [H* / (|H|² + 0)] · G = (1/H) · G

Pseudo-inverse filter: Modified inverse that clips when |H| is very small:

F̂ = G/H if |H| > threshold, else 0

Conclusion: Wiener generalizes inverse to include noise; without noise, reduces to inverse/pseudo-inverse.

### Q10. Different noise types with PDFs

Gaussian: p(z) = (1/σ√2π) · exp(−(z − μ)² / 2σ²) — sensors/electronic

Rayleigh: p(z) = (2/b)(z − a) · exp(−(z − a)²/b) — range imaging

Erlang: p(z) = (a^b · z^(b−1) / (b−1)!) · exp(−az) — laser

Exponential: p(z) = a · exp(−az) — laser (special case)

Uniform: p(z) = 1/(b − a) for a ≤ z ≤ b — simulation

Impulse (Salt-and-pepper): p(z) = P_a at z=a (pepper), P_b at z=b (salt)

| Noise | Source | Best Filter |
| --- | --- | --- |
| Gaussian | Sensor / electronic | Mean / Gaussian |
| Salt-pepper | Transmission | Median |
| Rayleigh | Range imaging | Geometric mean |
| Erlang | Laser | Geometric mean |
| Uniform | Simulation | Mean |

## ====== PROBLEM SET IV — UNIT IV ======

### Q1. Three color representations

RGB (Red, Green, Blue) — additive model for monitors, cameras

HSI (Hue, Saturation, Intensity) — perceptual model

CMY / CMYK (Cyan, Magenta, Yellow, Key/Black) — subtractive model for printing

Other valid: YCbCr (video, JPEG), YIQ (NTSC), Lab (perceptually uniform).

### Q2. Intensity component in HSI

Intensity (I) in HSI is the brightness component — how much gray/luminance the color has.

Formula: I = (R + G + B) / 3

Range: 0 (black) ≤ I ≤ 1 (white)

Properties:

Determines overall brightness

Independent of hue and saturation

Plot along central axis of HSI cone

Difference from H and S:

| Component | Meaning | Range | Controls |
| --- | --- | --- | --- |
| Hue (H) | Dominant wavelength | 0°–360° | Color type |
| Saturation (S) | Color purity | 0 – 1 | Vividness |
| Intensity (I) | Brightness | 0 – 1 | Darkness/lightness |

### Q3. Define color model — primary purpose

A color model is a mathematical system for representing colors as numerical tuples (usually 3 or 4 components).

Examples: RGB, HSI, CMYK, YCbCr, Lab

Primary Purposes:

Standardization — unambiguous color specification

Device compatibility — RGB for monitors, CMYK for printers

Image storage/transmission — YCbCr for compression

Processing convenience — HSI for manipulation

Color conversion between devices

Color analysis (segmentation, recognition)

### Q4. Saturation component in HSI — relation to purity

Saturation (S) measures purity of color — degree to which color is diluted by white light.

Formula: S = 1 − [3 · min(R, G, B) / (R + G + B)]

Range: 0 (gray) ≤ S ≤ 1 (pure)

Relation to purity:

High S (→1): Pure, fully saturated color

Low S (→0): Diluted, mixed with white

S = 0: Pure gray (no hue)

Examples:

| Color | R | G | B | S |
| --- | --- | --- | --- | --- |
| Pure red | 1.0 | 0.0 | 0.0 | 1.0 |
| Pink | 1.0 | 0.7 | 0.7 | 0.30 |
| Light red | 1.0 | 0.5 | 0.5 | 0.50 |
| Gray | 0.5 | 0.5 | 0.5 | 0.0 |

### Q5. Color quantization technique

Color quantization reduces the number of distinct colors in an image — from millions (24-bit, 16.7M) to a small palette (e.g., 256).

Algorithms:

Uniform quantization — equal subdivision (poor for natural images)

Median-cut — recursive split of color cube

Octree — tree-based subdivision

K-means clustering — cluster similar colors

Situations requiring it:

GIF format (256 colors max)

Older 8-bit displays (VGA)

Limited storage (IoT)

Image compression

Vector quantization step

Web optimization

Trade-off: Smaller palette → smaller file but color banding.

### Q6. Importance of image compression — 2 application areas

Importance:

Storage efficiency (4K raw 25 MB → JPEG 2 MB)

Bandwidth savings

Cost reduction

Real-time streaming

Mobile devices

Application Area 1 — Web Browsing / Streaming:

Need: Fast image/video load for billions

Solution: JPEG, WebP, MP4 (H.264, H.265)

CR: 10:1 to 100:1

Impact: Without compression, websites would load 50× slower

Application Area 2 — Medical Imaging Archives:

Need: Store millions of patient scans for years

Solution: JPEG 2000 (lossy + lossless modes)

CR: 5:1 to 20:1 for lossless

Impact: 80% reduction in storage costs

### Q7. RGB color cube — primary and secondary colors

RGB Color Cube Visualization:

White (1, 1, 1)
        /|
       / |
   Yellow ── Cyan
    /|     /|
   / |    / |
 Red ── Magenta ── Blue
   \  |   /
    \ |  /
   Black (0, 0, 0)

Primary Colors (single axis = 1):

| Color | R | G | B |
| --- | --- | --- | --- |
| Red | 1 | 0 | 0 |
| Green | 0 | 1 | 0 |
| Blue | 0 | 0 | 1 |

Secondary Colors (two axes = 1):

| Color | R | G | B | Combination |
| --- | --- | --- | --- | --- |
| Yellow | 1 | 1 | 0 | R + G |
| Cyan | 0 | 1 | 1 | G + B |
| Magenta | 1 | 0 | 1 | R + B |

Extremes: Black (0,0,0), White (1,1,1).

Gray line: Main diagonal from black to white (R = G = B).

24-bit RGB → 256³ = 16,777,216 distinct colors.

### Q8. Lossless vs Lossy compression — comparison

| Feature | Lossless | Lossy |
| --- | --- | --- |
| Data recovery | Bit-exact | Approximate |
| Quality loss | None | Some (controlled) |
| CR | 2:1 to 5:1 | 10:1 to 100:1 |
| Techniques | Huffman, RLE, LZW, Arithmetic, DPCM | DCT, Wavelet, VQ |
| Formats | PNG, GIF, TIFF | JPEG, MPEG, WebP, HEIC |
| Use cases | Medical, legal, text | Web, video, mobile |

Use Cases:

| Domain | Lossless | Lossy |
| --- | --- | --- |
| Medical imaging | ✓ Diagnosis | ✗ Loss affects diagnosis |
| Legal evidence | ✓ Court admissible | ✗ Tampering risk |
| Photographs | Optional | ✓ Smaller files |
| Video streaming | ✗ Too large | ✓ Bandwidth limited |

### Q9. Color constancy — modern cameras

Color constancy = ability to perceive consistent colors of objects despite changes in illumination.

Modern Methods:

Auto White Balance (AWB) — most common in cameras; estimates illumination and cancels it (Daylight, Tungsten, Fluorescent presets)

Gray-World Assumption — average of scene = gray; adjust R, G, B gains

White-Patch (Retinex) — brightest pixel in each channel = reference white

Bayesian Color Constancy — probabilistic illuminant estimation

Machine Learning / Deep Learning — modern AI cameras (Google Pixel, iPhone)

Multi-illuminant Detection — local processing for mixed lighting

Color Calibration — reference card (X-Rite ColorChecker)

Modern Camera Pipeline:

Capture → Raw → AWB → Color Correction → Color Space Conversion → Display

### Q10. Vector quantization — training, codebook, matching

Vector Quantization (VQ) maps groups (vectors) of pixels to a codebook of representative vectors, storing only indices.

Pipeline:

Image → Blocks → Find nearest codeword → Index → Bitstream
                                              ↓
                              Index → Codebook lookup → Reconstructed

1. Training Vector Selection:

Collect representative images

Divide into fixed-size blocks (e.g., 4×4 = 16-dim vectors)

Diverse training → better codebook

2. Codebook Generation (LBG Algorithm):

Step 1: Initialize codebook with K codewords (random or splitting)

Step 2: Assign each training vector to nearest codeword (Euclidean)

Step 3: Recompute codewords as centroids

Step 4: Repeat steps 2–3 until distortion converges

Step 5: Final codebook = K representative vectors

3. Matching Process:

Encoding: Divide input into blocks → find nearest codeword → store index

Decoding: Look up codeword for each index → reassemble

Compression Ratio: For 4×4 blocks (128 bits) → 8-bit index = 16:1.

Variants: Tree-structured VQ, mean-removed VQ, classified VQ.

Applications: Image compression, speech compression, pattern recognition.

## ====== PROBLEM SET V — UNIT V (MORPHOLOGICAL IMAGE PROCESSING) ======

### Q1. Two core morphological operations

The two core operations:

1. Dilation (A ⊕ B) — expands binary objects

2. Erosion (A ⊖ B) — shrinks binary objects

All other morphological operations are derived from these:

Opening = Erosion → Dilation

Closing = Dilation → Erosion

Hit-or-Miss = combined erosions

Boundary = original − erosion

Hole filling, skeleton, thinning — iterative dilation/erosion

### Q2. Hit-or-miss transformation + structuring element for top-left corners

Hit-or-Miss Transformation (HMT) detects specific shapes using two disjoint structuring elements.

Formula: A ⊛ B = (A ⊖ B₁) ∩ (A^c ⊖ B₂)

Where:

B₁ = foreground pattern (hit)

B₂ = background pattern (miss)

B₁ ∩ B₂ = ∅ (disjoint)

For Top-Left Corner Detection:

B₁ (foreground hit):

0 0 0
0 1 1
0 1 1

B₂ (background miss):

1 1 1
1 0 0
1 0 0

Center pixel = 1 only where top-left corner is detected.

Applications: Corner detection, isolated point detection, line endpoint detection, thinning, skeletonization.

### Q3. Erosion and dilation — examples + boundary effects

Erosion (A ⊖ B): A ⊖ B = { z | B_z ⊆ A } — shrinks objects.

A:                   A ⊖ B (3×3 SE):
0 0 0 0 0            0 0 0 0 0
0 1 1 1 0            0 0 0 0 0
0 1 1 1 0    →       0 0 1 0 0
0 1 1 1 0            0 0 0 0 0
0 0 0 0 0            0 0 0 0 0

Only center survives. Boundary moves inward.

Dilation (A ⊕ B): A ⊕ B = { z | (B̂)_z ∩ A ≠ ∅ } — grows objects.

A:                   A ⊕ B (3×3 SE):
0 0 0 0 0            0 0 0 0 0
0 0 0 0 0            0 1 1 1 0
0 0 1 0 0    →       0 1 1 1 0
0 0 0 0 0            0 1 1 1 0
0 0 0 0 0            0 0 0 0 0

Single pixel becomes 3×3 block. Boundary moves outward.

| Op | Effect | Boundary | Uses |
| --- | --- | --- | --- |
| Erosion | Shrink | Inward | Remove small objects/noise |
| Dilation | Grow | Outward | Fill small gaps, connect parts |

### Q4. Opening and closing — composed from erosion + dilation; smoothing, noise reduction, shape analysis

Opening: A ∘ B = (A ⊖ B) ⊕ B (Erode first, then dilate)

Geometric meaning: Removes small protrusions, smooths outer contours, preserves overall size.

Effects:

Salt noise removal (small bright specks)

Smoothing object outlines

Removing thin connections

Granulometry (size distribution analysis)

Properties: Anti-extensive (A ∘ B ⊆ A), idempotent.

Closing: A • B = (A ⊕ B) ⊖ B (Dilate first, then erode)

Geometric meaning: Fills small holes, connects nearby objects, smooths concave contours.

Effects:

Pepper noise removal (small dark holes)

Filling gaps

Connecting nearby objects

OCR (close character gaps)

Properties: Extensive (A ⊆ A • B), idempotent.

| Feature | Opening | Closing |
| --- | --- | --- |
| Sequence | Erode → Dilate | Dilate → Erode |
| Removes | Small bright | Small dark holes |
| Effect | Smooth outside | Smooth inside |

### Q5. Erosion, dilation, opening, closing — effects on boundaries, holes, gaps

| Operation | Formula | Boundary | Holes | Gaps |
| --- | --- | --- | --- | --- |
| Erosion (A ⊖ B) | Shrink | Moves inward | Enlarges | Widens |
| Dilation (A ⊕ B) | Grow | Moves outward | Shrinks/fills | Closes |
| Opening (A ∘ B) | (A ⊖ B) ⊕ B | Smooths outside | Slightly affected | Removes thin gaps |
| Closing (A • B) | (A ⊕ B) ⊖ B | Smooths inside | Fills small holes | Closes small gaps |

Detailed Effects:

Erosion: Boundary inward; holes larger; gaps widen; small objects vanish

Dilation: Boundary outward; holes shrink; gaps close

Opening: Outer contour smoothed; small objects removed

Closing: Inner contour smoothed; small holes filled

### Q6. Boundary extraction, hole filling, connected components — algorithms + examples

(1) Boundary Extraction: β(A) = A − (A ⊖ B)

Algorithm:

Erode A by B (3×3 SE)

Subtract eroded from original

Output = boundary pixels

A:                  β(A):
1 1 1 1 1           1 1 1 1 1
1 1 1 1 1           1 0 0 0 1
1 1 1 1 1   →       1 0 0 0 1
1 1 1 1 1           1 0 0 0 1
1 1 1 1 1           1 1 1 1 1

(2) Hole Filling: X_k = (X_{k-1} ⊕ B) ∩ A^c

Algorithm:

- Find seed inside hole; X_0 = {p}

- Dilate X by B

- Intersect with complement of A

- Repeat until X_k = X_{k-1}

- Final filled = A ∪ X_k

(3) Connected Component Extraction: X_k = (X_{k-1} ⊕ B) ∩ A

Algorithm:

- Seed pixel inside component

- Dilate X by B

- Intersect with A (not A^c)

- Repeat until convergence

- X_k = one connected component

Applications: Boundary extraction → contour mapping. Hole filling → OCR, shape analysis. Connected components → region labeling, blob detection.

### Q7. Morphological openings of increasing size → object size distributions (granulometry)

Concept: Apply opening with SE of increasing size; each opening removes objects smaller than the SE. Track area changes.

Algorithm:

Start with binary image A

For k = 1, 2, 3, ...:

Apply opening A_k = A ∘ B_k (B_k of radius k)

Compute area |A_k|

Compute differences d_k = |A_{k-1}| − |A_k|

d_k = total area of objects of size = k

Plot d_k vs k → granulometric curve

Numerical Example:

|A| = 100
|A ∘ B_1| = 95   → 5 pixels (size 0-1)
|A ∘ B_2| = 80   → 15 pixels (size 1-2)
|A ∘ B_3| = 50   → 30 pixels (size 2-3)
|A ∘ B_5| = 0    → all removed

Most objects are size 2-3.

Applications: Metallurgy (grain size), cell counting, soil science (particle size), industrial QC.

### Q8. Algorithm to extract boundaries of all objects in binary image

Algorithm: Boundary Extraction

Formula: β(A) = A − (A ⊖ B)

Step-by-step:

- Take binary image A

- Choose SE B (typically 3×3 square)

- Erode A by B: A ⊖ B = { z | B_z ⊆ A }

- Subtract eroded from original: β(A) = A − (A ⊖ B)

- Output boundary image

Why it works: Erosion shrinks each object by one pixel-layer; subtraction reveals outermost layer → boundary. Works simultaneously for all objects.

Example with multiple objects:

A:                            β(A):
0 0 0 0 0 0 0 0 0 0           0 0 0 0 0 0 0 0 0 0
0 1 1 1 0 0 1 1 1 0           0 1 1 1 0 0 1 1 1 0
0 1 1 1 0 0 1 1 1 0     →     0 1 0 1 0 0 1 0 1 0
0 1 1 1 0 0 1 1 1 0           0 1 1 1 0 0 1 1 1 0
0 0 0 0 0 0 0 0 0 0           0 0 0 0 0 0 0 0 0 0

Boundaries of both objects extracted simultaneously.

Properties: Handles multiple objects without segmentation; linear time; extensible with different SEs.

### Q9. Connected components extraction via morphological operations

Concept: Extract a connected component of A starting from a seed pixel using iterative constrained dilation.

Algorithm:

Choose seed pixel p inside desired component; X_0 = {p}

Iterate: X_k = (X_{k-1} ⊕ B) ∩ A

Repeat until X_k = X_{k-1} (convergence)

X_k = all pixels of connected component

To label all components: Pick unlabeled foreground pixel as seed; repeat for each component; assign unique label.

Comparison with other techniques:

| Method | Approach | Complexity |
| --- | --- | --- |
| Morphological (iterative dilation) | Set theory | O(passes · MN) |
| Two-pass labeling | Scan + union-find | O(MN) |
| Flood fill (BFS/DFS) | Graph traversal | O(MN) |
| Recursive labeling | DFS via recursion | O(MN), stack limits |

Example: Binary image with 3 disconnected regions → run algorithm 3 times with different seeds → label each as different component.

Applications: Object counting, image segmentation for OCR, robot vision, quality inspection.

### Q10. Morphological operations for noise removal (salt, pepper, mixed)

| Noise Type | Best Operation | Why |
| --- | --- | --- |
| Salt (bright specks) | Opening (A ∘ B) | Removes small bright objects |
| Pepper (dark specks) | Closing (A • B) | Fills small holes |
| Mixed (S+P) | Open-Close or Close-Open | Removes both |

Operation Effects:

Opening (A ∘ B): Salt removal — erode small bright pixels, dilate restores

Closing (A • B): Pepper removal — dilate fills holes, erode restores

Open-Close: First remove salt, then fill pepper (salt-dominated)

Close-Open: First fill pepper, then remove salt (pepper-dominated)

Effectiveness Table:

| Noise | Opening | Closing | Open-Close | Close-Open |
| --- | --- | --- | --- | --- |
| Salt only | ✅ Excellent | ❌ Poor | ✅ Good | ❌ Poor |
| Pepper only | ❌ Poor | ✅ Excellent | ❌ Poor | ✅ Good |
| Mixed | ⚠ Partial | ⚠ Partial | ✅ Good | ✅ Good |

Order matters: Opening and closing are non-commutative — Open-Close differs from Close-Open slightly.

Conclusion:

Salt only → Opening

Pepper only → Closing

Mixed → Open-Close OR Close-Open

# END OF PROBLEM SET SOLUTIONS