IMAGE PROCESSING — SYLLABUS NOTES

Course: BCA / B.Sc(IT) | Subject Code: UCS6105 | Sem VI

Final Semester VI Examination

Shri Ramswaroop Memorial University

Date: 15 May 2026

# IMAGE PROCESSING — COMPLETE SYLLABUS NOTES — UNITS 1 TO 5

Course: BCA / B.Sc(IT) | Subject Code: UCS6105 | Sem VI

# 🟦 UNIT – I: INTRODUCTION & DIGITAL IMAGE FUNDAMENTALS

## 📌 1.1 Digital Image Processing — Definition

├── Definition: Digital Image Processing (DIP) is the manipulation of a digital image f(x, y) using computer algorithms for enhancement, restoration, analysis, compression, or interpretation.

├── Key Points:

Image = 2-D function f(x, y) where x, y = spatial coordinates and f = intensity (gray level)

Three abstraction levels: low-level (preprocessing), mid-level (segmentation), high-level (interpretation)

Two processing domains: Spatial (pixel-based) and Frequency (transform-based)

Requires sampling (spatial discretization) + quantization (amplitude discretization)

Applied across medicine, remote sensing, robotics, forensics, entertainment

├── Important Terms:

Pixel: Smallest addressable picture element

f(x, y): 2-D image function — intensity at (x, y)

Gray level: Numerical value of pixel intensity

├── One-line Exam Answer: Digital Image Processing is the algorithmic manipulation of a digital image f(x, y) for enhancement, restoration, analysis, or compression.

## 📌 1.2 Fields Using Digital Image Processing

├── Definition: DIP is applied wherever images are captured, analyzed, transmitted, or interpreted in digital form.

├── Key Points:

Medical imaging: X-ray, CT, MRI, ultrasound, microscopy, mammography

Remote sensing: LANDSAT, weather satellites, Mars rovers, drone imagery

Industrial automation: PCB inspection, quality control, defect detection

Forensics & security: Fingerprint, face recognition, ANPR, CCTV

Astronomy: Hubble, James Webb telescope image processing

Robotics: Autonomous vehicles, drones, computer vision

Document processing: OCR, signature verification

Entertainment: CGI in movies, gaming, photo editing

├── One-line Exam Answer: DIP is used in medical, remote sensing, industrial, forensic, astronomical, robotic, OCR, and entertainment systems.

## 📌 1.3 Fundamental Steps in Digital Image Processing

├── Definition: The sequential pipeline of operations applied to a digital image from raw acquisition to final interpretation.

├── Key Points (10 steps + Knowledge Base):

Image Acquisition — capture with sensor (camera, scanner)

Image Enhancement — improve subjective appearance

Image Restoration — recover from known degradation

Color Image Processing — operate on color channels

Wavelets & Multi-resolution — multi-scale representation

Image Compression — reduce data size

Morphological Processing — analyze shapes

Image Segmentation — partition into regions

Representation & Description — extract features

Object Recognition — assign labels / classify

├── Diagram:

Acquisition → Enhancement → Restoration → Color → Wavelets →
Compression → Morphology → Segmentation → Representation → Recognition
                              ↑
                       [ Knowledge Base ]

├── One-line Exam Answer: The fundamental steps are Acquisition → Enhancement → Restoration → Color → Wavelets → Compression → Morphology → Segmentation → Representation → Recognition, supported by a knowledge base.

## 📌 1.4 Components of an Image Processing System

├── Definition: Hardware + software stack required to capture, process, store, display and transmit digital images.

├── Key Points:

| Component | Function |
| --- | --- |
| Image Sensors | Convert light → electrical signal (CCD, CMOS) |
| Specialized Hardware | Frame grabber, DSP, GPU for fast ops |
| Computer | General-purpose PC running DIP software |
| Image Processing Software | MATLAB, OpenCV, ImageJ, Photoshop |
| Mass Storage | RAM (short-term), HDD/SSD (online), tape/cloud (archival) |
| Image Display | Monitor, LCD/LED, projector |
| Hardcopy Device | Printer, plotter |
| Network | Transmit images between systems |

├── Diagram:

[Sensor] → [Digitizer] → [Specialized HW] → [Computer + Software]
                                                    ↓
[Storage] ←→ [Display] ←→ [Hardcopy] ←→ [Network]

├── One-line Exam Answer: A DIP system has sensors, digitizers, specialized hardware, a computer with software, mass storage, display, hardcopy device, and a network.

## 📌 1.5 Image Formation Model (Illumination × Reflectance)

├── Definition: The intensity of any pixel f(x, y) is the product of illumination i(x, y) falling on the scene and reflectance r(x, y) of the object surface.

├── Key Points:

Formula: f(x, y) = i(x, y) × r(x, y)

i(x, y) = illumination (0 < i < ∞) — depends on light source

r(x, y) = reflectance (0 ≤ r ≤ 1) — depends on surface

r = 0: total absorption (black); r = 1: total reflection (white/snow)

Range constraint: L_min ≤ f(x, y) ≤ L_max forms the gray scale

├── Sample Reflectance Values:

| Surface | r |
| --- | --- |
| Black velvet | 0.01 |
| Stainless steel | 0.65 |
| White wall | 0.80 |
| Snow | 0.93 |

├── One-line Exam Answer: The image formation model is f(x, y) = i(x, y) · r(x, y), where i is illumination and r is reflectance (0 ≤ r ≤ 1).

## 📌 1.6 Image Sampling

├── Definition: Sampling is the discretization of spatial coordinates (x, y) — converting a continuous image into a grid of pixels.

├── Key Points:

Determines spatial resolution of the image

Higher sampling rate → more pixels → finer detail

Nyquist criterion: sample rate ≥ 2× highest frequency to avoid aliasing

Low sampling → aliasing (moiré, jagged edges) and pixelation

Produces an M × N matrix of pixel positions

├── One-line Exam Answer: Sampling discretizes the spatial coordinates of an image and determines its spatial resolution; under-sampling causes aliasing.

## 📌 1.7 Image Quantization

├── Definition: Quantization is the discretization of intensity (amplitude) values — mapping continuous brightness into L discrete gray levels.

├── Key Points:

For k-bit image: L = 2^k gray levels

Common values: k=1 (binary), k=8 (256 levels), k=24 (color)

Storage = M × N × k bits

Too few levels → false contouring (visible ridges in smooth regions)

Determines intensity (gray-level) resolution

├── Sampling vs Quantization:

| Aspect | Sampling | Quantization |
| --- | --- | --- |
| Acts on | (x, y) coordinates | Amplitude f |
| Affects | Spatial resolution | Intensity resolution |
| Parameter | M × N | L = 2^k |
| Low-value problem | Aliasing | False contouring |

├── One-line Exam Answer: Quantization discretizes intensity into L = 2^k discrete gray levels; too few levels produces false contouring.

## 📌 1.8 False Contouring

├── Definition: False contouring is a visible-ridge artifact in smooth-gradient regions of an image caused by insufficient quantization levels.

├── Key Points:

Occurs when smooth intensity changes are forced into few discrete steps

Most visible in uniform / smooth regions (skin, sky)

Human eye is highly sensitive to artificial ridges

Typically appears below 6 bits (64 levels)

Solutions: increase quantization levels, apply dithering or error diffusion

├── One-line Exam Answer: False contouring is the appearance of visible ridges in smooth gradient regions caused by insufficient quantization levels; reduced by more bits or dithering.

## 📌 1.9 Pixel Neighborhoods (4-, Diagonal, 8-)

├── Definition: Geometric relationships between a pixel p(x, y) and its surrounding pixels — basis for all neighborhood operations.

├── Key Points:

N₄(p): 4 horizontal/vertical neighbors

N_D(p): 4 diagonal neighbors

N₈(p): all 8 surrounding pixels = N₄ ∪ N_D

Border pixels have fewer neighbors — special handling required

Used in connectivity, path finding, distance computation, segmentation

├── Diagram:

N4 (+ shape)        ND (X shape)         N8 (full square)
       [N]            [NW]   [NE]          [NW] [N] [NE]
   [W] [p] [E]            [p]               [W] [p] [E]
       [S]            [SW]   [SE]          [SW] [S] [SE]

├── One-line Exam Answer: A pixel's neighbors are N₄ (4 horizontal/vertical), N_D (4 diagonal), and N₈ (all 8 surrounding pixels).

## 📌 1.10 Pixel Adjacency (4-, 8-, m-connectivity)

├── Definition: Two pixels are connected if they are spatial neighbors AND their gray-level values belong to a similarity set V.

├── Key Points:

4-adjacency: q ∈ N₄(p) AND values in V

8-adjacency: q ∈ N₈(p) AND values in V

m-adjacency (mixed): q ∈ N₄(p) OR (q ∈ N_D(p) AND N₄(p) ∩ N₄(q) ∩ V = ∅)

8-adjacency can create multiple paths — ambiguous

m-adjacency was introduced to eliminate this path ambiguity

├── Comparison:

| Type | Diagonal allowed | Ambiguity |
| --- | --- | --- |
| 4-adjacency | No | None |
| 8-adjacency | Yes | Possible |
| m-adjacency | Conditional | Resolved |

├── One-line Exam Answer: Pixels are 4-, 8-, or m-adjacent based on neighborhood and similarity; m-adjacency resolves the path ambiguity of 8-adjacency.

## 📌 1.11 Distance Measures Between Pixels

├── Definition: Mathematical methods to measure how far apart two pixels p(x, y) and q(s, t) are in a digital image grid.

├── Key Points & Formulas:

Euclidean (D_e): D_e = √((x − s)² + (y − t)²) — true geometric distance

City-block (D₄): D₄ = |x − s| + |y − t| — Manhattan distance

Chessboard (D₈): D₈ = max(|x − s|, |y − t|) — king's move

All satisfy: positivity, symmetry, triangle inequality

├── Worked Example for p(2, 3), q(5, 7):

D_e = √((2-5)² + (3-7)²) = √(9 + 16) = √25 = 5
D₄  = |2-5| + |3-7|     = 3 + 4 = 7
D₈  = max(|2-5|, |3-7|) = max(3, 4) = 4

├── One-line Exam Answer: Distance between two pixels can be Euclidean, City-block (D₄), or Chessboard (D₈) depending on application.

# 🟩 UNIT – II: IMAGE ENHANCEMENT IN THE SPATIAL DOMAIN

## 📌 2.1 Spatial Domain

├── Definition: The spatial domain is the image plane itself; enhancement is performed by direct manipulation of pixels rather than through transforms.

├── Key Points:

General formulation: g(x, y) = T[f(x, y)]

T can be point operation (one pixel) or mask operation (neighborhood)

No Fourier transform involved

Faster than frequency domain for small kernels

Examples: histogram equalization, convolution, gamma correction

├── Spatial vs Frequency Domain:

| Feature | Spatial | Frequency |
| --- | --- | --- |
| Operates on | Pixels directly | Fourier coefficients |
| Transform | Not needed | DFT / DCT required |
| Examples | Mean filter, Sobel | Ideal LPF, Butterworth |
| Speed | Fast (small kernels) | Fast (FFT) |

├── One-line Exam Answer: The spatial domain is the image plane where enhancement is performed by direct pixel manipulation via g(x, y) = T[f(x, y)].

## 📌 2.2 Basic Gray-Level Transformations

├── Definition: Point operations s = T(r) mapping each input intensity r to an output intensity s using a global mathematical function.

├── Key Points & Formulas:

| Transform | Formula | Purpose |
| --- | --- | --- |
| Identity | s = r | No change |
| Negative | s = L − 1 − r | Invert gray levels (X-ray) |
| Log | s = c · log(1 + r) | Compress dynamic range |
| Inverse log | s = c · (e^r − 1) | Expand compressed values |
| Power-law / Gamma | s = c · r^γ | Gamma correction |
| Threshold | s = 0 if r<T else L−1 | Binary image |

├── One-line Exam Answer: Basic gray-level transformations s = T(r) include identity, negative, log, power-law (gamma), and threshold mappings.

## 📌 2.3 Image Negative

├── Definition: Negative transformation reverses gray levels — dark pixels become light and vice versa — using s = L − 1 − r.

├── Key Points:

For 8-bit images: s = 255 − r

Enhances white/gray detail in dark backgrounds

Used heavily in mammograms, X-rays, MRI

Reversible — applying twice gives the original

Pure point operation, O(MN) complexity

├── Lookup Example:

| r (input) | s = 255 − r |
| --- | --- |
| 0 | 255 |
| 100 | 155 |
| 200 | 55 |
| 255 | 0 |

├── One-line Exam Answer: Negative transformation s = L − 1 − r inverts gray levels and is used to enhance white/gray details in dark backgrounds such as X-rays.

## 📌 2.4 Log Transformation

├── Definition: Compresses the dynamic range of an image by mapping a wide range of input values into a narrower output range via the logarithm.

├── Key Points:

Formula: s = c · log(1 + r)

Expands dark pixels and compresses bright pixels

c chosen so max output = 255 (8-bit)

Used for displaying Fourier magnitude spectra with very wide range

Mimics human eye's logarithmic intensity perception

├── One-line Exam Answer: Log transformation s = c · log(1 + r) compresses dynamic range, expanding dark pixels and compressing bright ones — used for displaying Fourier spectra.

## 📌 2.5 Power-law (Gamma) Transformation

├── Definition: s = c · r^γ provides flexible contrast control by changing γ — used for gamma correction in displays.

├── Key Points:

γ < 1 brightens dark image (boost shadows)

γ > 1 darkens bright image (compress shadows)

γ = 1 is the identity transformation

CRT monitors have natural γ ≈ 2.2, so capture pre-applies γ ≈ 1/2.2

Foundation of gamma correction in cameras, TVs, monitors

├── Use Cases:

| Gamma | Effect | Use |
| --- | --- | --- |
| < 1 | Brighten | MRI dark images |
| > 1 | Darken | Reduce glare |
| = 2.2 | CRT correction | Display |

├── One-line Exam Answer: Power-law transformation s = c · r^γ is used for gamma correction; γ<1 brightens and γ>1 darkens an image.

## 📌 2.6 Histogram of an Image

├── Definition: A histogram plots the count of pixels n_k against each gray level r_k: h(r_k) = n_k.

├── Key Points:

Normalized histogram: p(r_k) = n_k / MN (probability)

Σ p(r_k) = 1 over all k = 0..L−1

Dark image → concentrated on left

Bright image → concentrated on right

Low-contrast → narrow peak; high-contrast → spread across full range

Bimodal histogram suggests object + background (used for thresholding)

├── Interpretation:

| Histogram Pattern | Image Quality |
| --- | --- |
| Concentrated left | Too dark |
| Concentrated right | Too bright |
| Narrow peak | Low contrast |
| Spread across | Good contrast |
| Bimodal | Object + background |

├── One-line Exam Answer: A histogram plots pixel count h(r_k) = n_k against gray level and reveals the brightness and contrast distribution of an image.

## 📌 2.7 Histogram Equalization (HE)

├── Definition: Histogram Equalization is an automatic contrast-enhancement method that maps pixel intensities through their cumulative distribution function (CDF) to produce an approximately uniform output histogram.

├── Key Points (Algorithm):

Compute histogram h(r_k) = n_k

Normalize: p_r(r_k) = n_k / MN

Compute CDF: cdf(r_k) = Σ p_r(r_j) for j = 0..k

Compute mapping: s_k = round((L − 1) × cdf(r_k))

Replace each pixel: r_k → s_k

├── Formula: s_k = T(r_k) = (L − 1) · Σ_{j=0..k} p_r(r_j)

├── Worked Example (3-bit, 64 pixels):

| r_k | n_k | p_r | cdf | (L-1)·cdf | s_k |
| --- | --- | --- | --- | --- | --- |
| 0 | 8 | 0.125 | 0.125 | 0.875 | 1 |
| 1 | 10 | 0.156 | 0.281 | 1.97 | 2 |
| 2 | 10 | 0.156 | 0.437 | 3.06 | 3 |
| 3 | 2 | 0.031 | 0.469 | 3.28 | 3 |
| 4 | 12 | 0.187 | 0.656 | 4.59 | 5 |
| 5 | 16 | 0.250 | 0.906 | 6.34 | 6 |
| 6 | 4 | 0.063 | 0.969 | 6.78 | 7 |
| 7 | 2 | 0.031 | 1.000 | 7.00 | 7 |

├── One-line Exam Answer: Histogram equalization maps pixels via s_k = (L − 1) · cdf(r_k) to obtain an approximately uniform histogram and maximize global contrast.

## 📌 2.8 Histogram Matching (Specification)

├── Definition: Histogram matching transforms an image so its output histogram has a user-specified shape — useful when uniform distribution (HE) is not desired.

├── Key Points (Algorithm):

Compute CDF of input image: s = T(r)

Compute CDF of desired histogram: v = G(z)

For each s, find z = G⁻¹(s) by inverse mapping

Map each input pixel: r → s → z

Output has the desired histogram distribution

├── HE vs Histogram Matching:

| Feature | Equalization | Matching |
| --- | --- | --- |
| Output | Uniform | User-specified |
| Flexibility | Low (automatic) | High (designer-defined) |
| Steps | 1 CDF mapping | 2 CDFs + inverse |
| Use | General enhancement | Match reference image |

├── One-line Exam Answer: Histogram matching produces an image with a user-specified histogram via two CDF mappings and an inverse transformation G⁻¹.

## 📌 2.9 Arithmetic Operations on Images

├── Definition: Pixel-wise mathematical operations between two images (or an image and a scalar) — addition, subtraction, multiplication, division.

├── Key Points & Applications:

| Operation | Formula | Application |
| --- | --- | --- |
| Addition | s = f + g | Noise reduction (averaging), double exposure |
| Subtraction | s = f − g | Motion detection, DSA (medical), background removal |
| Multiplication | s = f · g | Masking (ROI extraction), shading correction |
| Division | s = f / g | Flat-field correction, illumination normalization |

├── Image Averaging: ḡ(x, y) = (1/K) · Σ g_i(x, y) — noise variance reduces by 1/K

├── One-line Exam Answer: Arithmetic operations between images are used for noise reduction (add/average), motion detection (subtract), masking (multiply), and shading correction (divide).

## 📌 2.10 Logical Operations on Binary Images

├── Definition: Bitwise Boolean operations (AND, OR, NOT, XOR, NAND, NOR) on binary images for masking, region union, and change detection.

├── Truth Table:

| p | q | AND | OR | XOR | NAND | NOR |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 | 1 | 0 |
| 1 | 0 | 0 | 1 | 1 | 1 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 |

├── Applications:

AND — masking, ROI intersection

OR — region union, combining features

NOT — complement, invert mask

XOR — difference / change detection

├── One-line Exam Answer: Logical operations (AND, OR, NOT, XOR) on binary images are used for masking, region union/intersection, complement, and difference detection.

## 📌 2.11 Spatial Filtering (Convolution)

├── Definition: A neighborhood operation where a mask (kernel) is moved across the image and the output pixel is computed as a weighted sum of nearby pixels.

├── Key Points:

Formula: g(x, y) = Σ Σ w(s, t) · f(x + s, y + t)

Two categories: smoothing (low-pass) and sharpening (high-pass)

Mask size typically 3×3, 5×5, 7×7 (odd, symmetric)

Boundary handling — zero padding, replicate, mirror

Procedure: position mask → multiply → sum → place → slide

├── One-line Exam Answer: Spatial filtering convolves a mask w with image f to produce g(x, y) = ΣΣ w(s, t)·f(x+s, y+t) — used for smoothing or sharpening.

## 📌 2.12 Smoothing (Low-Pass) Filters

├── Definition: Smoothing filters blur the image, reduce noise, and remove small details by averaging neighborhood pixels.

├── Key Points:

Mean (Box) filter — simple arithmetic average (linear)

Gaussian filter — weighted average (linear, less blur)

Median filter — non-linear, sorts and picks middle (excellent for salt-and-pepper)

Larger kernel → more smoothing but more edge blur

Used as preprocessing for segmentation / edge detection

├── Common Masks:

3×3 Mean (1/9):           3×3 Gaussian (1/16):
  1 1 1                      1 2 1
  1 1 1                      2 4 2
  1 1 1                      1 2 1

├── Smoothing Comparison:

| Filter | Type | Best for | Edge Preservation |
| --- | --- | --- | --- |
| Mean (Box) | Linear | Gaussian noise | Poor |
| Gaussian | Linear | Gaussian noise | Better |
| Median | Non-linear | Salt-and-pepper | Excellent |

├── One-line Exam Answer: Smoothing filters (mean, Gaussian, median) blur the image and reduce noise; median preserves edges best.

## 📌 2.13 Sharpening (High-Pass) Filters

├── Definition: Sharpening filters enhance edges and fine details by emphasizing intensity discontinuities using derivatives.

├── Key Points:

First derivative: Sobel, Prewitt (gradient operators)

Second derivative: Laplacian (isotropic)

Gradient magnitude: |∇f| = √(G_x² + G_y²)

Gradient direction: θ = arctan(G_y / G_x)

Sharpened image = original + edge response

├── Sobel Operators (3×3):

G_x = [-1  0  1]      G_y = [-1 -2 -1]
      [-2  0  2]            [ 0  0  0]
      [-1  0  1]            [ 1  2  1]

├── Laplacian Mask:

∇²f = f(x+1,y) + f(x-1,y) + f(x,y+1) + f(x,y-1) - 4·f(x,y)

      [ 0 -1  0]
Mask: [-1  4 -1]
      [ 0 -1  0]

├── One-line Exam Answer: Sharpening filters use derivatives — Sobel/Prewitt (first-order gradient) detect edges and Laplacian (second-order) enhances all-direction discontinuities.

## 📌 2.14 Combining Spatial Enhancement Methods

├── Definition: Real enhancement often chains multiple techniques sequentially to handle complex image-quality problems.

├── Key Points (Skeletal Image Pipeline):

Apply Laplacian to sharpen edges

Apply Sobel gradient to produce edge mask

Smooth the Sobel image (5×5 box) for cleaner mask

Multiply Laplacian result × smoothed mask

Add the result to original

Apply power-law (gamma) transform for dynamic range

├── One-line Exam Answer: Combining spatial methods (Laplacian + Sobel + smoothing + gamma) chains operations to enhance edges, reduce noise, and stretch dynamic range.

# 🟨 UNIT – III: IMAGE RESTORATION

## 📌 3.1 Image Restoration vs Image Enhancement

├── Definition: Image restoration is the objective recovery of an image from a degraded version using a mathematical model of the degradation, whereas enhancement is the subjective improvement of appearance.

├── Key Points:

| Feature | Image Enhancement | Image Restoration |
| --- | --- | --- |
| Goal | Subjective appearance | Recover original from degradation |
| Approach | Heuristic / visual | Mathematical / objective |
| Knowledge needed | Visual preference | Degradation function H, noise η |
| Techniques | Histogram, sharpen, gamma | Inverse filter, Wiener, CLS |
| Output | "Looks better" | "Closer to original" |

├── Examples:

Enhancement: brightening a dark photo using gamma

Restoration: deblurring a motion-blurred satellite image with Wiener filter

├── One-line Exam Answer: Enhancement subjectively improves image appearance while restoration mathematically reverses a known degradation to recover the original.

## 📌 3.2 Image Degradation / Restoration Model

├── Definition: A model that expresses the degraded image g(x, y) as the convolution of the original image f(x, y) with a degradation function h(x, y) plus additive noise η(x, y).

├── Key Points & Formulas:

Spatial domain: g(x, y) = h(x, y) * f(x, y) + η(x, y)

Frequency domain: G(u, v) = H(u, v) · F(u, v) + N(u, v)

f(x, y) = original image (unknown, to recover)

h(x, y) = degradation function (PSF)

η(x, y) = additive noise

Restoration goal: estimate f̂(x, y) ≈ f(x, y)

├── Block Diagram:

η(x, y) noise
                              ↓
  f(x,y) → [ H(degrad.) ] → ⊕ → g(x,y) → [Restoration Filter] → f̂(x,y)

├── One-line Exam Answer: The degradation model is g(x, y) = h(x, y) * f(x, y) + η(x, y) — degraded image equals original convolved with degradation function plus noise.

## 📌 3.3 Noise Models

├── Definition: Mathematical descriptions of probability density functions (PDFs) governing noise that contaminates digital images.

├── Key Points & PDFs:

Gaussian: p(z) = (1/σ√2π) · exp(−(z − μ)² / 2σ²) → sensor / electronic noise

Rayleigh: p(z) = (2/b)(z − a) · exp(−(z − a)²/b) → range imaging

Erlang (Gamma): p(z) = (a^b · z^(b−1) / (b−1)!) · exp(−az) → laser imaging

Exponential: p(z) = a · exp(−az) → laser imaging

Uniform: p(z) = 1/(b − a) for a ≤ z ≤ b → simulation

Impulse (Salt-and-Pepper): p(z) = P_a (z=a), P_b (z=b)

├── Noise → Best Filter Mapping:

| Noise | Source | Best Filter |
| --- | --- | --- |
| Gaussian | Sensor / electronic | Mean / Gaussian |
| Salt-and-pepper | Transmission errors | Median |
| Salt only | High-value spikes | Min |
| Pepper only | Low-value spikes | Max |
| Rayleigh | Range imaging | Geometric mean |

├── One-line Exam Answer: Common noise PDFs are Gaussian, Rayleigh, Erlang, Exponential, Uniform, and impulse (Salt-and-Pepper) — each requires a specific filter.

## 📌 3.4 Mean Filters (Arithmetic, Geometric, Harmonic, Contra-harmonic)

├── Definition: Linear and non-linear filters that average neighborhood pixels to reduce noise.

├── Key Points & Formulas:

Arithmetic Mean: f̂ = (1/mn) · Σ g(s, t)   → Gaussian noise; blurs edges

Geometric Mean: f̂ = [Π g(s, t)]^(1/mn)   → Gaussian; less blur; fails on zero pixels

Harmonic Mean: f̂ = mn / Σ (1 / g(s, t))   → salt + Gaussian; fails on pepper

Contra-harmonic Mean: f̂ = Σ g^(Q+1) / Σ g^Q   → Q>0 removes pepper; Q<0 removes salt

├── Mean Filter Summary:

| Filter | Best for |
| --- | --- |
| Arithmetic | Gaussian / uniform noise |
| Geometric | Gaussian noise (better detail) |
| Harmonic | Salt + Gaussian noise |
| Contra-harmonic Q>0 | Pepper noise |
| Contra-harmonic Q<0 | Salt noise |

├── One-line Exam Answer: Mean filters include arithmetic, geometric, harmonic, and contra-harmonic — each suited to different noise types.

## 📌 3.5 Order-Statistic Filters (Median, Max, Min, Midpoint)

├── Definition: Non-linear filters whose output depends on ranking (sorting) the values in the neighborhood.

├── Key Points & Formulas:

Median: f̂ = median { g(s, t) } → best for salt-and-pepper

Max: f̂ = max { g(s, t) } → removes pepper; brightens

Min: f̂ = min { g(s, t) } → removes salt; darkens

Midpoint: f̂ = ½(max + min) → Gaussian, uniform

Alpha-trimmed mean — trim extremes then average; mixed noise

├── Order-Statistic Filter Summary:

| Filter | Removes | Side Effect |
| --- | --- | --- |
| Median | Salt-and-pepper | Slight smoothing |
| Max | Pepper | Brightens |
| Min | Salt | Darkens |
| Midpoint | Gaussian / uniform | Mild blur |

├── One-line Exam Answer: Order-statistic filters rank neighborhood pixels — median removes salt-and-pepper, max removes pepper, min removes salt.

## 📌 3.6 Motion Blur Degradation

├── Definition: A degradation caused by relative motion between the camera and the scene during exposure — modeled as a convolution with a linear PSF.

├── Key Points & Formulas:

Spatial PSF (1D motion length L): h(x, y) = 1/L for 0 ≤ x ≤ L, y = 0; else 0

Frequency domain: H(u, v) = (T / π(ua + vb)) · sin(π(ua + vb)) · exp(−jπ(ua + vb))

Parameters: length L, angle θ, exposure T

Causes: camera shake, object motion, long exposure, vibration

Restoration: inverse filter (no noise) or Wiener filter (with noise)

├── One-line Exam Answer: Motion blur is caused by relative motion during exposure and is modeled as a convolution with a linear PSF of length L and angle θ.

## 📌 3.7 Prior Knowledge in Restoration

├── Definition: Information about the degradation function, noise, scene, or imaging system known in advance — used to choose and parameterize restoration filters.

├── Key Points:

Without prior knowledge → blind deconvolution (hard, iterative)

With knowledge of H → enables inverse / Wiener filtering

With knowledge of noise PDF → enables noise-specific filters

Sources: optical system, atmosphere model, motion sensors, calibration

More knowledge → better restoration accuracy

├── One-line Exam Answer: Prior knowledge of the degradation function and noise statistics enables effective restoration via inverse, Wiener, or constrained least-squares filtering.

## 📌 3.8 Inverse Filter vs Wiener Filter

├── Definition: Inverse filter directly divides spectra (F̂ = G/H); Wiener filter is the optimum mean-square-error solution that balances inversion against noise suppression.

├── Key Formulas:

Inverse filter: F̂(u, v) = G(u, v) / H(u, v)

Wiener filter: F̂(u, v) = [H*(u, v) / (|H(u, v)|² + S_η/S_f)] · G(u, v)

├── Inverse vs Wiener Comparison:

| Feature | Inverse | Wiener |
| --- | --- | --- |
| Knowledge needed | H | H + S_η + S_f |
| Noise handling | Amplifies | Optimally suppresses |
| Best when | Noise = 0 | Noise present |
| Failure mode | Blows up at H ≈ 0 | Smoothly handles all freq |

├── Note: When noise → 0, Wiener filter reduces to the inverse / pseudo-inverse filter.

├── One-line Exam Answer: Inverse filter (F̂ = G/H) amplifies noise; Wiener filter is optimal in MSE sense and balances inversion against noise suppression.

# 🟧 UNIT – IV: COLOR IMAGE PROCESSING & IMAGE COMPRESSION

## 📌 4.1 Color Fundamentals

├── Definition: Color is the human perception of light wavelengths in the visible spectrum (400–700 nm), characterized by brightness, hue, and saturation.

├── Key Points:

Visible spectrum: 400 nm (violet) – 700 nm (red)

Additive primaries = R, G, B (used for light: monitors, sensors)

Subtractive primaries = C, M, Y (used for pigments: printing)

Three attributes: brightness (intensity), hue (dominant wavelength), saturation (purity)

Tri-stimulus theory: retina has S, M, L cones

├── Color Primaries:

| Type | Primaries | Use |
| --- | --- | --- |
| Additive | R G B | Monitors, lights |
| Subtractive | C M Y(K) | Printing, paint |

├── One-line Exam Answer: Color has three attributes — brightness, hue, saturation; additive primaries are RGB (light), subtractive are CMY (pigment).

## 📌 4.2 RGB Color Model

├── Definition: A 3-D color model that represents colors as additive combinations of Red, Green, and Blue primaries, visualized as a unit cube.

├── Key Points:

Used in monitors, cameras, image sensors

Each axis 0–1 (or 0–255 in 8-bit)

Black at (0, 0, 0); White at (1, 1, 1)

Main diagonal = gray line

24-bit RGB → 256³ = 16.7 million colors

├── Cube Vertices:

| Color | R | G | B |
| --- | --- | --- | --- |
| Black | 0 | 0 | 0 |
| Red | 1 | 0 | 0 |
| Green | 0 | 1 | 0 |
| Blue | 0 | 0 | 1 |
| Yellow | 1 | 1 | 0 |
| Cyan | 0 | 1 | 1 |
| Magenta | 1 | 0 | 1 |
| White | 1 | 1 | 1 |

├── Diagram:

White (1,1,1)
       /|
   Yellow ── Cyan
    /|     /|
 Red ── Magenta ── Blue
   \  |   /
   Black (0,0,0)

├── One-line Exam Answer: The RGB color model represents colors as additive combinations of Red, Green, and Blue primaries, visualized as a unit cube.

## 📌 4.3 HSI Color Model

├── Definition: A perceptual color model that decouples color into Hue (dominant wavelength), Saturation (purity), and Intensity (brightness) — closer to human vision.

├── Key Points:

H — angle 0°–360° around color circle

S — 0 (gray) to 1 (pure color)

I — 0 (black) to 1 (white)

Decouples color from brightness → ideal for image processing

Used in segmentation, object recognition, color manipulation

├── RGB → HSI Conversion Formulas:

I = (R + G + B) / 3
S = 1 − [3 · min(R, G, B) / (R + G + B)]
θ = arccos { [½((R−G) + (R−B))] / √((R−G)² + (R−B)(G−B)) }
H = θ        if B ≤ G
H = 360 − θ  if B > G

├── RGB vs HSI:

| Feature | RGB | HSI |
| --- | --- | --- |
| Basis | Light primaries | Human perception |
| Intensity decoupled | No | Yes |
| Hardware | Display, sensor | Processing, editing |
| Geometry | Cube | Cone / cylinder |

├── One-line Exam Answer: HSI represents color in terms of Hue (dominant wavelength), Saturation (purity), and Intensity (brightness) — useful for processing as it matches human perception.

## 📌 4.4 Pseudocoloring

├── Definition: Pseudocoloring assigns colors to gray-level values of a monochrome image to make features more visible — humans distinguish thousands of colors but only ~24 gray shades.

├── Key Points:

Intensity slicing — divide gray range into intervals, assign colors

Gray-to-color transforms — three independent f_R, f_G, f_B functions

Applications: medical (CT/MRI), thermal imaging, astronomy, weather radar

Not "true color" — purely a visualization aid

├── One-line Exam Answer: Pseudocoloring maps gray levels to colors via intensity slicing or independent R/G/B transformations to enhance visualization.

## 📌 4.5 Color Quantization

├── Definition: Color quantization reduces the number of distinct colors in an image (e.g., from 16.7M to 256) to fit limited displays or file formats.

├── Key Points:

Required for GIF (256-color max), older 8-bit displays, embedded systems

Algorithms: uniform quantization, median-cut, octree, K-means clustering

Trade-off: smaller palette → smaller file but color banding

Used in image compression (vector quantization)

├── Algorithm Comparison:

| Algorithm | Approach | Quality |
| --- | --- | --- |
| Uniform | Equal subdivision | Low |
| Median-cut | Recursive split at median | Good |
| Octree | Tree-based subdivision | Good |
| K-means | Clustering | Best (slower) |

├── One-line Exam Answer: Color quantization reduces an image's color palette using algorithms like median-cut or K-means — used in GIF and limited-display systems.

## 📌 4.6 Color Constancy

├── Definition: Color constancy is the ability of a vision system to perceive consistent colors of objects despite changes in illumination.

├── Key Points:

Without constancy, white paper looks yellow under tungsten light

Implemented in cameras as auto white-balance (AWB)

Algorithms: gray-world, white-patch (Retinex), Bayesian, ML

Critical for object recognition and machine vision

Modern AI cameras learn illumination from training data

├── One-line Exam Answer: Color constancy ensures consistent color perception under different lighting and is implemented via white balance, gray-world assumption, Retinex, or ML methods.

## 📌 4.7 Image Compression Fundamentals

├── Definition: Image compression reduces the data size needed to represent an image by exploiting redundancies in the data.

├── Key Points:

Four redundancy types: coding, spatial (interpixel), temporal, psycho-visual

Compression Ratio: CR = original size / compressed size

Two categories: lossless (exact recovery) and lossy (approximate)

Required for storage, transmission, streaming, mobile devices

Lossless CR ~ 2:1 – 5:1; lossy CR ~ 10:1 – 100:1

├── Redundancy Types:

| Type | Cause | Removed By |
| --- | --- | --- |
| Coding | Equal-length codes | Huffman, Arithmetic |
| Spatial | Pixel correlation | RLE, predictive |
| Temporal | Frame similarity | Motion compensation |
| Psycho-visual | Imperceptible info | Quantization |

├── One-line Exam Answer: Image compression reduces data size by removing coding, spatial, temporal, and psycho-visual redundancies.

## 📌 4.8 Lossless vs Lossy Compression

├── Definition: Lossless compression reconstructs the original exactly; lossy compression sacrifices some quality for higher compression.

├── Key Comparison:

| Feature | Lossless | Lossy |
| --- | --- | --- |
| Recovery | Bit-exact | Approximate |
| Compression ratio | 2:1 – 5:1 | 10:1 – 100:1 |
| Examples | PNG, GIF, TIFF | JPEG, MPEG, WebP |
| Techniques | Huffman, RLE, LZW, Arithmetic | DCT, wavelet, VQ |
| Use cases | Medical, legal, archival | Web, video, streaming |

├── Quality Metrics:

MSE = (1/MN) · Σ(f − f̂)²

PSNR = 10 · log₁₀(255² / MSE) dB

├── One-line Exam Answer: Lossless compression recovers data exactly (PNG, GIF); lossy compression sacrifices some quality for higher compression (JPEG, MPEG).

## 📌 4.9 Lossless Compression Techniques

├── Definition: Methods that exactly recover the original image data by exploiting coding and spatial redundancy.

├── Key Points:

| Technique | Approach | Used in |
| --- | --- | --- |
| Huffman | Variable-length prefix code | JPEG, PNG |
| RLE | Replace runs with (count, value) | BMP, FAX |
| LZW | Dictionary-based substring coding | GIF, TIFF |
| Arithmetic | Encodes message as single number | JPEG 2000 |
| DPCM | Predict + encode error | Lossless JPEG |

├── RLE Example: AAAAABBBCCDA → 5A 3B 2C 1D 1A

├── One-line Exam Answer: Lossless techniques include Huffman (variable-length coding), RLE (run-length), LZW (dictionary), Arithmetic, and DPCM (predictive).

## 📌 4.10 Lossy Predictive Coding (DPCM)

├── Definition: Predicts each pixel from past pixels and quantizes the prediction error for compression.

├── Key Points (Algorithm):

Predict the pixel: f̂(n) = a₁f(n−1) + a₂f(n−2) + ...

Compute error: e(n) = f(n) − f̂(n)

Quantize the error: ê(n) = Q(e(n))

Encode the quantized error

Decoder reconstructs by predictor + adding ê(n)

├── Trade-off: Smaller quantization step → better quality but larger file

├── One-line Exam Answer: Lossy predictive coding (DPCM) predicts pixels from neighbors and quantizes the prediction error to achieve compression.

## 📌 4.11 Vector Quantization (VQ)

├── Definition: A compression technique that maps groups (vectors) of pixels to a small codebook of representative vectors and stores only their indices.

├── Key Points:

Codebook generated using LBG (Linde-Buzo-Gray) or K-means algorithm

Encoding: divide image into blocks, find nearest codeword, store index

Decoding: lookup codeword by index, reassemble

CR depends on codebook size (e.g., 16-pixel block + 8-bit index = 16× compression)

Applications: image, speech, video compression

├── LBG Algorithm:

Initialize K codewords

Assign each training vector to nearest codeword (Euclidean)

Recompute codeword centroids

Iterate until distortion converges

├── One-line Exam Answer: Vector quantization maps image blocks to indices in a codebook built via LBG / K-means clustering.

## 📌 4.12 JPEG Compression Standard

├── Definition: JPEG (Joint Photographic Experts Group) is the most widely used lossy compression standard for photographic images, based on the 8×8 DCT.

├── Key Points (8-Step Algorithm):

Color conversion — RGB → YCbCr

Chrominance subsampling — 4:2:0 (halve Cb, Cr resolution)

Block splitting — divide image into 8×8 blocks

DCT — apply 2D-DCT to each 8×8 block

Quantization — divide DCT coefficients by Q-table (LOSSY step)

Zigzag scan — reorder 64 coefficients (DC → low → high freq)

Run-length encoding — encode runs of zeros

Huffman coding — entropy-code remaining values

├── Diagram:

RGB → YCbCr → Subsample(4:2:0) → 8×8 Blocks → DCT →
Quantize → Zigzag → RLE → Huffman → JPEG file

├── DCT Formula:

F(u, v) = (1/4) · C(u) · C(v) · ΣΣ f(x, y) · cos[(2x+1)uπ/16] · cos[(2y+1)vπ/16]

where C(u) = 1/√2 for u=0; 1 otherwise

├── CR achieved: 10:1 – 50:1 with minimal visible loss

├── One-line Exam Answer: JPEG pipeline: RGB → YCbCr → Subsample → 8×8 DCT → Quantize → Zigzag → RLE → Huffman; achieves 10:1 – 50:1 compression.

# 🟪 UNIT – V: MORPHOLOGICAL IMAGE PROCESSING

## 📌 5.1 Morphological Image Processing — Preliminaries

├── Definition: Morphological image processing uses set theory to analyze and process shapes in images, primarily binary images but extendable to grayscale.

├── Key Points:

Two basic operations: dilation (⊕) and erosion (⊖)

Two composite operations: opening (∘) and closing (•)

Operations use a structuring element (SE) as a probe

Binary image = set of foreground pixels in Z²

Applications: boundary extraction, noise removal, skeletonization, OCR

├── Set Operations Used:

| Operation | Symbol |
| --- | --- |
| Union | A ∪ B |
| Intersection | A ∩ B |
| Complement | A^c |
| Difference | A − B |
| Reflection | B̂ |
| Translation | (A)_z |

├── One-line Exam Answer: Morphological image processing applies set-theoretic operations (dilation, erosion, opening, closing, hit-or-miss) to analyze image shapes.

## 📌 5.2 Structuring Element (SE)

├── Definition: A structuring element is a small binary mask used as a probe to investigate the structure of a larger image.

├── Key Points:

Has a defined origin (typically the center)

Common shapes: 3×3 square, cross (+), disk, line

Choice of SE determines the effect of the operation

SE is reflected (B̂) in erosion and hit-or-miss operations

3×3 square is most common

├── Examples:

3×3 Square:          Cross (+):           Disk (5×5):
1 1 1                0 1 0                0 1 1 1 0
1 1 1                1 1 1                1 1 1 1 1
1 1 1                0 1 0                1 1 1 1 1
                                          1 1 1 1 1
                                          0 1 1 1 0

├── One-line Exam Answer: A structuring element is a small mask (square, cross, disk) with a defined origin, used as a probe in morphological operations.

## 📌 5.3 Dilation

├── Definition: Dilation of set A by structuring element B, denoted A ⊕ B, expands binary objects by adding pixels where B intersects A.

├── Key Points & Formula:

Formula: A ⊕ B = { z | (B̂)_z ∩ A ≠ ∅ }

Effects: grows / thickens objects, fills small holes, connects nearby parts

Properties: commutative, associative

Dual to erosion via complement: (A ⊖ B)^c = A^c ⊕ B̂

Applications: bridge small gaps, thicken thin features, OCR preprocessing

├── Example (1 pixel dilated by 3×3 SE → 3×3 block):

A:                A ⊕ B:
0 0 0 0 0         0 0 0 0 0
0 0 0 0 0         0 1 1 1 0
0 0 1 0 0    →    0 1 1 1 0
0 0 0 0 0         0 1 1 1 0
0 0 0 0 0         0 0 0 0 0

├── One-line Exam Answer: Dilation A ⊕ B grows binary objects, fills small holes, and connects nearby parts using SE B.

## 📌 5.4 Erosion

├── Definition: Erosion of set A by structuring element B, denoted A ⊖ B, shrinks binary objects by keeping only pixels where B fully fits inside A.

├── Key Points & Formula:

Formula: A ⊖ B = { z | B_z ⊆ A }

Effects: shrinks / thins objects, removes small noise specks, eliminates thin connections

NOT commutative

Dual to dilation: (A ⊖ B)^c = A^c ⊕ B̂

Applications: noise removal, separate touching objects, find boundaries

├── Example (3×3 block eroded → single pixel):

A:                A ⊖ B:
0 0 0 0 0         0 0 0 0 0
0 1 1 1 0         0 0 0 0 0
0 1 1 1 0    →    0 0 1 0 0
0 1 1 1 0         0 0 0 0 0
0 0 0 0 0         0 0 0 0 0

├── One-line Exam Answer: Erosion A ⊖ B shrinks objects, removes small noise specks, and eliminates thin features; dual to dilation.

## 📌 5.5 Opening

├── Definition: Opening of A by B (A ∘ B) is erosion followed by dilation — smooths contours, removes small bright objects, and breaks thin connections.

├── Key Points & Formula:

Formula: A ∘ B = (A ⊖ B) ⊕ B

Effects: removes small bright objects, smooths outer contours, preserves overall size

Anti-extensive: A ∘ B ⊆ A

Idempotent: (A ∘ B) ∘ B = A ∘ B

Applications: salt-noise removal, object outline smoothing, granulometry

├── One-line Exam Answer: Opening A ∘ B = (A ⊖ B) ⊕ B — erodes then dilates; removes small bright objects, smooths contours, removes salt noise.

## 📌 5.6 Closing

├── Definition: Closing of A by B (A • B) is dilation followed by erosion — fills small holes inside objects, connects nearby objects, and smooths concave contours.

├── Key Points & Formula:

Formula: A • B = (A ⊕ B) ⊖ B

Effects: fills small holes, connects nearby objects, smooths concave contours, preserves overall shape

Extensive: A ⊆ A • B

Idempotent: (A • B) • B = A • B

Applications: pepper-noise removal, fill gaps in characters (OCR)

├── Opening vs Closing:

| Feature | Opening (A ∘ B) | Closing (A • B) |
| --- | --- | --- |
| Sequence | Erode → Dilate | Dilate → Erode |
| Removes | Small bright objects | Small dark holes |
| Effect | Smooth outer contour | Smooth inner contour |
| Noise removed | Salt (bright specks) | Pepper (dark specks) |

├── One-line Exam Answer: Closing A • B = (A ⊕ B) ⊖ B — dilates then erodes; fills small holes and removes pepper noise.

## 📌 5.7 Hit-or-Miss Transformation (HMT)

├── Definition: A morphological operation that detects specific shapes / patterns in a binary image using TWO disjoint structuring elements — one for foreground (hit) and one for background (miss).

├── Key Points & Formula:

Formula: A ⊛ B = (A ⊖ B₁) ∩ (A^c ⊖ B₂)

B₁ = foreground pattern (must fit inside A)

B₂ = background pattern (must fit inside A^c)

B₁ and B₂ must be disjoint

Applications: corner detection, isolated point detection, line endpoint detection, thinning, skeletonization

├── Top-Left Corner Detection Example:

B₁ (foreground hit):     B₂ (background miss):
0 0 0                    1 1 1
0 1 1                    1 0 0
0 1 1                    1 0 0

├── One-line Exam Answer: Hit-or-miss A ⊛ B = (A ⊖ B₁) ∩ (A^c ⊖ B₂) detects specific shapes using a foreground-hit and background-miss SE pair.

## 📌 5.8 Basic Morphological Algorithms

├── Definition: Composite morphological algorithms built on dilation and erosion for shape analysis.

├── Algorithm Reference Table:

| Algorithm | Formula | Purpose |
| --- | --- | --- |
| Boundary extraction | β(A) = A − (A ⊖ B) | Find object edges |
| Hole filling | X_k = (X_{k-1} ⊕ B) ∩ A^c | Fill regions |
| Connected components | X_k = (X_{k-1} ⊕ B) ∩ A | Label regions |
| Convex hull | Iterative dilation with corner SEs | Smallest convex set |
| Thinning | A ⊗ B = A − (A ⊛ B) | Skeletons |
| Thickening | A ⊙ B = A ∪ (A ⊛ B) | Reverse of thinning |
| Skeleton | Successive thinning | Medial axis |
| Pruning | Remove short branches | Clean skeleton |

├── One-line Exam Answer: Basic morphological algorithms — boundary extraction, hole filling, connected components, convex hull, thinning, skeleton, pruning — are all built from dilation and erosion.

# 🟫 BONUS — IMAGE TRANSFORMS (Supplementary)

## 📌 6.1 Discrete Fourier Transform (DFT)

├── Definition: The DFT decomposes a digital image into its frequency components using sinusoidal basis functions.

├── Key Points:

Forward 2D-DFT: F(u, v) = ΣΣ f(x, y) · exp[−j2π(ux/M + vy/N)]

Inverse: f(x, y) = (1/MN) ΣΣ F(u, v) · exp[+j2π(ux/M + vy/N)]

Output is complex (magnitude + phase)

Implemented efficiently as FFT with O(N log N) complexity

Applications: filtering, restoration, compression

├── One-line Exam Answer: DFT transforms an image to the frequency domain using complex sinusoidal basis and is implemented efficiently as FFT.

## 📌 6.2 Discrete Cosine Transform (DCT)

├── Definition: The DCT decomposes an image into real-valued cosine basis functions — the foundation of JPEG and MPEG.

├── Key Points:

2D DCT (8×8 block):

F(u, v) = (1/4) · C(u) · C(v) · ΣΣ f(x, y) · cos[(2x+1)uπ/16] · cos[(2y+1)vπ/16]

where C(u) = 1/√2 for u=0; 1 otherwise

Real-valued output (vs complex DFT)

Energy compaction — concentrates info in low-frequency coefficients

Separable: row DCT then column DCT

Used in JPEG, MPEG, H.26x video codecs

├── DFT vs DCT:

| Feature | DFT | DCT |
| --- | --- | --- |
| Output | Complex | Real |
| Basis | Sinusoid | Cosine |
| Energy compaction | Moderate | High |
| Use | Filtering | JPEG, MPEG |

├── One-line Exam Answer: DCT decomposes an image into real cosine basis with strong energy compaction — foundation of JPEG and MPEG compression.

## 📌 6.3 Walsh-Hadamard Transform (WHT)

├── Definition: A transform using ±1 valued orthogonal basis functions — extremely fast as it requires no multiplications.

├── Key Points:

Hadamard matrix order 2: H₂ = ½ · [[1, 1], [1, −1]]

All basis values are +1 or −1

No multiplication required → very fast

Less energy compaction than DCT

Used in fast/simple compression, early video standards

├── One-line Exam Answer: The Walsh-Hadamard Transform uses ±1 orthogonal basis functions for fast image decomposition without multiplications.

## 📌 6.4 Haar Transform

├── Definition: The Haar Transform is the simplest wavelet transform using rectangular basis functions — captures both spatial location and frequency.

├── Key Points:

Simplest wavelet — rectangular basis

Multi-resolution analysis capability

Captures localized features (unlike DFT)

O(N) fast implementation

Foundation of JPEG 2000

├── One-line Exam Answer: The Haar Transform is the simplest wavelet using rectangular basis functions, supporting multi-resolution analysis used in JPEG 2000.

## 📌 6.5 Karhunen-Loève Transform (KLT / PCA)

├── Definition: The KLT decorrelates pixels by projecting them onto eigenvectors of the image covariance matrix — optimal in mean-square-error sense.

├── Key Points (Algorithm):

Compute mean m_x of pixel vectors

Compute covariance matrix C_x

Compute eigenvectors of C_x

Form transform matrix A with eigenvectors as rows

Transform: y = A(x − m_x)

Inverse: x = Aᵀy + m_x

├── Properties:

Optimal MSE — best energy compaction

Data-dependent — different basis per image

Computationally expensive (eigen decomposition)

Used in face recognition (Eigenfaces), compression theory

├── KLT vs DCT:

| Feature | KLT | DCT |
| --- | --- | --- |
| Optimality | Optimal MSE | Near-optimal |
| Basis | Data-dependent | Fixed |
| Computation | Expensive | Fast |
| Use | Theory, PCA | Standard codecs |

├── One-line Exam Answer: The KL Transform (PCA) projects pixels onto eigenvectors of the covariance matrix and is optimal in MSE but data-dependent and computationally expensive.

# END OF SYLLABUS NOTES