# DCCN — COMPLETE EXAM PREPARATION (Problem Set + Previous Year Paper)

Course: BCA / B.Sc(IT) | Subject Code: US6001 / UCS6001 | Sem VI

# PART A — PROBLEM SET 1-5 (EXPLAINED & SOLVED)

## 🟦 PROBLEM SET I — UNIT 1: INTRODUCTION TO NETWORKS

### Q1. Consequences of Connection Failure (5 devices in each topology)

What it tests: Understanding fault tolerance of network topologies.

a) Mesh Topology

Total dedicated links = n(n-1)/2 = 10 links (each device connected to all others)

If one link fails → traffic reroutes through alternate paths

Conclusion: No service disruption — high reliability

b) Star Topology (5 devices + central hub)

If a device-to-hub link fails → only that one device disconnects

If the central hub fails → entire network goes down (single point of failure)

c) Bus Topology

All devices share a single backbone cable

If a tap (drop line) fails → only that device disconnects

If backbone breaks → network splits into two non-functional halves

d) Ring Topology

Each device connected to exactly two neighbors

One break in unidirectional ring → entire network fails

Dual-ring (e.g., FDDI) provides self-healing capability

### Q2. Propagation & Transmission Time

What it tests: Difference between propagation delay (signal travel time) and transmission delay (time to push bits onto wire).

Given: Message = 2.5 KB, Bandwidth = 1 Gbps, Distance = 12,000 km, Speed = 2.4 × 10⁸ m/s

Step 1 — Convert units:

Message size = 2.5 × 1000 × 8 = 20,000 bits

Distance = 12,000 km = 1.2 × 10⁷ m

Step 2 — Propagation time:

T_prop = Distance / Speed = (1.2 × 10⁷) / (2.4 × 10⁸) = 0.05 s = 50 ms

Step 3 — Transmission time:

T_trans = Size / Bandwidth = 20,000 / 10⁹ = 20 μs

Insight: For short messages over long distances, propagation delay dominates.

### Q3. Channel Capacity (B=4 kHz, L=4, Noiseless)

What it tests: Nyquist's theorem for noiseless channel.

Formula: C = 2 × B × log₂(L)

Calculation: C = 2 × 4000 × log₂(4) = 8000 × 2 = 16,000 bps = 16 kbps

### Q4. Twisted Pair vs Coaxial vs Optical Fiber

What it tests: Knowledge of guided transmission media.

| Feature | Twisted Pair | Coaxial Cable | Optical Fiber |
| --- | --- | --- | --- |
| Cost | Cheapest | Moderate | Most expensive |
| Bandwidth | Up to 100 Mbps | Up to 1 Gbps | THz (very high) |
| Distance | ~100 m | ~500 m | 100+ km |
| EMI Resistance | Susceptible | Moderate | Immune |
| Signal Type | Electrical | Electrical | Light |
| Use | LAN, telephone | Cable TV | Internet backbone |

### Q5. Packet/Frame at Each Hop (A → R1 → D)

What it tests: Difference between logical (IP) and physical (MAC) addresses across hops.

Setup: A(40) — LAN1 — R1(B/42, C/82) — LAN2 — D(80)

HOP 1 (A → R1 across LAN1):

Network layer (Packet): SrcIP = A, DstIP = D

Data link (Frame): SrcMAC = 40, DstMAC = 42

HOP 2 (R1 → D across LAN2):

Network layer (Packet): SrcIP = A, DstIP = D (unchanged)

Data link (Frame): SrcMAC = 82, DstMAC = 80

Key insight: IP addresses remain constant end-to-end; MAC addresses change at every hop.

### Q6. Presentation Layer in Internet Model

What it tests: Differences between OSI and TCP/IP layer mappings.

Answer: In the TCP/IP model, there is no separate Presentation Layer. The duties of translation, encryption, and compression are handled by the Application Layer.

Reason: TCP/IP collapses OSI's Application + Presentation + Session into one Application Layer. Programs use protocols like SSL/TLS (encryption), MIME (translation), and gzip (compression).

### Q7. Corrupted Logical Destination Address

What it tests: Understanding of error handling at network layer.

What happens:

Routers use the corrupted IP for routing table lookup

If no matching route → packet dropped, ICMP Destination Unreachable sent to source

If accidentally valid → packet reaches wrong host (likely dropped silently)

If wandering with no destination → TTL expires → ICMP Time Exceeded

Source notification: Through ICMP error messages sent back to source IP.

### Q8. Two Computers + Ethernet Hub at Home

What it tests: Classification of networks by geographic scope.

Answer: This is a LAN (Local Area Network).

Reasons:

Limited to a single home (small geographic area)

Owned/operated by one user

Uses Ethernet — a LAN technology

LAN < 1 km; MAN = city; WAN = country/global

### Q9. Two Acknowledgement Strategies

What it tests: Trade-offs in protocol design.

Strategy 1 — Per-packet ACK (no whole-file ACK):

✅ Quick error detection, retransmit only lost packet

❌ ACK overhead, last ACK may be lost (sender thinks complete)

Strategy 2 — Single whole-file ACK:

✅ Less ACK overhead, simpler

❌ One bit error → retransmit entire file

Best practice: Combine both (TCP + application layer).

### Q10. Image Transmission Time (1920×1280, 5 bytes/pixel, 56 kbps)

What it tests: Real-world calculation of transmission time.

Step 1 — Image size:

Total pixels = 1920 × 1280 = 2,457,600

Total bytes = 2,457,600 × 5 = 12,288,000 bytes

Total bits = 12,288,000 × 8 = 98,304,000 bits

Step 2 — Time:

Time = 98,304,000 / 56,000 = 1755.4 sec ≈ 29 min 15 sec

## 🟩 PROBLEM SET II — UNIT 2: PHYSICAL & DATA LINK LAYER

### Q1. Services of Data Link Layer

What it tests: Core functions of layer 2.

Framing — divides bit stream into frames

Physical (MAC) addressing — adds source/destination MAC

Flow control — prevents overwhelming slow receiver

Error control — CRC, ARQ for detection/correction

Access control — manages shared medium (CSMA, polling)

Reliable delivery — ACK and retransmission

### Q2. Hamming Distance Calculations

What it tests: Understanding of bit-level differences.

Definition: Number of bit positions where two strings differ. Use XOR + count 1's.

| Code Words | Comparison | Hamming Distance |
| --- | --- | --- |
| d(10000, 00000) | Position 1 differs | 1 |
| d(10101, 10000) | Positions 3,5 differ | 2 |
| d(11111, 11111) | None differ | 0 |
| d(000, 000) | None differ | 0 |

### Q3. Even Parity Bit Calculation

What it tests: Single-bit error detection technique.

Rule: Even parity bit = 0 if number of 1's is already even; 1 if odd.

| Data | Count of 1's | Parity Bit |
| --- | --- | --- |
| 1001011 | 4 (even) | 0 |
| 0001100 | 2 (even) | 0 |
| 1000000 | 1 (odd) | 1 |
| 1110111 | 6 (even) | 0 |

### Q4. 7-bit Hamming Code for 1011 + Correction of 1011011

What it tests: Construction and correction of Hamming code.

Position layout: P1 P2 D1 P4 D2 D3 D4 → 1 2 3 4 5 6 7

Data 1011: D1=1, D2=0, D3=1, D4=1

Parity bit calculations (even parity):

P1 (positions 1,3,5,7) → P1 ⊕ 1 ⊕ 0 ⊕ 1 = 0 → P1 = 0

P2 (positions 2,3,6,7) → P2 ⊕ 1 ⊕ 1 ⊕ 1 = 0 → P2 = 1

P4 (positions 4,5,6,7) → P4 ⊕ 0 ⊕ 1 ⊕ 1 = 0 → P4 = 0

Hamming codeword: 0110011

Now correct the received codeword 1011011:

| Position | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit | 1 | 0 | 1 | 1 | 0 | 1 | 1 |

C1 = 1⊕1⊕0⊕1 = 1

C2 = 0⊕1⊕1⊕1 = 1

C4 = 1⊕0⊕1⊕1 = 1

Syndrome = C4C2C1 = 111 = 7 → Error at bit 7 → Flip bit 7 → Corrected: 1011010

### Q5. Packet Switching vs Circuit Switching

What it tests: Switching paradigms and delay characteristics.

| Feature | Circuit Switching | Packet Switching |
| --- | --- | --- |
| Path | Dedicated end-to-end | No dedicated path |
| Phases | Setup, Transfer, Teardown | Just Transfer |
| Bandwidth | Reserved | Shared |
| Delay | High setup, then minimal | Variable per packet |
| Reliability | More reliable | Less reliable |
| Example | Telephone | Internet |

Delay analysis:

Circuit: Total = setup + transmission + propagation

Packet: Total = N × (queuing + processing + transmission + propagation)

### Q6. CRC for 10011101 with Generator x³+1 (= 1001)

What it tests: Error detection using polynomial division.

Step 1 — Append 3 zeros: 10011101000

Step 2 — XOR division:

10011101000
⊕ 10010000000   (XOR at position 0)
  -----------
  00001101000
⊕ 00001001000   (XOR at position 4)
  -----------
  00000100000
⊕ 00000100100   (XOR at position 5)
  -----------
  00000000100   ← Remainder = 100

Step 3 — Transmitted: 10011101 + 100 = 10011101100

Step 4 — If 3rd bit inverted: 10111101100

Receiver divides by 1001:

10111101100
⊕ 10010000000
  -----------
  00101101100
⊕ 00100100000  (at position 2)
  -----------
  00001001100
⊕ 00001001000  (at position 4)
  -----------
  00000000100   ← Remainder = 100 ≠ 0

Conclusion: Non-zero remainder ⇒ error detected ✓

### Q7. CSMA/CD vs CSMA/CA

What it tests: Wired vs wireless medium access protocols.

| Feature | CSMA/CD | CSMA/CA |
| --- | --- | --- |
| Used in | Wired (Ethernet 802.3) | Wireless (Wi-Fi 802.11) |
| Approach | Detect collision after | Avoid collision before |
| Mechanism | Listen → Send → Detect → Backoff | Listen → IFS → Backoff → RTS/CTS → Send → ACK |
| Why different | Wired can detect collisions | Wireless cannot (hidden node) |

CSMA/CD Flow: Sense → if idle send → if collision detected, send jam → exponential backoff → retry

CSMA/CA Flow: Sense → wait IFS → random backoff → RTS → wait CTS → send data → wait ACK

### Q8. Checksum (32-bit data: 10011001 11100010 00100100 10000100)

What it tests: Internet checksum (1's complement addition).

Segments: A=10011001, B=11100010, C=00100100, D=10000100

Sender side:

A + B = 10011001 + 11100010 = 1 01111011 → wrap-around → 01111100 (124)

(A+B) + C = 01111100 + 00100100 = 10100000 (160)

(A+B+C) + D = 10100000 + 10000100 = 1 00100100 → wrap → 00100101 (37)

Sum = 00100101 → Checksum = 1's complement = 11011010

Receiver side: Adds all segments + checksum = 11111111 → 1's complement = 00000000 → No error ✓

### Q9. Go-Back-N ARQ Working

What it tests: Sliding window protocol for reliable delivery.

Rules:

Sender window = N, Receiver window = 1

Cumulative ACK

Loss/timeout → retransmit lost frame and ALL after it

Receiver discards out-of-order frames

Diagram:

Sender:    F0  F1  F2  F3  F4  F5
              ↓   ↓   X   ↓   ↓
Receiver:  F0✓ F1✓     F3✗ F4✗ (discarded)
              ACK1 ACK2
                       TIMEOUT
Sender:    F2  F3  F4  F5  (all retransmitted)

Drawback: Wastes bandwidth retransmitting already-received frames.

### Q10. Framing & its Methods

What it tests: Techniques to delimit frames at data link layer.

Definition: Process of dividing bit stream into frames with start/end markers.

Methods:

Character (Byte) Count — first byte = frame length

Flag Bytes with Byte Stuffing — escape character before flag in data

Flag Bits with Bit Stuffing — insert 0 after every 5 consecutive 1's

Physical Layer Coding Violations — illegal bit patterns mark boundaries

## 🟨 PROBLEM SET III — UNIT 3: LLC / MAC

### Q1. Why Go-Back-N is more popular than Stop-and-Wait

Answer: Go-Back-N pipelines transmission while Stop-and-Wait waits for each ACK individually, wasting bandwidth on long links. Modern high-speed networks have high bandwidth-delay product, where Stop-and-Wait achieves <5% utilization. Go-Back-N is also simpler than Selective Repeat (no buffer at receiver).

Efficiency: η_GBN = N / (1 + 2a) vs η_SW = 1 / (1 + 2a)

### Q2. Pure ALOHA vs Slotted ALOHA at Low Load

Answer: Pure ALOHA has less delay at low load.

Reason: In slotted ALOHA, even if channel is idle, station must wait for slot boundary. Pure ALOHA can transmit immediately. At low load, collisions are rare in both, so pure ALOHA's immediate-send wins.

Trade-off: At high load, slotted ALOHA performs better (36.8% vs 18.4% efficiency).

### Q3. Unicast vs Multicast vs Broadcast

| Type | Meaning | IP Range |
| --- | --- | --- |
| Unicast | One-to-one | Class A/B/C |
| Multicast | One-to-group | 224.0.0.0 – 239.255.255.255 (Class D) |
| Broadcast | One-to-all | 255.255.255.255 (limited) |

### Q4. Networks and Hosts in IP Address Classes

| Class | Range | # Networks | # Hosts/Network |
| --- | --- | --- | --- |
| A | 0–127 | 128 (126 usable) | 16,777,214 |
| B | 128–191 | 16,384 | 65,534 |
| C | 192–223 | 2,097,152 | 254 |
| D | 224–239 | Multicast | – |
| E | 240–255 | Reserved | – |

### Q5. ICMP and its Types

ICMP = Internet Control Message Protocol — error reporting and diagnostic protocol at network layer.

Error-reporting messages:

Destination Unreachable

Source Quench (deprecated)

Time Exceeded

Parameter Problem

Redirect

Query messages:

Echo Request/Reply (ping)

Timestamp Request/Reply

Address Mask Request/Reply

Router Solicitation/Advertisement

### Q6. Subnets and Hosts for 255.255.255.224

Mask: 11111111.11111111.11111111.11100000

Subnet bits = 3 → Subnets = 2³ = 8

Host bits = 5 → Hosts/subnet = 2⁵ – 2 = 30

### Q7. CSMA/CD vs CSMA/CA

(Same as Unit 2 Q7 — refer above.)

### Q8. Maximum Hosts for 255.255.240.0 + CIDR

Mask: 11111111.11111111.11110000.00000000

Host bits = 12 → Maximum hosts = 2¹² – 2 = 4094

CIDR (Classless Inter-Domain Routing):

Replaces classful addressing with prefix notation /n (e.g., 192.168.1.0/24)

Supports VLSM (Variable-Length Subnet Mask)

Enables route aggregation (supernetting)

Mask 255.255.240.0 = /20 in CIDR

### Q9. Selective Repeat ARQ

Concept: Sender sends N frames; receiver buffers out-of-order frames; only LOST frame retransmitted.

Rules:

Sender window = N, Receiver window = N

Each frame individually ACKed

Receiver buffers out-of-order frames

Diagram:

Sender:    F0  F1  F2  F3  F4
              ↓   ↓   X   ↓   ↓
Receiver:  F0✓ F1✓     F3 buf F4 buf
                       TIMEOUT for F2
Sender:    F2 only
Receiver:  F2✓ → delivers F2,F3,F4 in order

Advantage: Bandwidth efficient. Disadvantage: Receiver buffer needed.

### Q10. IPv4 Header Format

Total: 20–60 bytes

| Field | Size | Role |
| --- | --- | --- |
| Version | 4 bits | IP version (4) |
| IHL | 4 bits | Header length in 32-bit words |
| Type of Service | 8 bits | Priority/QoS |
| Total Length | 16 bits | Datagram total size |
| Identification | 16 bits | Fragment grouping |
| Flags | 3 bits | DF, MF |
| Fragment Offset | 13 bits | Position of fragment |
| TTL | 8 bits | Time to live |
| Protocol | 8 bits | Upper-layer (TCP=6, UDP=17, ICMP=1) |
| Header Checksum | 16 bits | Error detection |
| Source IP | 32 bits | Sender IP |
| Destination IP | 32 bits | Receiver IP |
| Options + Padding | Variable | Optional |

## 🟧 PROBLEM SET IV — UNIT 4: NETWORK / TRANSPORT

### Q1. TCP vs UDP Headers

TCP (20-60 bytes): Source/Dest Port, Seq#, ACK#, HLen, Flags (URG/ACK/PSH/RST/SYN/FIN), Window, Checksum, Urgent Ptr, Options

UDP (8 bytes): Source/Dest Port, Length, Checksum

Fields TCP has that UDP lacks:

| Field | Purpose |
| --- | --- |
| Sequence # | Ordered delivery |
| ACK # | Reliable delivery |
| HLen | Variable header due to options |
| Flags | Connection control |
| Window Size | Flow control |
| Urgent Pointer | Out-of-band data |

Reason: TCP needs these for reliable, ordered, flow-controlled, connection-oriented service. UDP is unreliable and connectionless, so it doesn't need them.

### Q2. Cryptography in Presentation Layer

Need: Confidentiality, integrity, authentication, non-repudiation.

| Feature | Symmetric | Asymmetric |
| --- | --- | --- |
| Keys | One shared | Public + Private |
| Speed | Fast | Slow |
| Distribution | Hard | Easy |
| Algorithms | DES, AES, 3DES | RSA, ECC, DSA |
| Use | Bulk data | Key exchange, signatures |

### Q3. Digital Signature & Security Attacks

Digital Signature:

Sender hashes message → encrypts with private key → signature

Sends message + signature

Receiver decrypts signature with sender's public key → recovers hash

Computes hash of message → compares

If match → valid (authentic + integrity + non-repudiation)

Security Attacks:

Passive: Eavesdropping, Traffic analysis

Active: Masquerade, Replay, Modification, Denial of Service (DoS)

### Q4. RSA: N=119, E=5, D=77, send 'F'

Step 1: F = 6 (alphabet position)

Step 2: Cipher = M^E mod N = 6⁵ mod 119

6² = 36

6⁴ = 36² = 1296 mod 119 = 1296 – 1190 = 106

6⁵ = 106 × 6 = 636 mod 119 = 636 – 595 = 41

Cipher Text C = 41

Decryption: M = 41⁷⁷ mod 119 = 6 ✓

### Q5. Sockets & Socket Types

Socket: Endpoint of bidirectional communication identified by (IP + Port).

Types:

Stream socket (SOCK_STREAM) — TCP, reliable, connection-oriented

Datagram socket (SOCK_DGRAM) — UDP, unreliable, connectionless

Raw socket (SOCK_RAW) — direct access to lower layers (used by ping, traceroute)

Sequenced packet socket (SOCK_SEQPACKET) — reliable + record-oriented

### Q6. Transport Service Primitives & Three-Way Handshake

Primitives: LISTEN, CONNECT, SEND, RECEIVE, DISCONNECT

Three-Way Handshake (TCP setup):

Client                          Server
  |---- SYN, seq=x ------------>|
  |<--- SYN+ACK, seq=y, ack=x+1 |
  |---- ACK, ack=y+1 ---------->|
  |==== Connection Established ==|

Problem solved: Synchronization of sequence numbers, prevents duplicate old connection requests, ensures both parties are alive and ready.

### Q7. QoS Parameters

Reliability — error rate, loss percentage

Delay (Latency) — end-to-end time

Jitter — variation in delay

Bandwidth — throughput (bps)

### Q8. Firewall: Packet Filter vs Proxy Server

| Feature | Packet Filter | Proxy Server |
| --- | --- | --- |
| Layer | Network (L3) | Application (L7) |
| Inspects | Headers (IP/port/protocol) | Full content |
| Speed | Very fast | Slower |
| Security | Basic | High |
| Caching | No | Yes |
| User Auth | No | Yes |
| Example | iptables, ACL | Squid |

### Q9. Leaky Bucket Algorithm

Purpose: Traffic shaping — smooths bursty traffic to constant output rate.

Concept:

Bursty input
       ↓ ↓ ↓
   ┌─────────┐
   │ Bucket  │ ← FIFO queue
   │_________│
        |
        ↓ (constant rate)
   Smooth output

Working: Packets poured in; leak at constant rate; if bucket overflows, drop packets.

Pros: Controls average rate. Cons: No flexibility for bursts (Token Bucket allows this).

### Q10. ICMP & Types

(Same as Unit 3 Q5 — refer above.)

## 🟪 PROBLEM SET V — UNIT 5: APPLICATION LAYER

### Q1. Proxy Server vs Web Server

Web Server: Hosts and delivers web pages over HTTP/HTTPS (port 80/443). Examples: Apache, Nginx.

Proxy Server: Intermediate server between client and origin. Functions: caching, filtering, anonymity, load balancing.

Types: Forward proxy (for clients), Reverse proxy (for servers).

### Q2. DNS — What if all DNS servers crashed?

DNS: Hierarchical distributed database mapping domain names → IP addresses (UDP port 53).

If all DNS crashed:

Domain name resolution fails

Only direct IP access works (e.g., http://142.250.190.46)

Cached entries work until TTL expires

Email delivery fails (MX lookup)

Most internet apps break

Practically the entire internet becomes unusable

### Q3. HTTP vs SMTP

| Feature | HTTP | SMTP |
| --- | --- | --- |
| Purpose | Web pages | Email transfer |
| Type | Pull | Push |
| Port | 80 / 443 | 25 / 587 |
| State | Stateless | Stateful |
| Direction | Client ↔ Server | Server ↔ Server |

### Q4. Hyperlinks & Hypermedia

Hyperlink: Clickable reference within hypertext that navigates to another resource (<a href="url">)

Hypermedia: Hypertext + multimedia (images, audio, video, animations)

### Q5. Hierarchy of Name Servers

Root DNS Servers (13 globally, A–M)

TLD Servers (.com, .org, .in)

Authoritative DNS Servers (specific domains)

Local DNS Servers (ISP resolvers)

Resolution flow: Client → Local → Root → TLD → Authoritative → IP returned

### Q6. Why FTP sends Control Out-of-Band

FTP uses 2 TCP connections: Port 21 (control), Port 20 (data).

Reasons:

Allows ABORT/STAT during transfer (data busy, control free)

Cleaner protocol design (no escape sequences in stream)

Multiple data transfers possible

Better error handling

Independent connection lifecycle (control persists, data per-transfer)

### Q7. Types of HTTP Messages

Request methods: GET, POST, PUT, DELETE, HEAD, OPTIONS, TRACE, CONNECT

Response status codes:

1xx: Informational

2xx: Success (200 OK)

3xx: Redirection (301 Moved)

4xx: Client error (404 Not Found)

5xx: Server error (500)

Format: Request line / Status line + Headers + blank line + Body

### Q8. SMTP Commands

HELO/EHLO — identify client

MAIL FROM: — sender's address

RCPT TO: — recipient's address

DATA — begin message body (terminated by .)

RSET — reset transaction

VRFY — verify user exists

NOOP — keep alive

QUIT — end session

### Q9. Active vs Dynamic Web Page

| Feature | Active | Dynamic |
| --- | --- | --- |
| Where executed | Client (browser) | Server |
| Tools | JavaScript, Applets | PHP, ASP, JSP |
| Server load | Low | High |
| Example | Form validation | DB-driven product page |

DB Interface flow: Browser → Web server → App server (PHP) → SQL → Database → Result → HTML → Browser

### Q10. FTP vs TFTP

| Feature | FTP | TFTP |
| --- | --- | --- |
| Transport | TCP | UDP |
| Port | 20, 21 | 69 |
| Connections | Two | One |
| Authentication | Yes | No |
| Reliability | Reliable | Limited |
| Use | General file transfer | Bootstrapping, firmware |

# PART B — PREVIOUS YEAR SOLVED (UCS6001 — End Sem 2024-25)

Time: 3 hours | Max Marks: 60

## SECTION A — Very Short Answer (10 × 1 = 10 marks)

### Q1. Why HTTP is called a stateless protocol?

Answer: HTTP is stateless because each request from client to server is independent. The server does not retain any information about previous requests — every transaction is treated as new. State (e.g., login session) is maintained externally using cookies, sessions, tokens, or URL parameters, not by HTTP itself.

### Q2. Number of IPs and First/Last IP in 167.199.170.82/27

Step 1: /27 → 32-27 = 5 host bits

Step 2: Number of IP addresses = 2⁵ = 32

Step 3: Find subnet of 167.199.170.82

Last octet 82 = 01010010

Mask /27 → first 3 bits of last octet are network: 01000000 = 64

So subnet starts at 167.199.170.64

First IP (Network): 167.199.170.64

Last IP (Broadcast): 167.199.170.95

Total IPs: 32 (usable hosts: 30)

### Q3. TCP applications

Suitable for: HTTP, HTTPS, FTP, SMTP, SSH, Telnet, IMAP, POP3

Why: TCP provides reliable, ordered, error-checked delivery. Web pages, emails, file transfers must arrive complete and in order — TCP guarantees this via 3-way handshake, sequence numbers, ACKs, and retransmission.

### Q4. What is a Repeater?

Repeater is a Layer 1 (Physical) networking device that regenerates and amplifies weak signals to extend network range.

Why used: Compensates for signal attenuation over long cables, allowing signals to travel beyond physical media limits without degradation.

### Q5. Functions of Transport Layer (OSI)

Process-to-process delivery using ports

Connection management (setup, maintain, terminate)

Segmentation and reassembly of data

Flow control (sliding window)

Error control (checksum, retransmission)

Congestion control

Multiplexing/demultiplexing

Examples: TCP (reliable), UDP (unreliable).

### Q6. Propagation delay (Speed = 8 × 10⁸ m/s, Distance = 400 km)

Calculation:

Distance = 400 km = 4 × 10⁵ m

Speed = 8 × 10⁸ m/s

Propagation delay = Distance / Speed = (4 × 10⁵) / (8 × 10⁸) = 5 × 10⁻⁴ s = 0.5 ms

### Q7. Transmission delay (20,000 bits, 4 Mbps)

Calculation:

Packet size = 20,000 bits

Bandwidth = 4 Mbps = 4 × 10⁶ bps

Transmission delay = 20,000 / (4 × 10⁶) = 5 × 10⁻³ s = 5 ms

### Q8. Even Parity Bits

A. 1001011: Number of 1's = 4 (even) → Parity bit = 0

B. 1011101: Number of 1's = 5 (odd) → Parity bit = 1

### Q9. Simplex Mode

Definition: Communication occurs in only one direction (one-way only). The sender cannot become receiver, like a one-way street.

Examples (input devices using simplex):

Keyboard — sends data only to computer

Mouse — sends data only to computer

(Other examples: TV broadcast, radio broadcast)

### Q10. Download time (56 million bytes at 280 Mbps)

Calculation:

File size = 56 × 10⁶ bytes = 56 × 10⁶ × 8 = 448 × 10⁶ bits = 448 Mb

Channel = 280 Mbps

Time = 448 / 280 = 1.6 seconds

## SECTION B — Short Answer (Attempt any 8 of 9 × 2.5 = 20 marks)

### Q1. Circuit Switching: Advantages & Disadvantages

Working: Dedicated path established between sender and receiver before data transfer (3 phases: setup, transfer, teardown). Used in PSTN telephone network.

Advantages:

Guaranteed bandwidth (no congestion)

Predictable performance, low jitter

Suitable for real-time voice

Simple sequential delivery

Disadvantages:

Setup delay

Bandwidth wasted when idle

Inflexible for bursty data

Single point of failure (circuit break = call drop)

### Q2. Ring Topology: Advantages & Disadvantages

Working: Each node connected to two neighbors forming a ring. Data flows in one direction (or both in dual-ring). Token-passing controls access.

Advantages:

Equal access for all nodes (fair)

No collisions (token-based)

Performs well under heavy load

Cheaper than mesh (no central hub needed)

Disadvantages:

Single break disrupts entire ring (single ring)

Adding/removing devices disrupts network

Slower than star (data passes through nodes)

Difficult to troubleshoot

### Q3. Internet Checksum for 1001 1100 1010 0011 (4-bit words)

Words: 1001 (9), 1100 (12), 1010 (10), 0011 (3)

Step 1 — Sum with wrap-around (1's complement):

1001 + 1100 = 10101 → wrap: 0101 + 1 = 0110 (6)

0110 + 1010 = 10000 → wrap: 0000 + 1 = 0001 (1)

0001 + 0011 = 0100 (4)

Sum = 0100

Checksum = 1's complement = 1011

Verification: Receiver sums all 5 (4 data + checksum) → should yield 1111. Indeed 0100 + 1011 = 1111 ✓

### Q4. ALOHA Protocol Working

ALOHA is the earliest random access protocol developed at U. Hawaii (1970s).

Pure ALOHA:

Station transmits whenever it has data

After transmission, waits for ACK

If no ACK (collision occurred) → wait random time → retransmit

Vulnerable period: 2 × frame time (collision possible if any transmission overlaps)

Max efficiency: 18.4% (1/2e)

Slotted ALOHA:

Time divided into discrete slots (size = frame time)

Station can only transmit at slot boundary

Reduces vulnerable period to 1 × frame time

Max efficiency: 36.8% (1/e)

Pure:    [  Frame from A  ]
            [ Frame from B ]   ← collision anywhere
Slotted: |slot|slot|slot|
         [ A ][ B ]            ← collisions only within slot

### Q5. Forwarding Tables for Nodes A and F → Smallest Network

Given tables (cost = 1 per link):

Node A: B→1→B, C→1→C, D→2→B, E→3→C, F→2→C

Node F: A→2→C, B→3→C, C→1→C, D→2→C, E→1→E

Direct links from A: A—B (cost 1), A—C (cost 1)

Direct links from F: F—C (cost 1), F—E (cost 1)

Path analysis:

A→D via B (cost 2) → so B—D (cost 1)

A→F via C (cost 2) → so C—F (cost 1) ✓

A→E via C (cost 3) → A—C—F—E (1+1+1=3) ✓

F→A via C (cost 2) → F—C—A (1+1=2) ✓

F→D via C (cost 2) → so C—D (cost 1)

F→B via C (cost 3) → F—C—A—B (1+1+1=3) ✓

Smallest network has 6 nodes & 6 links:

A
       / \
      B   C
      |  /|\
      D-' | F
            \
             E

Links: A—B, A—C, B—D, C—D, C—F, F—E

### Q6. Aggregate /24 networks

Networks: 212.56.132.0/24, 212.56.133.0/24, 212.56.134.0/24, 212.56.135.0/24

Third octet binary:

132 = 10000100

133 = 10000101

134 = 10000110

135 = 10000111

Common prefix: 100001 (6 bits) — last 2 bits vary.

Aggregate prefix length: 8 + 8 + 6 = /22

Aggregated network: 212.56.132.0/22

(Covers all addresses from 212.56.132.0 to 212.56.135.255)

### Q7. Network Address Translation (NAT)

NAT is a technique that maps multiple private IP addresses to a single (or few) public IP addresses, conserving IPv4 space and adding security.

Types:

Static NAT — one-to-one fixed mapping

Dynamic NAT — one-to-many from a pool

PAT/NAPT (Port Address Translation) — many-to-one using port numbers

How it works:

[Internal: 192.168.1.5:5000] → [NAT Router] → [Public: 203.0.113.1:6001]
                                  Translation Table
                                  192.168.1.5:5000 ↔ 203.0.113.1:6001

Reply path: Public→NAT→Internal using table lookup.

Benefits:

Conserves IPv4 addresses

Hides internal addresses (security)

Allows multiple devices to share one public IP

### Q8. Why UDP is Connectionless

Reasons UDP is connectionless:

No 3-way handshake — sender can transmit immediately

No session state — each datagram is independent

No acknowledgments — sender does not wait/track ACKs

No retransmission — lost packets are not resent

No flow/congestion control — sender transmits at its own rate

Lower overhead (8-byte header vs TCP's 20-byte minimum)

Use cases: DNS, DHCP, SNMP, video streaming, VoIP, online gaming — where speed matters more than reliability.

### Q9. FTP and its Applications

FTP = File Transfer Protocol, a TCP-based application-layer protocol (RFC 959) for transferring files between client and server.

Features:

Uses two TCP connections: Port 21 (control) and Port 20 (data)

Out-of-band control — commands separate from data

Supports authentication (username/password)

Two transfer modes: ASCII / Binary

Two operation modes: Active / Passive

Applications:

Uploading websites to web hosting servers

Downloading software (open-source tools, drivers)

Backup and restore of files between servers

File sharing in offices/educational institutions

Transferring large media files (videos, datasets)

Server maintenance by sysadmins

## SECTION C — Long Answer (Attempt any 5 of 6 × 6 = 30 marks)

### Q1. Distance Vector Routing — Global DV Tables (3 stages)

Network: A—C(3), A—B(8), B—E(1), B—D(2), C—F(6), D—E(2)

Stage I — Each node knows only direct-neighbor distances:

| From\To | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| A | 0 | 8 | 3 | ∞ | ∞ | ∞ |
| B | 8 | 0 | ∞ | 2 | 1 | ∞ |
| C | 3 | ∞ | 0 | ∞ | ∞ | 6 |
| D | ∞ | 2 | ∞ | 0 | 2 | ∞ |
| E | ∞ | 1 | ∞ | 2 | 0 | ∞ |
| F | ∞ | ∞ | 6 | ∞ | ∞ | 0 |

Stage II — Each node receives and updates from neighbors:

| From\To | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| A | 0 | 8 | 3 | 10 | 9 | 9 |
| B | 8 | 0 | 11 | 2 | 1 | ∞ |
| C | 3 | 11 | 0 | ∞ | ∞ | 6 |
| D | 10 | 2 | ∞ | 0 | 2 | ∞ |
| E | 9 | 1 | ∞ | 2 | 0 | ∞ |
| F | 9 | ∞ | 6 | ∞ | ∞ | 0 |

Stage III — After second exchange (converged or near-converged):

| From\To | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| A | 0 | 8 | 3 | 10 | 9 | 9 |
| B | 8 | 0 | 11 | 2 | 1 | 17 |
| C | 3 | 11 | 0 | 13 | 12 | 6 |
| D | 10 | 2 | 13 | 0 | 2 | 19 |
| E | 9 | 1 | 12 | 2 | 0 | 18 |
| F | 9 | 17 | 6 | 19 | 18 | 0 |

Algorithm used: Bellman-Ford. Each node updates: D(x,y) = min over neighbors n of [c(x,n) + D(n,y)]

### Q2. Routing Table Forwarding for 5 Destinations

Routing table:

| SubnetNumber | SubnetMask | NextHop |
| --- | --- | --- |
| 128.96.39.0 | 255.255.255.128 | Interface 0 |
| 128.96.39.128 | 255.255.255.128 | Interface 1 |
| 128.96.40.0 | 255.255.255.128 | R2 |
| 192.4.153.0 | 255.255.255.192 | R3 |
| (default) | – | R4 |

Process: AND destination IP with each subnet mask; if result matches a SubnetNumber, forward to that NextHop. Otherwise default.

i. 128.96.39.10:

AND with 255.255.255.128 → 128.96.39.0 → Interface 0

ii. 128.96.40.12:

AND with 255.255.255.128 → 128.96.40.0 → R2

iii. 128.96.40.151:

AND with 255.255.255.128 → 128.96.40.128 → not in table → R4 (default)

iv. 192.4.153.17:

AND with 255.255.255.192 → 192.4.153.0 → R3

v. 192.4.153.90:

90 in binary = 01011010

AND with 11000000 = 01000000 = 64

Result: 192.4.153.64 → not in table → R4 (default)

### Q3. Framing Methods for "A B ESC FLAG"

Given codes:

A = 01000111

B = 11100011

FLAG = 01111110

ESC = 11100000

I. Byte Count:

First byte = total bytes including count = 5

Transmitted: 00000101 01000111 11100011 11100000 01111110

II. Flag Bytes with Byte Stuffing:

Frame surrounded by FLAG bytes. Inside data, stuff ESC before any FLAG or ESC byte.

Data: A, B, ESC, FLAG

A → A

B → B

ESC → ESC ESC (stuff ESC before ESC)

FLAG → ESC FLAG (stuff ESC before FLAG)

Transmitted:

FLAG | A | B | ESC ESC | ESC FLAG | FLAG

= 01111110 01000111 11100011 11100000 11100000 11100000 01111110 01111110

III. Flag Bits with Bit Stuffing:

Frame delimited by FLAG (01111110). After every 5 consecutive 1's in data, stuff a 0.

Original data bits (32):

01000111 11100011 11100000 01111110

Scan for 5+ consecutive 1's:

Bits 6–11: six 1's → stuff 0 after 5th 1 (after bit 10)

Bits 15–19: five 1's → stuff 0 after bit 19

Bits 26–31: six 1's (FLAG) → stuff 0 after 5th 1 (after bit 30)

Stuffed data: 01000 11111 0 1 000 11111 0 000000 11111 0 10

Concatenated stream:

01111110 01000111110100011111000000011111010 01111110

(8 + 35 + 8 = 51 bits)

### Q4. Cookies — Why used, How created and stored

Cookie: Small text file (typically <4 KB) stored on the client browser by a web server, containing data about the user's session/preferences.

Why used:

Session management — login state, shopping carts

Personalization — language, theme, preferences

Tracking — analytics, advertising

Authentication tokens — keep user logged in

How created (Server-side):

Client sends HTTP request: GET /index.html

Server responds with: Set-Cookie: sessionID=abc123; Expires=...; Secure; HttpOnly

Browser stores the cookie locally

How stored (Client-side):

Stored in browser's cookie database (file or storage area)

Each cookie has: name, value, domain, path, expiration, secure flag, HttpOnly flag

On subsequent requests:

Browser sends Cookie: sessionID=abc123 header automatically with every request to the same domain.

Types:

Session cookies — deleted when browser closes

Persistent cookies — have expiration date

Secure cookies — sent only over HTTPS

HttpOnly cookies — inaccessible to JavaScript (XSS protection)

Third-party cookies — set by domains other than the visited site

### Q5. Go-Back-N Protocol (Transport Layer)

Concept: Pipelined sliding window protocol that allows sender to transmit up to N unacknowledged frames. On loss, retransmit the lost frame and ALL subsequent ones.

Rules:

Sender window size = N (e.g., 4)

Receiver window size = 1

Cumulative ACK (ACKn means "received all up to n-1")

Receiver discards out-of-order frames

On timeout: sender retransmits all N pending frames

Example with window size N=4:

Time →
Sender:    F0 F1 F2 F3 (window full, wait for ACK)
              ↓  ↓  X  ↓
Receiver:  F0 F1    F3 (out of order — discard)
              ↑  ↑
              A1 A2 (cumulative ACK: A2 = "received up to F1")
              
              [Timeout for F2]
              
Sender:    F2 F3 F4 F5 (retransmit F2 and onwards)
              ↓  ↓  ↓  ↓
Receiver:  F2 F3 F4 F5 ✓
              ↑  ↑  ↑  ↑
              A3 A4 A5 A6

Pros: Simpler than Selective Repeat (no buffer at receiver), better than Stop-and-Wait.

Cons: Wastes bandwidth retransmitting frames already correctly received.

Efficiency: η = N / (1 + 2a) where a = T_prop/T_trans

### Q6. Slow Start (Exponential Increase) Algorithm

Slow Start is a TCP congestion control algorithm that exponentially increases the congestion window (cwnd) until either packet loss occurs or threshold (ssthresh) is reached.

Algorithm:

Initialize cwnd = 1 MSS (Maximum Segment Size)

For each ACK received → cwnd = cwnd + 1 MSS (effectively doubling per RTT)

Continue until cwnd ≥ ssthresh (slow start threshold)

Switch to Congestion Avoidance (linear increase)

On packet loss (timeout):

ssthresh = cwnd / 2

cwnd = 1 MSS

Restart slow start

Why "slow"? It starts slowly (cwnd = 1) but grows fast (exponential).

Diagram:

cwnd
  |                               (timeout)
  |              /\               /
  |             /  \-------------/   (linear: Congestion Avoidance)
  |            /     ssthresh new
  |           /       /
  |          /       /
  |         /  (exp)/
  |        / Slow  /
  |       /  Start/
  |      /       /
  |     /       /
  |    /       /
  |   /       /
  |  /       /
  | /       /
  +--------+----------+----------+----> Time (RTT)
                  (loss)

Window growth (RTT-by-RTT):

RTT 1: cwnd = 1

RTT 2: cwnd = 2

RTT 3: cwnd = 4

RTT 4: cwnd = 8

RTT 5: cwnd = 16 → if ≥ ssthresh, switch to linear

...

Goal: Probe network capacity quickly without overwhelming, then settle into stable operation.

# END OF EXAM PREPARATION DOCUMENT

**Total content:** 50 problem-set questions explained + 25 previous-year questions solved (10 + 9 + 6 = 25 attempted)

**Use this file** as your **complete final-prep** companion in the last 48 hours before exam.