# DCCN — IMPORTANT QUESTION BANK WITH ANSWER STRATEGY

Course: BCA / B.Sc(IT) | Subject Code: US6001 | Sem VI

Use this file in the **last 24 hours** before the exam to focus on highest-probability questions.

# 🔴 TOP 10 MOST LIKELY 5-MARK QUESTIONS

### Q1. Compare OSI and TCP/IP Models.

Answer (Key Points):

OSI: 7 layers; TCP/IP: 4 (or 5) layers

OSI: theoretical; TCP/IP: practical/implemented

Application layer in TCP/IP combines OSI's Application + Presentation + Session

Network access in TCP/IP combines OSI's Data Link + Physical

Tabulate layer-by-layer mapping

→ How to attempt this in exam: Draw both stacks side-by-side, then write a comparison table of 6-7 differences. Examiner expects clear visualisation.

### Q2. Explain CSMA/CD with flowchart.

Answer: Sense channel → if busy wait, if idle transmit → if collision detected, send jam signal → wait random backoff time → retry.

Flowchart with decision diamonds

Used in Ethernet (wired LAN)

Backoff is exponential

→ How to attempt this in exam: Begin with full form, then draw flowchart with 5-6 decision steps; close with where it's used (Ethernet 802.3).

### Q3. Differentiate between TCP and UDP.

Answer (Tabulate):

TCP: connection-oriented, reliable, ordered, slow, header 20 bytes

UDP: connectionless, unreliable, fast, header 8 bytes

TCP examples: HTTP, SMTP, FTP; UDP examples: DNS, DHCP, video streaming

→ How to attempt this in exam: Make a 6-7 row table covering connection, reliability, order, speed, header size, applications, flow control.

### Q4. Explain working of Go-Back-N ARQ with diagram.

Answer:

Sliding window protocol with window size N

Multiple frames in transit, cumulative ACK

On lost frame → retransmit lost + all subsequent

Receiver discards out-of-order frames

→ How to attempt this in exam: Draw timeline with frames F0-F5, show lost F2, then retransmission of F2,F3,F4,F5. Mention efficiency formula.

### Q5. Explain Hamming code construction with example.

Answer: For 4-bit data → 7-bit code; parity bits at positions 1, 2, 4 covering specific positions; even parity calculation; receiver computes syndrome to find error position.

Show example: data 1011 → code 0110011

→ How to attempt this in exam: Show position table, parity coverage chart, then construct step-by-step with XOR. Show error detection and correction with syndrome.

### Q6. Explain ICMP and its types of messages.

Answer:

ICMP = Internet Control Message Protocol

Used for error reporting & diagnostics, encapsulated in IP

Two categories: error-reporting (Destination Unreachable, Time Exceeded, Source Quench, Redirect) + query (Echo, Timestamp, Address Mask)

Used by ping, traceroute

→ How to attempt this in exam: Define ICMP, then group messages into Error and Query with brief explanation; mention ping uses Echo Request/Reply.

### Q7. Compare Symmetric vs Asymmetric Cryptography.

Answer (Tabulate):

Symmetric: 1 key, fast, key distribution problem (DES, AES)

Asymmetric: 2 keys (public/private), slow, easy distribution (RSA, ECC)

TLS uses both (asymmetric for key exchange, symmetric for data)

→ How to attempt this in exam: Define cryptography, then compare in 5-6 row table with examples and use case.

### Q8. Explain Three-Way Handshake with diagram.

Answer: Client → SYN; Server → SYN+ACK; Client → ACK. Synchronizes seq numbers, prevents duplicate connections, ensures readiness.

→ How to attempt this in exam: Draw 3-arrow timeline diagram with seq=x, ack=x+1 etc. Always end by stating the problem solved (sync of seq numbers).

### Q9. Explain Leaky Bucket algorithm.

Answer:

Bucket with constant output rate (hole at bottom)

Bursty input is smoothed

Bucket overflow → drop packets

Used for traffic shaping

→ How to attempt this in exam: Draw bucket with arrows showing variable-in, constant-out. Mention "constant output rate" as key feature.

### Q10. Explain DNS hierarchy and resolution process.

Answer:

Hierarchical: Root → TLD → Authoritative

13 root servers globally

Local DNS caches results

Iterative or recursive query types

Uses UDP port 53

→ How to attempt this in exam: Draw inverted tree diagram, then list 4 server levels and explain resolution flow with arrows.

# 🟠 TOP 10 MOST LIKELY 10-MARK QUESTIONS

### Q1. Explain OSI Reference Model with all 7 layers and their functions.

Answer Outline:

Define OSI (1984, ISO)

Diagram of 7-layer stack

Function table for each layer (Physical, Data Link, Network, Transport, Session, Presentation, Application)

PDU and devices for each layer

Encapsulation/decapsulation flow

Mnemonic: "All People Seem To Need Data Processing"

→ How to attempt this in exam: Reserve at least 2 pages. Draw the layered diagram first, then write table of layers with function/PDU/device, then explain each layer in 2-3 lines.

### Q2. Compare and contrast all ARQ protocols (Stop-and-Wait, Go-Back-N, Selective Repeat) with diagrams.

Answer Outline:

Define ARQ

Each protocol's working mechanism

Diagrams of each

Comparison table (window, buffer, retransmission, complexity, efficiency)

When to use which

→ How to attempt this in exam: Draw 3 timeline diagrams (one for each), then a comparison table. End with "Stop-and-Wait simple but slow; Go-Back-N popular; Selective Repeat efficient but complex."

### Q3. Explain IPv4 Header format with all fields.

Answer Outline:

IPv4 header is 20-60 bytes

Diagram with all 13 fields

Detail each field's role and size

Special focus on TTL, Protocol, Header Checksum, Fragmentation fields

→ How to attempt this in exam: Draw header diagram first occupying 1/2 page, then table all fields with size, role, example values.

### Q4. Explain Subnetting with example. What are the subnets and hosts for 255.255.255.224 mask?

Answer Outline:

Define subnetting

Why subnetting (efficient IP usage, security, broadcast control)

Formulas: subnets = 2ⁿ, hosts = 2ʰ – 2

Example calculation for /27

Show subnet ranges

Mention CIDR and VLSM

→ How to attempt this in exam: Define, explain why, give formulas, then solve numerically with subnet ranges. End with CIDR notation /27.

### Q5. Explain TCP/IP Protocol Suite with all layers and their protocols.

Answer Outline:

4 layers (or 5 with physical separated)

Layer-wise protocol list

Comparison with OSI

Encapsulation flow

→ How to attempt this in exam: Draw stack with protocols at each layer (HTTP, FTP at app; TCP/UDP at transport; IP at network; Ethernet at access). Tabulate.

### Q6. Explain various Multiple Access Protocols (ALOHA, CSMA, CSMA/CD, CSMA/CA).

Answer Outline:

Define channel allocation problem

Random vs Controlled vs Channelization

Pure ALOHA (1971), Slotted ALOHA

CSMA variations (1-persistent, non-persistent, p-persistent)

CSMA/CD (Ethernet)

CSMA/CA (Wi-Fi)

→ How to attempt this in exam: Hierarchical structure: list categories → describe each protocol with diagram → compare efficiencies.

### Q7. Explain Cryptography in detail with RSA example.

Answer Outline:

Need for cryptography (CIA + non-repudiation)

Symmetric vs Asymmetric

DES, AES, RSA briefly

RSA algorithm steps

Numerical example (e.g., N=119, E=5, M=6 → C=41)

Digital signature concept

→ How to attempt this in exam: Explain need, compare symmetric/asymmetric, then walk through RSA steps with calculation. Cover digital signature in last paragraph.

### Q8. Explain Routing Algorithms (Distance Vector, Link State).

Answer Outline:

Routing definition

Distance Vector (RIP) — Bellman-Ford

Link State (OSPF) — Dijkstra

Comparison: convergence, complexity, scalability

Count-to-infinity problem in DV

→ How to attempt this in exam: Define routing, explain DV with example, explain LS with Dijkstra's algorithm steps, then comparison table.

### Q9. Explain Application Layer Protocols (HTTP, FTP, SMTP, DNS).

Answer Outline:

HTTP: web pages, port 80, request/response, methods, status codes

FTP: file transfer, ports 20/21, two connections, out-of-band control

SMTP: email push, port 25, commands

DNS: name resolution, port 53, hierarchy

→ How to attempt this in exam: Tabulate first (protocol/port/transport/use), then expand each in a paragraph with example.

### Q10. Explain Congestion Control mechanisms (TCP slow start, congestion avoidance, leaky/token bucket).

Answer Outline:

Congestion definition (router queues filling up)

Effects: packet loss, increased delay

TCP algorithms: Slow Start (exponential), Congestion Avoidance (linear), Fast Retransmit, Fast Recovery

Open-loop: Leaky bucket, Token bucket

Diagrams of cwnd vs RTT

→ How to attempt this in exam: Define, then split into TCP closed-loop and open-loop. Draw cwnd vs time graph. Compare leaky vs token bucket.

# 🟡 TOP 15 MOST LIKELY 2-MARK / SHORT ANSWER QUESTIONS

### Q1. What is a Protocol?

A set of rules governing data communication between two or more devices. Three key elements: syntax, semantics, timing.

→ Strategy: Define + 3 elements.

### Q2. Define Bandwidth.

The range of frequencies a channel can carry (Hz) OR the data rate of a channel (bps).

→ Strategy: Give both definitions.

### Q3. What is the difference between Hub and Switch?

Hub: physical layer, broadcasts to all ports. Switch: data link layer, forwards selectively using MAC table.

→ Strategy: Mention layer + behavior.

### Q4. What is a MAC Address?

48-bit physical address assigned to NIC, used for hop-by-hop delivery (e.g., 00:1A:2B:3C:4D:5E).

→ Strategy: Bits + purpose + format.

### Q5. Define Throughput vs Bandwidth.

Bandwidth = max channel capacity (theoretical). Throughput = actual data delivered per second (≤ bandwidth).

→ Strategy: Use "max" vs "actual" keywords.

### Q6. What is a Port Number?

16-bit number identifying a process/service on a host. Range: 0-65535. Well-known ports: 0-1023 (e.g., HTTP=80, FTP=21).

→ Strategy: Bits + range + 2 examples.

### Q7. What is Subnet Mask?

32-bit number used to divide IP address into network and host portions. Example: 255.255.255.0 = /24.

→ Strategy: Define + example.

### Q8. What is Encapsulation?

Process by which each OSI layer adds its own header (and trailer) to data from upper layer before passing down.

→ Strategy: Layer-wise data growth.

### Q9. Define Hamming Distance.

Number of bit positions in which two equal-length binary words differ. Example: d(10101, 10000) = 2.

→ Strategy: Definition + example.

### Q10. What is ARP?

Address Resolution Protocol. Maps a known IP address to its MAC address by broadcasting request on local network.

→ Strategy: Full form + function in 1 line.

### Q11. What is DNS used for?

DNS (Domain Name System) translates human-readable domain names to IP addresses. Uses UDP port 53.

→ Strategy: Full form + function + port.

### Q12. What are the two types of Transmission Modes?

Simplex (one direction), Half-duplex (both directions but not simultaneously), Full-duplex (both simultaneously).

→ Strategy: Give all 3 with example each.

### Q13. What is a Firewall?

Hardware/software that filters network traffic based on security rules to protect internal networks. Types: packet filter, stateful, proxy gateway.

→ Strategy: Define + 3 types.

### Q14. What is Piggybacking?

Technique where ACKs are sent along with outgoing data frames (instead of separate ACK frames) to reduce overhead.

→ Strategy: Define + benefit.

### Q15. What is Multiplexing?

Technique to combine multiple signals on a single transmission medium. Types: FDM, TDM, WDM, CDM.

→ Strategy: Define + 4 types in one line.

# 🟢 TOP 5 NUMERICAL PROBLEMS EXPECTED

### Q1. Calculate Propagation/Transmission time.

Formula:

Propagation Time = Distance / Speed of light in medium

Transmission Time = Message size / Bandwidth

Example: 2.5 KB at 1 Gbps, 12,000 km, 2.4×10⁸ m/s →

T_prop = 12×10⁶ / 2.4×10⁸ = 50 ms

T_trans = 20,000 / 10⁹ = 20 μs

→ Strategy: Always convert units (KB → bits, km → m) BEFORE substituting.

### Q2. Channel Capacity (Nyquist & Shannon).

Nyquist: C = 2B × log₂L

Shannon: C = B × log₂(1 + SNR)

Example: B=4 kHz, L=4 → C = 8000 × 2 = 16 kbps

→ Strategy: Identify whether channel is noiseless (Nyquist) or noisy (Shannon).

### Q3. CRC computation.

Append n zeros (n = degree of generator)

Divide by generator polynomial using XOR

Remainder = CRC bits

Transmitted = data + CRC

Example: Data 10011101, generator x³+1 (1001) → CRC = 100 → Transmitted = 10011101100

→ Strategy: Show step-by-step XOR division. Verify by re-dividing transmitted code (should give 0).

### Q4. Subnetting with given mask.

Example: Mask 255.255.255.224 (or 255.255.240.0)

Find borrowed bits → subnets = 2ⁿ

Find host bits → hosts = 2ʰ – 2

For 255.255.255.224 (/27): 8 subnets × 30 hosts each.

For 255.255.240.0 (/20): hosts = 4094.

→ Strategy: Convert mask to binary, count borrowed bits, apply formulas.

### Q5. RSA Encryption/Decryption.

Example: N=119, E=5, D=77, M=6 (character F)

C = M^E mod N = 6⁵ mod 119 = 41

Decryption: M = C^D mod N = 41⁷⁷ mod 119 = 6

→ Strategy: Use modular arithmetic step-by-step. Show 6², 6⁴, 6⁵ computations.

# ⭐ TOP 10 REPEATED / CLASSIC EXAM QUESTIONS

### Q1. Explain OSI Reference Model.

(Asked almost every year — guaranteed.)

→ Strategy: Memorize the 7-layer table cold. Draw it before writing.

### Q2. Compare OSI and TCP/IP.

(High frequency — pair with Q1.)

→ Strategy: Tabular comparison + side-by-side stack diagram.

### Q3. Explain CSMA/CD vs CSMA/CA with flowchart.

(Tied to Ethernet/Wi-Fi — frequent.)

→ Strategy: Two flowcharts + one comparison table = full marks.

### Q4. Compare TCP and UDP.

(Most common transport question.)

→ Strategy: Header diagrams + 8-row comparison + applications.

### Q5. Explain Three-Way Handshake.

(Classic transport question.)

→ Strategy: Diagram with seq/ack numbers + state problem solved.

### Q6. Construct Hamming Code for 4-bit data.

(Numerical, high-frequency.)

→ Strategy: Position table + parity calc + correction example.

### Q7. Explain CRC with example.

(Numerical favorite of examiners.)

→ Strategy: Show division clearly with XORs aligned.

### Q8. Explain DNS with hierarchy.

(Application layer staple.)

→ Strategy: Inverted tree + resolution flow.

### Q9. Compare FTP and TFTP.

(Easy table, frequent.)

→ Strategy: 8-row comparison table.

### Q10. Explain RSA with numerical example.

(Crypto numerical.)

→ Strategy: Algorithm steps + step-by-step modular calculation.

# END OF IMPORTANT QUESTIONS BANK