# DCCN COMPLETE SYLLABUS NOTES — UNITS 1 TO 5

Course: BCA / B.Sc(IT) | Subject Code: US6001 | Sem VI

# 🟦 UNIT – I: INTRODUCTION TO NETWORKS

## 📌 1.1 Computer Network – Definition

├── Definition: A computer network is a collection of two or more autonomous devices (computers, printers, servers) interconnected through a transmission medium to share resources and information.

├── Key Points:

Allows resource sharing (files, printers, internet)

Provides communication (email, chat, video)

Improves reliability through redundancy

Saves cost via shared resources

Enables centralized management

Types: LAN, MAN, WAN

├── Important Terms:

Node: Any device on the network

Link: Connection between two nodes

Protocol: Set of rules for communication

├── One-line Exam Answer: A computer network is an interconnected collection of autonomous computing devices that share resources and exchange data.

## 📌 1.2 Network Topologies

├── Definition: A topology is the physical or logical arrangement of nodes (computers) and links in a network.

├── Key Points:

Mesh: Every device connected to every other; n(n-1)/2 links; high reliability, expensive

Star: All devices connected to a central hub/switch; easy to manage; hub failure = total failure

Bus: All devices share single backbone cable; cheap; backbone failure = total failure

Ring: Each device connected to next forming a ring; data travels in one direction

Tree: Hierarchical combination of bus and star

Hybrid: Combination of two or more topologies

├── Diagrams:

MESH:                STAR:           BUS:
A----B-----C         A   B   C       A--+--B--+--C
|\  /|\   /|          \  |  /           |     |
| \/ | \ / |           \ | /             D     E
| /\ | / \ |            HUB
|/  \|/   \|           / | \
D----E-----F          D  E  F

RING:                TREE:
   A                    HUB
  / \                   / \
 D   B                Sub  Sub
  \ /                 /\    /\
   C                 A  B  C  D

├── One-line Exam Answer: Topology is the geometric arrangement of computers (nodes) and links in a network.

## 📌 1.3 Protocols and Standards

├── Definition: A protocol is a set of rules governing data communication. Standards are documented agreements ensuring interoperability across vendors.

├── Key Points:

Three key elements of protocol: Syntax (format), Semantics (meaning), Timing (when to send)

Standards bodies: IEEE, ISO, ITU-T, ANSI, IETF

De facto standards: not formally approved (e.g., Bluetooth originally)

De jure standards: legally formal (e.g., OSI model)

├── Important Terms:

IEEE 802.3 – Ethernet

IEEE 802.11 – Wi-Fi

TCP/IP – Internet protocol suite

├── One-line Exam Answer: A protocol is a set of rules and conventions for data exchange; standards ensure these rules are universally followed.

## 📌 1.4 OSI Reference Model (7 Layers)

├── Definition: The OSI (Open Systems Interconnection) model is a 7-layer conceptual framework for understanding network communication, developed by ISO in 1984.

├── Key Points (Mnemonic – "All People Seem To Need Data Processing"):

| Layer | Name | Function | PDU | Devices |
| --- | --- | --- | --- | --- |
| 7 | Application | User interface (HTTP, SMTP, FTP) | Message | Gateway |
| 6 | Presentation | Translation, encryption, compression | Message | Gateway |
| 5 | Session | Dialog control, synchronization | Message | Gateway |
| 4 | Transport | End-to-end delivery (TCP, UDP) | Segment | – |
| 3 | Network | Routing, logical addressing (IP) | Packet | Router |
| 2 | Data Link | Framing, MAC addressing, error control | Frame | Switch, Bridge |
| 1 | Physical | Bits over medium (signals) | Bit | Hub, Repeater |

├── Diagram:

+---------------------+
|  7. Application     |  ← HTTP, FTP, SMTP
+---------------------+
|  6. Presentation    |  ← Encryption, Compression
+---------------------+
|  5. Session         |  ← Dialog Control
+---------------------+
|  4. Transport       |  ← TCP, UDP
+---------------------+
|  3. Network         |  ← IP, Routers
+---------------------+
|  2. Data Link       |  ← Switches, MAC
+---------------------+
|  1. Physical        |  ← Cables, Hubs
+---------------------+

├── One-line Exam Answer: The OSI model is a 7-layer reference architecture (Physical, Data Link, Network, Transport, Session, Presentation, Application) standardizing network functions.

## 📌 1.5 TCP/IP Protocol Suite (Internet Model)

├── Definition: The TCP/IP model is a practical 4-layer (or sometimes 5-layer) protocol stack used in the Internet, developed before OSI.

├── Key Points:

| TCP/IP Layer | OSI Equivalent | Protocols |
| --- | --- | --- |
| Application | Application + Presentation + Session | HTTP, FTP, SMTP, DNS |
| Transport | Transport | TCP, UDP |
| Internet | Network | IP, ICMP, ARP |
| Network Access (Link) | Data Link + Physical | Ethernet, Wi-Fi |

├── Diagram:

TCP/IP                OSI
+-----------+        +-------------+
|Application|        |Application  |
|           |        +-------------+
|           |        |Presentation |
|           |        +-------------+
|           |        |Session      |
+-----------+        +-------------+
| Transport |        |Transport    |
+-----------+        +-------------+
| Internet  |        |Network      |
+-----------+        +-------------+
| Network   |        |Data Link    |
| Access    |        +-------------+
|           |        |Physical     |
+-----------+        +-------------+

├── One-line Exam Answer: The TCP/IP model is a 4-layer protocol stack (Network Access, Internet, Transport, Application) that is the basis of the modern Internet.

## 📌 1.6 Transmission Media

├── Definition: Physical paths through which data travels between nodes; either guided (cables) or unguided (wireless).

### A. Guided Media:

| Type | Bandwidth | Distance | Use |
| --- | --- | --- | --- |
| Twisted Pair (UTP/STP) | Up to 100 Mbps | ~100 m | LAN, Telephone |
| Coaxial Cable | Up to 1 Gbps | ~500 m | Cable TV, older LAN |
| Optical Fiber | THz | 100+ km | Internet backbone |

### B. Unguided Media (Wireless):

Radio waves – broadcast, AM/FM

Microwaves – cellular, satellite

Infrared – TV remote, short range

Bluetooth, Wi-Fi – personal/local wireless

├── One-line Exam Answer: Transmission media are physical/wireless paths used for data signal travel; categorized into guided (wired) and unguided (wireless).

## 📌 1.7 Addressing in Networks

├── Definition: Addressing is the mechanism to uniquely identify devices and processes on a network.

├── Types of Addresses:

| Layer | Address Type | Example | Length |
| --- | --- | --- | --- |
| Application | Specific (URL, Email) | www.google.com | Variable |
| Transport | Port number | 80, 443 | 16 bits |
| Network | IP address (logical) | 192.168.1.5 | 32 bits (IPv4) / 128 bits (IPv6) |
| Data Link | MAC address (physical) | 00:1A:2B:3C:4D:5E | 48 bits |

├── One-line Exam Answer: Addressing schemes (MAC, IP, Port) at different OSI layers uniquely identify hosts, networks, and processes for end-to-end communication.

## 📌 1.8 Networking Devices

| Device | Layer | Function |
| --- | --- | --- |
| Repeater | Physical | Regenerates weak signals |
| Hub | Physical | Connects multiple devices (broadcasts to all) |
| Bridge | Data Link | Connects two LAN segments |
| Switch | Data Link | Multi-port bridge with MAC table (forwards selectively) |
| Router | Network | Connects different networks, routes packets |
| Gateway | All | Connects networks of different protocols |
| Modem | Physical | Modulates/demodulates analog ↔ digital |
| NIC | Data Link | Network Interface Card – connects host to network |

├── One-line Exam Answer: Networking devices (hub, switch, router, etc.) operate at different OSI layers to connect, segment, and route traffic in a network.

## 📌 1.9 LAN, MAN, WAN

├── Definition: Networks are classified by geographic scope.

| Type | Range | Speed | Example |
| --- | --- | --- | --- |
| LAN (Local Area) | <1 km (room/building) | High (1 Gbps+) | Office Ethernet |
| MAN (Metropolitan) | City-wide (10-50 km) | Medium | Cable TV network |
| WAN (Wide Area) | Country/global (>100 km) | Lower | Internet |
| PAN (Personal) | <10 m | Bluetooth | Wireless earbuds |

├── One-line Exam Answer: Networks are categorized by geographic scope: PAN (<10m), LAN (<1km), MAN (city), WAN (country/global).

# 🟩 UNIT – II: PHYSICAL LAYER & DATA LINK LAYER

## 📌 2.1 Data and Signals

├── Definition: Data is the information to be communicated; signal is the electric/electromagnetic representation of data.

├── Key Points:

Analog signal: continuous waveform (sine wave)

Digital signal: discrete values (0s and 1s)

Analog signal: amplitude, frequency, phase

Digital signal: bit rate, bit length

Bandwidth = range of frequencies (Hz)

Bit rate = bits per second (bps)

├── One-line Exam Answer: Data is information; signals are analog or digital electromagnetic representations of that data over a medium.

## 📌 2.2 Transmission Impairments

├── Definition: Imperfections that degrade signal quality during transmission.

├── Three main types:

Attenuation – loss of signal energy with distance (use amplifiers/repeaters)

Distortion – signal shape changes (different frequencies travel differently)

Noise – unwanted signals interfering with the original

Thermal noise (random electron motion)

Induced noise (motors, appliances)

Crosstalk (between adjacent wires)

Impulse noise (lightning, power surges)

├── One-line Exam Answer: Transmission impairments (attenuation, distortion, noise) are signal degradations that reduce data communication quality.

## 📌 2.3 Data Rate Limits — Nyquist & Shannon

├── Definition: Maximum theoretical bit rate possible over a channel.

├── Two Formulas:

1. Nyquist (Noiseless Channel):

**C = 2 × B × log₂(L)**

B = bandwidth (Hz)

L = number of signal levels

C = maximum bit rate (bps)

2. Shannon (Noisy Channel):

**C = B × log₂(1 + SNR)**

SNR = Signal-to-Noise Ratio (linear)

SNR(dB) = 10 × log₁₀(SNR)

├── Example: B=4kHz, L=4 → Nyquist: C = 2×4000×log₂4 = 16 kbps

├── One-line Exam Answer: Nyquist gives max bit rate for noiseless (2B log₂L); Shannon gives max for noisy channel (B log₂(1+SNR)).

## 📌 2.4 Performance Metrics

| Metric | Definition |
| --- | --- |
| Bandwidth | Range of frequencies / data rate of channel |
| Throughput | Actual data delivered per second (≤ Bandwidth) |
| Latency (Delay) | Time from sender → receiver |
| Jitter | Variation in delay |
| Bandwidth-Delay Product | Capacity = Bandwidth × RTT (bits in flight) |

Total Latency = Propagation + Transmission + Queuing + Processing delays

├── One-line Exam Answer: Performance is measured by bandwidth, throughput, latency, jitter, and bandwidth-delay product.

## 📌 2.5 Switching

├── Definition: Mechanism by which data is transferred between nodes through intermediate devices.

### Three types:

Circuit Switching – dedicated path (telephone)

Packet Switching – packets routed independently (Internet)

Datagram approach – each packet treated separately

Virtual Circuit approach – logical path established

Message Switching – store-and-forward of complete messages (obsolete)

├── One-line Exam Answer: Switching is the technique by which data is moved between nodes — circuit, packet, or message switching.

## 📌 2.6 Multiplexing

├── Definition: Technique to share a single transmission medium among multiple signals/users simultaneously.

### Types:

FDM (Frequency Division Multiplexing) – different frequencies (radio)

TDM (Time Division Multiplexing) – different time slots (digital phone)

WDM (Wavelength Division) – different wavelengths in fiber (optical)

CDM (Code Division) – different codes (3G mobile)

├── Diagram:

FDM:  [Signal A][Signal B][Signal C]   (different frequency bands)
TDM:  [A1][B1][C1][A2][B2][C2]...      (time slots rotated)

├── One-line Exam Answer: Multiplexing combines multiple signals onto one channel via FDM (frequency), TDM (time), WDM (wavelength), or CDM (code).

## 📌 2.7 Data Link Layer Services

├── Definition: Layer 2 of OSI; provides reliable transfer of frames between adjacent nodes.

├── Services:

Framing – divides bits into frames

Physical Addressing – adds source/destination MAC

Flow Control – prevents overwhelming receiver

Error Control – detect and correct errors

Access Control – manages shared medium

Reliable Delivery – through ACKs and retransmissions

├── One-line Exam Answer: Data link layer provides framing, addressing, error control, flow control, and access control services for hop-by-hop reliable delivery.

## 📌 2.8 Framing

├── Definition: Process of dividing the bit stream from network layer into discrete units (frames) at the data link layer.

├── Methods:

Character Count – first byte indicates frame length

Byte Stuffing (Flag bytes) – escape character before flag in data

Bit Stuffing (Flag bits) – insert 0 after every 5 ones in data

Physical Layer Coding Violations – use illegal bit patterns

├── Bit Stuffing Example:

Data: 01111110 → after stuffing: 011111**0**10 (extra 0 inserted after 5 ones)

├── One-line Exam Answer: Framing is the data link layer technique to break the bit stream into manageable frames using count, byte/bit stuffing, or physical layer methods.

## 📌 2.9 Error Detection & Correction

### Detection methods:

Parity check (single bit – even/odd)

Two-dimensional parity (rows + columns)

Checksum (sum of segments + 1's complement)

CRC (Cyclic Redundancy Check) – uses polynomial division

### Correction methods:

Hamming Code – inserts parity bits at positions 1, 2, 4, 8...

Hamming Distance = number of differing bits between two codewords

For correction of d errors → minimum distance = 2d+1

├── One-line Exam Answer: Errors are detected via parity, checksum, CRC; corrected via Hamming code, requiring redundant bits in the message.

## 📌 2.10 ARQ Protocols (Automatic Repeat reQuest)

| Protocol | Window | Receiver Buffer | Retransmission |
| --- | --- | --- | --- |
| Stop-and-Wait | 1 | None | Single frame |
| Go-Back-N | N | None | All from lost frame onward |
| Selective Repeat | N | N | Only the lost frame |

├── Efficiency: η = N / (1+2a), where a = T_prop/T_trans

├── One-line Exam Answer: ARQ protocols (Stop-and-Wait, Go-Back-N, Selective Repeat) provide reliable delivery via acknowledgments and retransmission.

# 🟨 UNIT – III: LLC / MAC SUB-LAYER

## 📌 3.1 LLC (Logical Link Control) Sub-Layer

├── Definition: Upper sub-layer of data link layer (IEEE 802.2); handles flow control, error control, framing.

├── Key Points:

Provides interface to network layer

Independent of physical medium

Three service types: unacknowledged connectionless, connection-oriented, acknowledged connectionless

├── One-line Exam Answer: LLC is the upper data link sublayer providing flow/error control and a uniform interface to upper layers regardless of MAC.

## 📌 3.2 MAC (Medium Access Control) Sub-Layer

├── Definition: Lower sub-layer of data link layer; controls how devices access the shared communication medium.

├── Key Points:

Solves "channel allocation problem" in shared media

Provides MAC addresses

Implements multiple access protocols

├── One-line Exam Answer: MAC sub-layer manages access to shared transmission medium and provides physical addressing.

## 📌 3.3 Channel Allocation Problem

├── Definition: Problem of allocating a shared channel to multiple competing users.

├── Two approaches:

Static allocation – FDM, TDM (wasteful for bursty traffic)

Dynamic allocation – allocated as needed (more efficient)

├── One-line Exam Answer: Channel allocation is deciding which station can transmit on a shared medium; can be static (FDM/TDM) or dynamic (random/controlled access).

## 📌 3.4 Multiple Access Protocols

├── Categories:

Random Access: ALOHA, CSMA, CSMA/CD, CSMA/CA

Controlled Access: Polling, Reservation, Token Passing

Channelization: FDMA, TDMA, CDMA

## 📌 3.5 ALOHA & Slotted ALOHA

├── Definition: Earliest random access protocols developed at the University of Hawaii.

| Feature | Pure ALOHA | Slotted ALOHA |
| --- | --- | --- |
| Timing | Send anytime | Send only at slot boundaries |
| Vulnerable period | 2 × frame time | 1 × frame time |
| Max efficiency | 18.4% | 36.8% |
| Collisions | More | Fewer |

├── Diagrams:

Pure ALOHA:    A--+    Frame from A
             B--+      Frame from B (collision possible anytime)
                |
Slotted ALOHA: |slot|slot|slot|slot|
               A    B    C
               (collisions only within slot)

├── One-line Exam Answer: ALOHA is a random access protocol where stations transmit anytime (pure) or at slot boundaries (slotted, doubling efficiency).

## 📌 3.6 CSMA (Carrier Sense Multiple Access)

├── Definition: Stations listen ("sense") to the channel before transmitting to avoid collisions.

├── Variations:

1-persistent: if idle, send immediately (high collision risk)

Non-persistent: if busy, wait random time

p-persistent: if idle, transmit with probability p

├── One-line Exam Answer: CSMA reduces collisions by sensing the channel before transmitting; variations differ in waiting strategy.

## 📌 3.7 CSMA/CD vs CSMA/CA

| CSMA/CD | CSMA/CA |
| --- | --- |
| Used in Ethernet (wired) | Used in Wi-Fi (wireless) |
| Detect collision after | Avoid collision before |
| Sense → Send → Detect → Backoff | Sense → Wait IFS → Backoff → RTS/CTS → Send → ACK |

├── One-line Exam Answer: CSMA/CD detects collisions in wired networks; CSMA/CA avoids them in wireless via RTS/CTS handshake.

## 📌 3.8 IPv4 Addressing

├── Definition: 32-bit address used to uniquely identify a host on the Internet, written in dotted decimal (e.g., 192.168.1.1).

├── Classes:

| Class | First Octet | Default Mask | # Networks | # Hosts |
| --- | --- | --- | --- | --- |
| A | 0-127 | /8 | 128 | 16,777,214 |
| B | 128-191 | /16 | 16,384 | 65,534 |
| C | 192-223 | /24 | 2,097,152 | 254 |
| D | 224-239 | – | (multicast) | – |
| E | 240-255 | – | (reserved) | – |

├── Special Addresses:

127.0.0.1 – loopback (localhost)

0.0.0.0 – default route

255.255.255.255 – limited broadcast

169.254.x.x – APIPA

├── One-line Exam Answer: IPv4 is a 32-bit logical address divided into 5 classes (A-E) for unicast, multicast, and reserved use.

## 📌 3.9 Subnetting & CIDR

├── Definition: Subnetting divides a large network into smaller subnetworks; CIDR replaces classful addressing with prefix notation.

├── Formulas:

Number of subnets = 2ⁿ (n = borrowed bits)

Number of usable hosts = 2ʰ – 2 (h = host bits)

├── Example: 255.255.255.224 → mask /27

Subnet bits = 3 → 8 subnets

Host bits = 5 → 30 hosts/subnet

├── CIDR notation: 192.168.1.0/24 (instead of 192.168.1.0 + 255.255.255.0)

├── VLSM: Variable-Length Subnet Mask – different subnets, different sizes

├── One-line Exam Answer: Subnetting divides a network into smaller logical networks; CIDR uses /n prefix notation enabling efficient and flexible IP allocation.

## 📌 3.10 IPv6 Addressing

├── Definition: 128-bit address scheme to replace IPv4, written in hexadecimal (8 groups of 4 hex digits).

├── Example: 2001:0DB8:85A3:0000:0000:8A2E:0370:7334

├── Key Features:

128-bit (≈ 3.4 × 10³⁸ addresses)

No NAT needed (huge address space)

Built-in IPSec security

No broadcast (uses multicast/anycast)

Simplified header

Auto-configuration

├── One-line Exam Answer: IPv6 is a 128-bit address scheme designed to replace IPv4, providing vast address space, security, and auto-configuration.

## 📌 3.11 ICMP (Internet Control Message Protocol)

├── Definition: Network-layer protocol for error reporting and diagnostics in IP networks.

├── Types of messages:

Error-reporting: Destination Unreachable, Time Exceeded, Source Quench, Redirect, Parameter Problem

Query: Echo Request/Reply (ping), Timestamp, Address Mask, Router Solicitation

├── One-line Exam Answer: ICMP is an IP-layer helper protocol used for error reporting and diagnostic queries (ping, traceroute).

## 📌 3.12 ARP & RARP

├── ARP (Address Resolution Protocol): Maps IP → MAC address.

├── RARP (Reverse ARP): Maps MAC → IP address (used by diskless workstations; mostly replaced by DHCP).

├── Working of ARP:

Host A wants to send to IP X

A broadcasts: "Who has IP X?"

Host with IP X replies: "I am at MAC YY:YY:YY:YY:YY:YY"

A caches the IP-to-MAC mapping

├── One-line Exam Answer: ARP resolves IP to MAC; RARP resolves MAC to IP — both used in IPv4 networks.

## 📌 3.13 Routing Protocols

### A. Unicast Routing (one-to-one)

Distance Vector (RIP) – Bellman-Ford algorithm; uses hop count

Link State (OSPF) – Dijkstra's algorithm; full topology view

Path Vector (BGP) – inter-AS routing

### B. Multicast Routing (one-to-many)

DVMRP – Distance Vector Multicast

MOSPF – OSPF for multicast

PIM – Protocol Independent Multicast (Sparse/Dense modes)

├── One-line Exam Answer: Routing protocols (RIP, OSPF, BGP for unicast; DVMRP, PIM for multicast) determine the best path for IP packets.

# 🟧 UNIT – IV: NETWORK & TRANSPORT LAYER

## 📌 4.1 Transport Layer Overview

├── Definition: Layer 4 of OSI; provides end-to-end (process-to-process) communication via ports.

├── Functions:

Process-to-process delivery (using ports)

Connection establishment and termination

Flow control and error control

Congestion control

Reliable (TCP) or unreliable (UDP) delivery

├── One-line Exam Answer: Transport layer provides end-to-end process-to-process delivery, connection management, and flow/error/congestion control.

## 📌 4.2 TCP (Transmission Control Protocol)

├── Definition: Connection-oriented, reliable, ordered, byte-stream transport protocol.

├── Features:

Connection-oriented (3-way handshake)

Reliable (sequence numbers, ACKs, retransmission)

Ordered delivery

Flow control (sliding window)

Congestion control (slow start, congestion avoidance)

Full-duplex

├── TCP Header Fields (20-60 bytes):

Source/Dest Port (16 bits each)

Sequence Number (32 bits)

ACK Number (32 bits)

Data Offset (4 bits) + Reserved (6 bits) + Flags (URG, ACK, PSH, RST, SYN, FIN)

Window Size (16 bits)

Checksum (16 bits)

Urgent Pointer (16 bits)

Options (variable)

├── One-line Exam Answer: TCP is a reliable, connection-oriented transport protocol providing ordered byte-stream delivery with flow/congestion control.

## 📌 4.3 UDP (User Datagram Protocol)

├── Definition: Connectionless, unreliable, lightweight transport protocol.

├── Features:

No connection setup (faster)

No reliability/ordering

No congestion control

Low overhead (8-byte header)

Used for: DNS, DHCP, streaming, VoIP, online gaming

├── UDP Header (8 bytes):

Source Port (16 bits)

Dest Port (16 bits)

Length (16 bits)

Checksum (16 bits)

├── One-line Exam Answer: UDP is a connectionless, unreliable transport protocol with minimal overhead used for time-sensitive applications.

## 📌 4.4 Three-Way Handshake (TCP)

├── Diagram:

Client                    Server
  |--- SYN, seq=x -------->|
  |<-- SYN+ACK, y, ack=x+1 |
  |--- ACK, ack=y+1 ------>|
  |==== Connected =========|

├── One-line Exam Answer: TCP three-way handshake (SYN, SYN-ACK, ACK) synchronizes sequence numbers and establishes a reliable connection.

## 📌 4.5 Congestion Control

├── Definition: Mechanism to prevent network overloading by controlling sender's transmission rate.

├── TCP Algorithms:

Slow Start – cwnd starts at 1, doubles every RTT until threshold

Congestion Avoidance – linear increase after threshold

Fast Retransmit – retransmit on 3 duplicate ACKs

Fast Recovery – halve cwnd on triple-dup ACK (instead of restart)

├── Algorithms (Open-loop):

Leaky Bucket – output at constant rate

Token Bucket – allows bursts up to bucket size

├── One-line Exam Answer: Congestion control regulates sender rate (slow start, congestion avoidance, fast retransmit/recovery) to prevent network overload.

## 📌 4.6 Leaky Bucket vs Token Bucket

| Feature | Leaky Bucket | Token Bucket |
| --- | --- | --- |
| Output rate | Constant | Variable (bursts allowed) |
| Bucket holds | Packets | Tokens |
| Burst | Not allowed | Allowed (up to bucket size) |
| Use | Strict shaping | Flexible shaping |

## 📌 4.7 Sockets

├── Definition: An endpoint of bidirectional communication identified by IP + Port.

├── Types:

Stream socket (SOCK_STREAM) – TCP

Datagram socket (SOCK_DGRAM) – UDP

Raw socket (SOCK_RAW) – low-level protocol access

├── One-line Exam Answer: A socket is the (IP, Port) endpoint of network communication, available as stream (TCP), datagram (UDP), or raw types.

## 📌 4.8 QoS (Quality of Service)

├── Parameters:

Reliability (error/loss rate)

Delay (latency)

Jitter (delay variation)

Bandwidth (throughput)

├── Techniques: Scheduling (FIFO, Priority, Weighted Fair Queueing), Traffic shaping (Leaky/Token bucket), Resource reservation (RSVP), Admission control

├── One-line Exam Answer: QoS uses techniques like scheduling, traffic shaping, and admission control to guarantee parameters of reliability, delay, jitter, and bandwidth.

## 📌 4.9 Cryptography

├── Definition: Science of secure communication using mathematical algorithms.

├── Types:

| Type | Key | Speed | Algorithms |
| --- | --- | --- | --- |
| Symmetric | Same key | Fast | DES, AES, 3DES |
| Asymmetric | Public/Private pair | Slow | RSA, ECC, DSA |

├── Goals (CIA): Confidentiality, Integrity, Authentication (also Non-repudiation)

├── One-line Exam Answer: Cryptography secures data via symmetric (single key) or asymmetric (public/private key pair) algorithms.

## 📌 4.10 RSA Algorithm

├── Definition: Asymmetric encryption based on the difficulty of factoring large primes.

├── Steps:

Choose two primes p, q

Compute n = p × q

Compute φ(n) = (p-1)(q-1)

Choose e such that gcd(e, φ(n)) = 1 (public key)

Find d such that e × d ≡ 1 mod φ(n) (private key)

Encryption: C = M^e mod n

Decryption: M = C^d mod n

├── Example (N=119, E=5, D=77, M=6):

C = 6⁵ mod 119 = 41

├── One-line Exam Answer: RSA is an asymmetric encryption algorithm using public key (e,n) for encryption and private key (d,n) for decryption.

## 📌 4.11 Digital Signature

├── Definition: Cryptographic technique providing authentication, integrity, and non-repudiation.

├── Process:

Sender hashes message → encrypts hash with private key → digital signature

Receiver decrypts signature with sender's public key → recovers hash

Compares with computed hash; if match → valid

├── One-line Exam Answer: Digital signature is encrypted hash (using sender's private key) that authenticates origin and ensures integrity.

## 📌 4.12 Firewall

├── Definition: Hardware/software that filters network traffic between trusted and untrusted networks.

├── Types:

Packet Filter – inspects headers (IP/port/protocol) at L3

Stateful Inspection – tracks connection state

Application/Proxy Gateway – inspects content at L7

Next-Gen Firewall – combines all + IDS/IPS

├── One-line Exam Answer: A firewall is a network security device that filters traffic based on rules to protect internal networks from external threats.

# 🟪 UNIT – V: APPLICATION LAYER

## 📌 5.1 DNS (Domain Name System)

├── Definition: Hierarchical, distributed database that translates domain names (google.com) to IP addresses.

├── Key Points:

Uses UDP port 53 (TCP for zone transfers)

Hierarchy: Root → TLD → Authoritative

Types of records: A (IPv4), AAAA (IPv6), MX (mail), CNAME (alias), NS (name server), PTR (reverse)

Resolution: Iterative or Recursive

├── Hierarchy Diagram:

[Root]
        /  |  \
    .com  .org  .in
     |     |     |
  google  wiki  iitb
     |
    www

├── One-line Exam Answer: DNS is a hierarchical distributed naming system that translates domain names to IP addresses using UDP port 53.

## 📌 5.2 HTTP (HyperText Transfer Protocol)

├── Definition: Application-layer protocol for distributed, collaborative, hypermedia information systems (Web).

├── Features:

Port 80 (HTTP), Port 443 (HTTPS)

Stateless

Request-Response based

Uses TCP

├── HTTP Methods: GET, POST, PUT, DELETE, HEAD, OPTIONS, TRACE, CONNECT

├── Status Codes: 1xx (info), 2xx (success), 3xx (redirect), 4xx (client error), 5xx (server error)

├── One-line Exam Answer: HTTP is a stateless, request-response application-layer protocol on TCP port 80 used for transferring web resources.

## 📌 5.3 SMTP / Email

├── Definition: Push protocol used to send email between mail servers.

├── Email Components:

MUA (Mail User Agent) – client (Outlook, Gmail)

MTA (Mail Transfer Agent) – server (sendmail, postfix)

MDA (Mail Delivery Agent) – delivers to user mailbox

├── Protocols:

SMTP (port 25/587) – sending

POP3 (port 110) – downloading

IMAP (port 143) – server-stored access

├── SMTP Commands: HELO/EHLO, MAIL FROM, RCPT TO, DATA, RSET, VRFY, NOOP, QUIT

├── One-line Exam Answer: SMTP is the email-sending protocol on TCP port 25 using text commands like HELO, MAIL FROM, RCPT TO, DATA, QUIT.

## 📌 5.4 FTP (File Transfer Protocol)

├── Definition: Application protocol for transferring files between hosts.

├── Key Points:

Uses two TCP connections:

Port 21 – Control (commands)

Port 20 – Data (file transfer)

Out-of-band control (commands separate from data)

Two modes: ASCII / Binary

Active vs Passive mode

├── Common commands: USER, PASS, LIST, RETR (download), STOR (upload), QUIT

├── One-line Exam Answer: FTP is a TCP-based file transfer protocol using two connections — control (port 21) and data (port 20).

## 📌 5.5 TFTP (Trivial FTP)

├── Definition: Simple, lightweight version of FTP using UDP.

├── Features:

UDP port 69

No authentication

Used for: bootstrapping, firmware updates, embedded devices

5 message types: RRQ, WRQ, DATA, ACK, ERROR

├── One-line Exam Answer: TFTP is a simplified UDP-based file transfer protocol on port 69 with no authentication, used for bootstrapping.

## 📌 5.6 WWW (World Wide Web)

├── Definition: A global hypertext system of interlinked documents accessed via HTTP.

├── Components:

Web Browser (Chrome, Firefox)

Web Server (Apache, Nginx)

HTML/CSS/JS – page languages

URL – Uniform Resource Locator

HTTP/HTTPS – transport

├── One-line Exam Answer: WWW is a hypertext system of linked web pages accessed via browsers using HTTP over the Internet.

## 📌 5.7 Hyperlinks & Hypermedia

├── Hyperlink: A clickable reference within a hypertext document, linking to another resource (<a href="url">)

├── Hypermedia: Hypertext + multimedia (images, audio, video, animations) — superset of hypertext

├── One-line Exam Answer: Hyperlinks are clickable references; hypermedia extends hypertext by including multimedia (images, audio, video).

## 📌 5.8 Proxy Server

├── Definition: Intermediate server between client and destination server.

├── Functions:

Caching (improves speed)

Filtering (blocks unwanted content)

Anonymity (hides client IP)

Logging

Load balancing (reverse proxy)

├── Types: Forward proxy, Reverse proxy, Open proxy, Transparent proxy

├── One-line Exam Answer: A proxy server is an intermediate server providing caching, filtering, anonymity, and load-balancing services.

## 📌 5.9 Web Server

├── Definition: Server software/hardware that hosts and delivers web content (HTML, images) over HTTP/HTTPS.

├── Examples: Apache, Nginx, IIS, LiteSpeed

├── Ports: 80 (HTTP), 443 (HTTPS)

├── One-line Exam Answer: A web server hosts websites and serves content to clients via HTTP on port 80 / HTTPS on port 443.

## 📌 5.10 Active vs Dynamic Web Pages

| Feature | Active | Dynamic |
| --- | --- | --- |
| Execution | Client-side (browser) | Server-side |
| Tools | JavaScript, applets | PHP, ASP, JSP |
| Server load | Low | High |
| Example | Form validation | Product page from DB |

├── One-line Exam Answer: Active pages run scripts in the browser (JS); Dynamic pages are generated on-the-fly by server-side code accessing a database.

# END OF SYLLABUS NOTES (UNITS 1-5)